National Geospatial Program

Prepared in cooperation with the Asian Development Bank

Increasing Water Security on Pacific Islands Through Increased Data Availability and Understanding—Volume 1, Overview

By Gabriel B. Senay1, Tomas Smieszek1, and Matthew E. Andersen1

1U.S. Geological Survey.

Any use of trade, firm, or product names is for descriptive purposes only and does not imply endorsement by the U.S. Government.

Although this information product, for the most part, is in the public domain, it also may contain copyrighted materials as noted in the text. Permission to reproduce copyrighted items must be secured from the copyright owner.

This draft manuscript is distributed solely for purposes of courtesy review and comments received will be addressed and treated as appropriate to ensure there is no conflict of interest. Its content is deliberative and predecisional, so it must not be disclosed or released by reviewers. Because the manuscript has not yet been approved for publication by the U.S. Geological Survey (USGS), it does not represent any official USGS finding or policy.

Acknowledgments

This work was conducted in collaboration with the Strengthening Resilient and Sustainable Urban and Water Service Delivery in the Pacific project of the Water and Urban Development Sector of the Asian Development Bank (ADB). ADB provided financial support from some USGS technical salaries and associated administrative costs, including some of the travel necessary to administer and manage the work.

Outline

Acknowledgments

Introduction

Purpose and Scope

Methods

Applications of Remote Sensing for Water Assessments

Precipitation

A. Early Run

B. Late Run

C. Final Run

Evapotranspiration

Water Balance

Drought Monitoring

IMERG

Historical Baseline

Drought Indices

Quintile-Based Rainfall Index: 1-Month

Quintile-Based Rainfall Index: 3-Month and 6-Month Rolling

PEAC 4/8-inch Index

Developing the Monitoring Report Template

Informal Settlements Mapping

Satellite Imagery Sources

Study Area Boundaries

Image Co-registration

Building footprint extraction workflow

Image Preprocessing

Pan-sharpening

RGB Composite

Model Selection

Detection Parameters

Post-Processing Pipeline

Stage 1 -  AOI/Coastal Clip

Stage 2 - Area Filter

Stage 3 - Repair Geometry

Stage 4 - Regularize Building Footprints

Stage 5 - Manual QA/QC

Urban Growth Metric Computation

Seismic Risk

Summary

References Cited

Figures

Figure 1. Map of the Pacific Island nations of Federated States of Micronesia (FSM), Fiji, Nauru, Papua New Guinea, Republic of the Marshall Islands (RMI), Solomon Islands, and Vanuatu. The study sites are highlighted in a dark color (black) for each nation.

Figure 2. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

Figure 3. Workflow of IMERG precipitation data processing.

Figure 4. Example precipitation monitoring chart for a single location, Kosrae in the FSM. Orange bars represent current-year monthly totals; the blue line is the long-term monthly median; the gray ribbon is the interquartile range of the 2000–2024 baseline. The summary table to the right reports the current-month precipitation, long-term median, and all three drought index classifications for the most recently completed month.

Figure 5. Pacific Plate boundaries and relative motion. Source: USGS. Public Domain.

Tables

International System of Units to U.S. customary units

Table 1. Assessment tasks for each study site. The marker “x”  indicates a relevant activity for the site.

Table 2. Key Processing Parameters

Table 3. Study site summary - six informal settlement sites across three Pacific Island nations with survey epochs and sensor sources.

Table 4. Spatial resolution specifications for the three VHR sensor platforms used in this study

Table 5. Each stage is described in detail below.

Table 6. Deep learning detection parameters applied by country group. Lower confidence threshold at PNG sites was necessary to maximize recall under dense canopy cover.

Conversion Factors

International System of Units to U.S. customary units

| Multiply | By | To obtain |
| --- | --- | --- |
| Length |  |  |
| centimeter (cm) | 0.3937 | inch (in.) |
| millimeter (mm) | 0.03937 | inch (in.) |
| meter (m) | 3.281 | foot (ft) |
| kilometer (km) | 0.6214 | mile (mi) |
| kilometer (km) | 0.5400 | mile, nautical (nmi) |
| meter (m) | 1.094 | yard (yd) |
| Area |  |  |
| square meter (m2) | 0.0002471 | acre |
| hectare (ha) | 2.471 | acre |
| square hectometer (hm2) | 2.471 | acre |
| square kilometer (km2) | 247.1 | acre |
| square centimeter (cm2) | 0.001076 | square foot (ft2) |
| square meter (m2) | 10.76 | square foot (ft2) |
| square centimeter (cm2) | 0.1550 | square inch (in2) |
| square hectometer (hm2) | 0.003861 | section (640 acres or 1 square mile) |
| hectare (ha) | 0.003861 | square mile (mi2) |
| square kilometer (km2) | 0.3861 | square mile (mi2) |
| Volume |  |  |
| cubic meter (m3) | 6.290 | barrel (petroleum, 1 barrel = 42 gal) |
| liter (L) | 33.81402 | ounce, fluid (fl. oz) |
| liter (L) | 2.113 | pint (pt) |
| liter (L) | 1.057 | quart (qt) |
| liter (L) | 0.2642 | gallon (gal) |
| cubic meter (m3) | 264.2 | gallon (gal) |
| cubic decimeter (dm3) | 0.2642 | gallon (gal) |
| cubic meter (m3) | 0.0002642 | million gallons (Mgal) |
| cubic kilometer (km3) | 264.2 | billion gallons (Ggal) |
| cubic centimeter (cm3) | 0.06102 | cubic inch (in3) |
| cubic decimeter (dm3) | 61.02 | cubic inch (in3) |
| liter (L) | 61.02 | cubic inch (in3) |
| cubic decimeter (dm3) | 0.03531 | cubic foot (ft3) |
| cubic meter (m3) | 35.31 | cubic foot (ft3) |
| cubic meter (m3) | 1.308 | cubic yard (yd3) |
| cubic kilometer (km3) | 0.2399 | cubic mile (mi3) |
| cubic meter (m3) | 0.0008107 | acre-foot (acre-ft) |
| cubic hectometer (hm3) | 810.7 | acre-foot (acre-ft) |
| Flow rate |  |  |
| cubic meter per second (m3/s) | 70.07 | acre-foot per day (acre-ft/d) |
| cubic meter per year (m3/yr) | 0.000811 | acre-foot per year (acre-ft/yr) |
| cubic hectometer per year (hm3/yr) | 811.03 | acre-foot per year (acre-ft/yr) |
| meter per second (m/s) | 3.281 | foot per second (ft/s) |
| meter per minute (m/min) | 3.281 | foot per minute (ft/min) |
| meter per hour (m/h) | 3.281 | foot per hour (ft/h) |
| meter per day (m/d) | 3.281 | foot per day (ft/d) |
| meter per year (m/yr) | 3.281 | foot per year ft/yr) |
| cubic meter per second (m3/s) | 35.31 | cubic foot per second (ft3/s) |
| cubic meter per second per square kilometer ([m3/s]/km2) | 91.49 | cubic foot per second per square mile ([ft3/s]/mi2) |
| cubic meter per day (m3/d) | 35.31 | cubic foot per day (ft3/d) |
| liter per second (L/s) | 15.85 | gallon per minute (gal/min) |
| cubic meter per day (m3/d) | 264.2 | gallon per day (gal/d) |
| cubic meter per day per square kilometer ([m3/d]/km2) | 684.28 | gallon per day per square mile ([gal/d]/mi2) |
| cubic meter per second (m3/s) | 22.83 | million gallons per day (Mgal/d) |
| cubic meter per day per square kilometer ([m3/d]/km2) | 0.0006844 | million gallons per day per square mile ([Mgal/d]/mi2) |
| meter per hour (m/h) | 39.37 | inch per hour (in/h) |
| millimeter per year (mm/yr) | 0.03937 | inch per year (in/yr) |
| kilometer per hour (km/h) | 0.6214 | mile per hour (mi/h) |
| Mass |  |  |
| gram (g) | 0.03527 | ounce, avoirdupois (oz) |
| kilogram (kg) | 2.205 | pound avoirdupois (lb) |
| metric ton (t) | 1.102 | ton, short [2,000 lb] |
| metric ton (t) | 0.9842 | ton, long [2,240 lb] |
| Pressure |  |  |
| kilopascal (kPa) | 0.009869 | atmosphere, standard (atm) |
| kilopascal (kPa) | 0.01 | bar |
| kilopascal (kPa) | 0.2961 | inch of mercury at 60°F (in Hg) |
| kilopascal (kPa) | 0.1450 | pound-force per inch (lbf/in) |
| kilopascal (kPa) | 20.88 | pound per square foot (lb/ft2) |
| kilopascal (kPa) | 0.1450 | pound per square inch (lb/ft2) |
| Density |  |  |
| kilogram per cubic meter (kg/m3) | 0.06242 | pound per cubic foot (lb/ft3) |
| gram per cubic centimeter (g/cm3) | 62.4220 | pound per cubic foot (lb/ft3) |
| Energy |  |  |
| joule (J) | 0.0000002 | kilowatthour (kWh) |
| Radioactivity |  |  |
| becquerel per liter (Bq/L) | 27.027 | picocurie per liter (pCi/L) |
| Specific capacity |  |  |
| liter per second per meter ([L/s]/m) | 4.831 | gallon per minute per foot ([gal/min]/ft) |
| Hydraulic conductivity |  |  |
| meter per day (m/d) | 3.281 | foot per day (ft/d) |
| Hydraulic gradient |  |  |
| meter per kilometer (m/km) | 5.27983 | foot per mile (ft/mi) |
| Transmissivity |  |  |
| meter squared per day (m2/d) | 10.76 | foot squared per day (ft2/d) |
| Application rate |  |  |
| kilogram per hectare per year ([kg/ha]/yr) | 0.8921 | pound per acre per year ([lb/acre]/yr) |
| Leakance |  |  |
| meter per day per meter ([m/d]/m) | 1 | foot per day per foot ([ft/d]/ft) |
| millimeter per year per meter ([mm/yr]/m) | 0.012 | inch per year per foot ([in/yr]/ft) |

Abbreviations

ADB	Asian Development Bank

FSM	Federated States of Micronesia

RMI	Republic of the Marshall Islands

USGS	U.S. Geological Survey

ET	Evapotranspiration

Abstract

The Asian Development Bank (ADB) has sought to support sustainable economic development in developing nations of Asia since 1965. Since 2024, the Water and Urban Development Sector office has been in consultation with seven Pacific Ocean nations. One result of the consultations was that all seven nations sought technical assistance with managing water resources. ADB asked the United States Geological Survey (USGS) if there were one or more ways USGS could help increase water security on these islands. USGS responded by making use of Earth observation satellite data to determine the availability of water from precipitation, less evapotranspiration, and to calculate some of the expected demand for water. While the annual water balance for these locations is in general positive due to more rain falling than the amount of water evaporating, the timing of the precipitation is problematic because acute water shortages occur for weeks or months at a time when the wet monsoon does not restart promptly.

In recent years, the monsoon timing and intensity showed more variability. If water conservation and storage strategies were desired, then the data and analyses presented in this report could be useful for guiding timing and volume of storage. In addition to water data, USGS also offered to report on nearby seismicity for some of the islands studied to help understand and assess the degree of risk from earthquakes and tsunamis.

Introduction

Freshwater is critically important for life but is often in short supply for weeks or months at a time on islands of the Pacific Ocean. Utilities, managers, and the public need the best information available to support knowledgeable water management, especially for reducing the negative impacts of water shortages to the extent possible. The primary objective of the Asian Development Bank (ADB) is to foster economic growth and cooperation in Asia, including providing technical assistance (ADB, 1965; see: www.adb.org) through donor-funded assistance. Dependable access to safe and reliable water sources is a critically important foundation of sustainable economic growth because water security underpins long‑term economic stability. Sustainable management becomes more effective when water managers understand how much water is available and when it is available. These insights can aid in decision-making when infrastructure investments—such as expanded storage capacity—would meaningfully reduce vulnerability. As information improves, storage systems can be operated more efficiently, and development planning can be grounded in realistic expectations of future water availability.

The U.S. Geological Survey (USGS) is part of the Department of the Interior and provides relevant data about natural resources and hazards to monitor, analyze, and predict Earth’s changing systems. Science is clear and reliable to provide hazard prediction, strengthen water infrastructure and promotes economic growth (see: www.usgs.gov). Beginning in 2024, the USGS entered a collaborative relationship with ADB to assess the utility of various remotely sensed data sets including precipitation, evapotranspiration, land cover and land use from satellites orbiting the Earth to evaluate and analyze water management challenges on islands of selected Pacific Ocean nations, including Federated States of Micronesia (FSM), Fiji, Nauru, Papua New Guinea, Republic of the Marshall Islands (RMI), Solomon Islands, and Vanuatu. Besides the satellite data, field observations from rainfall gauges were used to independently assess for accuracy. One advantage of the methods employed is that surface water resources may be estimated with reasonable accuracy where little or no on-the-ground monitoring occurs. In addition, satellite data provide spatially continuous coverage that captures precipitation or evapotranspiration patterns across the entire island nation, rather than only at isolated gauge locations. Their consistent, long-term records make it possible to evaluate interannual variability and climate-driven changes in water availability, even in periods when ground-based measurements are incomplete or unavailable.

The USGS conducted an assessment of regional seismicity for several of the islands, providing important context for understanding broader environmental risk. Although the primary focus of this work is water availability and drought monitoring, seismic activity can influence island stability, coastal integrity, and the potential for tsunami generation, which can threaten groundwater resources. Incorporating seismic information alongside hydrologic data therefore offers a more complete picture of the natural hazards faced by these islands and supports more informed and risk-reduction strategies.

The combined evaluation of water balance dynamic, drought conditions, forecasting capabilities, and regional seismicity offers a comprehensive and integrated understanding of the hydrologic vulnerability of the islands. This multifaceted perspective equips stakeholders with a stronger scientific foundation for anticipating environmental stressors and developing resilient, forward-looking water management strategies that account for both climatic and geophysical risks.

Purpose and Scope

The USGS and ADB are working together to reduce water insecurity by improving how Pacific Island communities’ access and use watermonitoring information. Through this collaboration, the USGS provides scientific data, forecasting tools, and technical expertise, while the ADB supports the integration of these resources into local planning, hazard reduction, and infrastructure development. By making reliable, timely water information more widely available, the partnership helps utilities, managers, and policymakers anticipate shortages, manage supplies more efficiently, and strengthen long-term resilience to climate driven variability.

This effort spans a region unlike any other: the subject countries are distributed across a vast and remote expanse of the Pacific, where islands are separated by thousands of kilometers of open ocean and face highly localized climate and hydrologic conditions. Understanding this geographic context is essential, because the isolation and dispersion of these islands shape everything from data availability to the feasibility of infrastructure investments. An overview map of the study region (Figure 1) shows which islands were used in the study and helps illustrate the scale of how widely the islands are spread across the Pacific Ocean.

Figure 1. Map of the Pacific Island nations of Federated States of Micronesia (FSM), Fiji, Nauru, Papua New Guinea, Republic of the Marshall Islands (RMI), Solomon Islands, and Vanuatu. The study sites are highlighted in a dark color (black) for each nation.

Based on the large geographic region and the very localized conditions for each study site the following assessments were conducted for each study site (Table 1). The tasks were decided by the countries to meet their individual needs to characterize water availability, identify vulnerabilities, and outline opportunities for strengthening water security through improved monitoring, planning, and management.

Table 1. Assessment tasks for each study site. The marker “x”  indicates a relevant activity for the site.

| Locations | Island/City/Basin | Watershed | Precipitation | Evapotranspiration | Water Balance | Drought monitoring | 7-day forecast | Settlement | Seismicity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Federated States of Micronesia (FSM) |  |  |  |  |  |  |  |  |  |
|  | Yap | x | x | x | x | x | x |  | x |
|  | Weno | x | x | x | x | x | x |  | x |
|  | Pohnpei | x | x | x | x | x | x |  | x |
|  | Kosrae | x | x | x | x | x | x |  | x |
| Fiji (FIJ) |  |  |  |  |  |  |  |  |  |
|  | Viti Levu (Suva) |  | x | x | x |  |  | x |  |
| Nauru (NAU) |  |  |  |  |  |  |  |  |  |
|  | Nauru |  | x | x | x | x | x |  |  |
| Papua New Guinea (PNG) |  |  |  |  |  |  |  |  |  |
|  | Port Moresby | x | x | x | x | x |  | x |  |
| Republic of the Marshall Islands (RMI) |  |  |  |  |  |  |  |  |  |
|  | Majuro |  | x | x | x | x | x |  | x |
| Solomon Islands (SOL) |  |  |  |  |  |  |  |  |  |
|  | Honiara | x | x | x | x | x |  | x | x |
|  | Noro/Munda | x | x | x | x | x | x |  | x |
| Vanuatu (VAN) |  |  |  |  |  |  |  |  |  |
|  | Port Vila | x | x | x | x |  |  |  |  |
|  | Mele Basin | x | x | x | x |  |  |  |  |
|  | La Colle Basin | x | x | x | x |  |  |  |  |
|  | Tagabe Basin | x | x | x | x |  |  |  |  |
|  | Taeouma Basin | x | x | x | x |  |  |  |  |

The hydrologic assessments examined the core components of the water balance across the study region. Precipitation and evapotranspiration were used in this analysis, providing a first-order understanding of how much water enters and leaves island systems over time. The Integrated Multi-satellitE Retrievals for GPM (IMERG) precipitation data set was used due to its high spatial resolution (0.1°) and high temporal resolution (30-minute to daily intervals) since 1998 including the entire study region. The operational Simplified Surface Energy Balance (SSEBop) model was used to determine evaporative demand for each study location. The model is a USGS product that provides a global, satellite-derived actual evapotranspiration (ETa) by combining thermal observations with reference ET from gridded climate data sets. The data has a spatial resolution of  0.01° and a temporal resolution of every dekad (10 days) since 2012.

Those two data sources provided the basis for the water balance assessment. By comparing long-term averages with recent observations, the report highlights where climatic shifts, or seasonal extremes may be altering the reliability of freshwater supplies. These datasets also reveal gaps in monitoring coverage, with many islands lacking consistent rainfall records, evaporation measurements, or streamflow observations. Addressing these gaps is essential for improving early warning capabilities and for designing infrastructure that matches local hydrologic realities.

Drought monitoring was conducted for selected study sites (Table 1) to provide a systematic, satellite-based assessment of rainfall deficits relative to long-term climatic norms. The monitoring system uses NASA’s Integrated Multi-satellitE Retrievals for GPM (IMERG) monthly precipitation data to compute rolling Rainfall Indices over 1-month, 3-month, and 6-month accumulation windows. Each index compares recent observed precipitation against a 25-year (2000-2024) baseline period to identify the onset, persistence, and severity of meteorological drought conditions. By examining multiple accumulation windows, the system captures both short-duration rainfall deficits that affect day-to-day water supply operations and longer-duration anomalies that signal more sustained hydrologic stress. The site-specific rainfall indices provide water managers and utility operators with a clear, quantitative basis for assessing current drought severity and anticipating emerging water supply challenges before they become critical.

Complementing the backward-looking drought monitoring, a 7-day precipitation forecasting system was developed for selected study sites (Table 1) to provide near-term operational guidance on expected rainfall. The system retrieves forecast data from the United States National Oceanic and Atmospheric Administration’s (NOAA) Global Forecast System (GFS), a global numerical weather prediction model that produces gridded forecasts at 0.25° spatial resolution, updated four times daily. For each study site, the system extracts daily accumulated precipitation forecasts for the upcoming seven days, generating both spatial forecast maps showing the geographic distribution of expected rainfall around each location and summary bar charts that distill the forecast into an accessible, site-level view of anticipated conditions. The forecasts are compiled into automated reports and distributed to designated recipients via email, enabling water managers and utility operators to incorporate short-range rainfall expectations into their daily planning and operational decisions. This forward-looking capability fills a critical gap in water management across the Pacific, where sparse ground-based weather station networks and limited access to numerical weather prediction products have historically left many island communities without reliable short-term rainfall outlooks.

Informal settlement mapping was conducted for urban study sites selected by ADB (Table 1) to characterize the spatial extent and growth patterns of unplanned or informal housing areas that are particularly vulnerable to water supply disruptions and climate-related hazards. The mapping approach uses deep learning-based building footprint extraction applied to high-resolution satellite imagery across multiple temporal epochs, spanning approximately 2005 to 2025, to quantify changes in building density and settlement expansion over time. Individual building footprints were extracted from satellite scenes using pretrained convolutional neural network models, and the results were refined through a standardized post-processing workflow that included area-based filtering to remove spurious detections and geometric regularization to produce clean, orthogonal building outlines. By comparing footprint counts, total built-up area, and spatial distributions across epochs, the analysis reveals where and how rapidly informal settlements have expanded, providing planners and infrastructure agencies with an empirical basis for targeting water supply investments, assessing infrastructure demand, and understanding population growth pressures in areas that are often poorly documented by conventional census or cadastral records.

The following volumes of this report present the individual details of these investigations organized by nation and island. This format will allow for adding more results in the future if desired.

Methods

The methods used in this study integrate multiple data sources and analytical approaches to evaluate hydrologic conditions across the different study sites. Satellite‑derived precipitation and evapotranspiration products, ground‑based rainfall observations, and supporting geophysical information were combined to characterize water availability, drought severity, and long‑term climatic variability. Standardized procedures were applied to process, validate, and compare datasets, and water‑balance calculations were used to quantify net water availability. The following sections describe the datasets, processing steps, and analytical techniques employed to develop a consistent and scientifically robust assessment of the islands’ hydrologic status.

Applications of Remote Sensing for Water Assessments

Using remote sensing data sets for water assessments is well established in recent years (e.g., Lin and Song, 2026; Che et al., 2025; Bijeesh and Narasimhamurthy, 2020; Hou et al., 2020). But remote sensing has played an important role in water assessments for more than half a century, evolving from simple arial photography to today’s satellite-based monitoring systems. Early applications in the 1960’s and 1970’s relied on aircraft imagery to map for example surface water extent. As satellite programs such as Landsat matured, the ability to track changes in water bodies and rivers with repeatable and consistent observations increased the utility of these data. Over time, advances in spectral sensors and thermal imaging opened the door to estimating water quality, detecting sediment and algal blooms, and inferring evapotranspiration and soil moisture from space. These developments expanded the scope of hydrologic studies, enabling assessments of water availability, drought severity, and long‑term environmental change across regions where ground‑based measurements were sparse or nonexistent. Remote sensing now the foundation of many modern water‑resource evaluations, offering broad spatial coverage, historical depth, and the ability to integrate with models and in‑situ data to support planning and management of water resources.

Precipitation

Satellite precipitation products such as IMERG, TRMM, and CHIRPS have been widely used to quantify rainfall patterns, assess storm intensity, and evaluate long‑term climate variability. These datasets are especially valuable in small island and coastal environments where rainfall can vary sharply over short distances and where gauge networks are often limited to a single station. Numerous studies have demonstrated the utility of satellite rainfall estimates for drought monitoring, flood forecasting, and water‑balance modeling, showing strong agreement with gauge observations when properly validated.

We selected the IMERG dataset for this project due to its availability across all small islands in the study area and its near-real-time temporal resolution. IMERG provides three processing runs: (1) Early Run (≈4-hour latency) for real-time applications like flood monitoring, (2) Late Run (≈12-hour latency) for improved accuracy, and (3) Final Run (≈3-month latency) for research-quality data. Each product was utilized for different analyses. The annual mean for the period of 2015 - 2024 for the Late Run dataset highlights the regional variation of precipitation (Figure 2).

Figure 2. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

A. Early Run

The Early Run delivers IMERG precipitation data with roughly 4-hour latency, making it suitable for real-time applications such as flood monitoring. The United States National Aeronautics and Space Administration (NASA) provides documentation on the data at https://gpm.nasa.gov/data/directory (last accessed 2/27/2026). The data can be accessed from multiple locations, here the following location was used: https://jsimpsonhttps.pps.eosdis.nasa.gov/imerg/gis/early/ (last accessed 2/27/2026). The access requires a user to register for PPS (Precipitation Processing System) data access at http://registration.pps.eosdis.nasa.gov/registration/ (last accessed 2/27/2026). The chosen data location provides the data in 30-minute, 3-hour, and 1-day files all available in the same directory, as geotiffs, with the timespan indicated within the filename (e.g. 3B-HHR-E.MS.MRG.3IMERG.20200207-S053000-E055959.0330.V07B.1day.tif  is a 1 day file). Precipitation values are scaled by a factor of x10 (0.1mm) for 30-minute, 3-hour, 1-day files.

B. Late Run

The Late Run provides improved accuracy with about 12-hour latency, supporting more detailed analyses and operational assessments. To download the data the user needs a NASA Earthdata account (https://www.earthdata.nasa.gov/data/earthdata-login#toc-how-do-i-register-with-earthdata-login, last accessed 2/27/2026). Data downloaded from https://gpm1.gesdisc.eosdis.nasa.gov/data/GPM_L3/GPM_3IMERGDL.07/ (last accessed 2/27/2026) which is the 1-day data in .NC4 format. The filename is e.g. 3B-DAY-L.MS.MRG.3IMERG.20240101-S000000-E235959.V07B.nc4 where 3B=Level 3 processing, 20240101 = date as YYYYMMDD, S000000-E235959 = start and end time, and V07B = version of the data. Directory GPM_3IMERGHHL.07 includes the 30-minute data.

C. Final Run

The Final Run offers research-quality precipitation data with an approximate 3-month latency, ideal for retrospective and comprehensive studies. There are key differences between IMERG Early and Late Run. The half-hourly Final Run adjusts to the monthly Final Run product using a month-to-month ratio multiplier based on GPCC gauge analysis. The Late Run, computed 14 hours after observation, may miss some microwave overpasses. These are later included in the Final Run, affecting that half hour and possibly nearby periods (https://gpm.nasa.gov/data/directory, last accessed 2/26/2026). The Final Run data was acquired for the 30-minute time step from https://gpm1.gesdisc.eosdis.nasa.gov/data/GPM_L3/GPM_3IMERGHH.07/ (last accessed 2/26/2027) as .HDF5 file format and for the 1-day time step from https://gpm1.gesdisc.eosdis.nasa.gov/data/GPM_L3/GPM_3IMERGDF.07/ (last accessed 2/26/2027) as .NC4 file format. The filename for the 1-day data is e.g. 3B-HHR-.MS.MRG.3IMERG.20240101-S000000-E002959.0000.V07B.HDF5 where 3B=Level 3 processing, 20240101 = date as YYYYMMDD, S000000-E235959 = start and end time, 0000 = is a counter, and V07B = version of the data. And the filename is e.g. 3B-DAY.MS.MRG.3IMERG.20240101-S000000-E235959.V07B.nc4 where 3B=Level 3 processing, 20240101 = date as YYYYMMDD, S000000-E235959 = start and end time, and V07B = version of the data.

The precipitation summaries were created from the acquired or processed “.TIF” files as zonal average for each given boundary, which could be a single pixel, watershed, or whole island or atoll. The output format is a “.CSV” table where every row represents the time step, from 30-minute to daily and every column is one study site, showing the precipitation amount in mm.

Figure 3. Workflow of IMERG precipitation data processing.

The workflow from download to .CSV output product is automated in a script using Python programming language (Figure 2). The scripts are documented in the USGS data release.

Evapotranspiration

Evapotranspiration (ET) retrievals derived from thermal and optical satellite sensors—such as SSEBop, MOD16, and SEBAL—have become essential for estimating water loss from vegetation, soils, and open water bodies. These products provide consistent ET estimates across large areas, enabling assessments of agricultural water use, drought stress, and ecosystem water demand. Remote sensing–based ET has been particularly important in data‑limited regions, where direct measurements of ET are rarely available and ground‑based methods are costly or impractical. Here we used the operational Simplified Surface Energy Balance (SSEBop) model developed at the USGS EROS (Earth Resources Observation Science) Center. The model is a satellite-driven method for estimating actual Evapotranspiration (ETa) using the energy-balance framework to determine cold and hot boundary conditions of the landscape. Cold pixels represent well‑watered vegetation (high latent heat/ET), while hot pixels represent dry surfaces (low latent heat/ET). This allowed direct calculation of ET fraction, which, when multiplied by a reference ET, produces actual ET. The SSEBop model is widely used because of its simplicity and only requires readily available satellite data sets. In addition, it produces consistent, scalable ET estimates on a global scale starting in 2012 to current with good performance when compared to independent datasets over diverse hydroclimatic settings (Senay et al.,2020, 2023, Volk et al., 2024, Khand et al., 2025). Over the years the approach has been refined and continues evolving with new satellite sensors and improved algorithms.

Here we used the latest version, Version 6, as of February 2026 (USGS data release info). The data has a spatial resolution of  0.01° and is updated every dekad (10 days) since 2012, and is available to download from the USGS Data Portal (https://earlywarning.usgs.gov/fews/, accessed 2/26/2026) at dekadal, monthly or annual time steps. Besides the ETa, ET percent anomaly from median are also available.

Once the data was acquired from the website, the same script used in the precipitation summaries was used to create ETa summaries for each study site (USGS data release info for scripts).

Water Balance

Remote sensing has also been widely applied to derive water‑balance components, particularly in regions where hydrologic monitoring networks are sparse. By combining satellite‑based precipitation products with evapotranspiration estimates, researchers can calculate net water availability through a basic water balance, expressed as:

WB = 𝑃 – 𝐸𝑇a - dS					(1)

where, WB = Water Balance; P = Precipitation; ETa = Actual Evapotranspiration, dD = Change in Storage where it is considered negligible at annual time scale.

WB provides insight into periods of surplus and deficit across monthly to annual timescales. Although the change in storage term (dS) can be assumed negligible at the annual time scale, the monthly WB is simply estimated as the difference between P and ETa without accounting for dS and thus can only be interpreted as a relative indicator and not in absolute terms.  Satellite-based gridded datasets allow a consistent evaluation of hydrologic conditions even in locations where streamflow, groundwater, or soil‑moisture observations are unavailable. Satellite‑derived water‑balance assessments have been used to monitor drought development, map sources and sinks (Senay et al., 2014), quantify recharge potential, and evaluate long‑term shifts in freshwater availability under changing climate conditions. For small island environments, where direct measurements of runoff and storage are often limited or absent, remote sensing–based water‑balance methods offer a practical and scientifically robust means of understanding hydrologic vulnerability and supporting water‑resource planning. In this study we used the simple WB method to calculate the net water availability for the given study sites.

Drought Monitoring

To support evidence-based water resource management, disaster risk reduction, and climate adaptation planning across the region, a satellite-based monthly drought monitoring system was developed for nine study locations spanning four Pacific Island nations: the Marshall Islands, the Federated States of Micronesia (FSM), the Solomon Islands, and Nauru. The monitoring system uses Integrated Multi-satellitE Retrievals for Global Precipitation Measurement (IMERG) satellite data to generate monthly precipitation estimates at each location and evaluates those estimates against a 25-year (2000–2024) historical baseline using two complementary drought indices. The first is a quintile-based Rainfall Index that classifies conditions as Drier, Dry, Typical, Wet, or Wetter relative to the historical distribution for that specific location and calendar month. The second is the Pacific ENSO Applications Climate Center (PEAC) 4/8-inch index, which applies fixed monthly thresholds to flag periods of potential risk to water supply and catchment systems regardless of local climatic context. Building on the Marshall Islands monitoring framework of Senay and others (2023), the expanded protocol introduces 3- and 6-month rolling precipitation indices to detect developing seasonal drought and prolonged hydrological stress, which single-month indices cannot capture reliably.

IMERG

NASA produces IMERG in three processing streams with increasing latency and accuracy: the Early Run (available within hours), the Late Run (1-day lag, incorporating additional satellite inputs), and the Final Run (1–6 month lag, incorporating monthly rain-gauge analyses). The Early Run product is used here because its latency is compatible with operational monthly reporting workflows. Early Run data for a complete calendar month can be processed within days of the month's end, enabling timely delivery of drought index updates. For each monitoring location, a polygon shapefile delineating the relevant atoll or basin boundary was used to define the spatial domain of precipitation extraction. Zonal statistics (mean of all pixels intersecting the monitoring polygon) were computed using the R statistical computing environment (R Core Team, 2024) with the terra (Hijmans, 2024) and raster spatial processing packages.

Daily IMERG Early Run values are aggregated to monthly totals by summing all valid daily estimates within the calendar month for each monitoring polygon. Processing is parallelized across years using the future.apply R package (Bengtsson, 2021), with three concurrent sessions, substantially reducing computation time for the full 2000–2026 record. Monthly totals and per-location time-series data are exported to comma-separated value (CSV) files for downstream index computation and visualization.

Historical Baseline

A 25-year baseline (2000–2024) was chosen to calculate statistical reference distributions for each location and calendar month, providing enough data to capture seasonal and ENSO-driven precipitation variability. This period covers several El Niño and La Niña cycles and includes a balanced mix of these conditions, ensuring the baseline quintile thresholds reflect realistic climate variation at each site.

Monthly precipitation at Pacific Island sites tends to be right-skewed, especially in dry areas or seasons with low rainfall but occasional heavy convective events. For this reason, nonparametric quintile statistics are used to characterize the baseline in preference to parametric approaches based on the normal distribution ( Senay et al., 2023). The monthly median is used as the measure of central tendency rather than the mean, as the median is more resistant to the influence of extreme high-rainfall months and provides a more stable estimate of typical precipitation.

Drought Indices

Quintile-Based Rainfall Index: 1-Month

The 1-month Rainfall Index compares each location's precipitation for the current month to quintiles calculated from 25 years of historical data specific to that site and calendar month. These quintile thresholds are determined separately for each location and month, accounting for notable differences in rainfall patterns by area and season. There are five classification groups, each defined by its corresponding quintile range as follows:

- Drier — Below the 20th percentile (bottom quintile): precipitation is among the lowest 20 percent of observations for that location and month in the 25-year baseline

- Dry — 20th to 39th percentile (second quintile): precipitation is below the median but above the Drier threshold

- Typical — 40th to 59th percentile (middle quintile): precipitation spans the median and represents the most common range of historical observations

- Wet — 60th to 79th percentile (fourth quintile): precipitation exceeds the median but is below the Wetter threshold

- Wetter — Above the 80th percentile (top quintile): precipitation is among the highest 20 percent of observations for that location and month

Each classification category is rendered as a distinct color in the monthly report charts and summary tables to enable rapid visual assessment of conditions across the monitoring network. It is important to recognize that the Rainfall Index characterizes relative, location-specific conditions. A chronically dry site may receive very little precipitation in absolute terms but still be classified as Typical if the observed total falls within the middle quintile of its own historical record. For this reason, the Rainfall Index is always interpreted alongside the absolute PEAC 4/8-inch index described later below.

Quintile-Based Rainfall Index: 3-Month and 6-Month Rolling

The original Marshall Islands monitoring protocol (Senay et al., 2023) employed a 1-month Rainfall Index as its primary drought classification tool. While the 1-month index provides sensitive detection of immediate precipitation anomalies, it is susceptible to month-to-month variability and does not characterize the accumulation of precipitation deficits across multiple months — the hydrological process most directly linked to drought impacts on water supply systems, groundwater recharge, and ecosystem health.

To address this limitation, 3- and 6-month rolling precipitation indices were introduced as a methodological enhancement in the expanded Pacific monitoring protocol. For each location and each month in the time series, 3- and 6-month rolling precipitation sums are computed by summing precipitation totals over the preceding 3 or 6 calendar months respectively. Rolling sums are computed using the rollsum function from the zoo R package with right alignment, such that the rolling total for a given month encompasses that month and the preceding 2 or 5 months. Historical quintile thresholds for the rolling sums are then derived from the corresponding 25-year sequences of rolling totals, computed independently for each location and terminal calendar month, yielding the same five-category classification (Drier through Wetter) with thresholds appropriate to the multi-month accumulation context.

The interpretive significance of the 3- and 6-month indices differs from the 1-month index in important ways. Persistent Dry or Drier classification across the 3- and 6-month indices indicates that cumulative precipitation deficits extend across multiple consecutive months, signaling elevated risk of hydrological drought that may affect reservoir storage, groundwater recharge, and catchment replenishment. A single Dry month that is preceded by Wet months will typically not produce a Dry 3-month index, distinguishing a transient deficit from a sustained one. Conversely, a month classified as Typical in the 1-month index may contribute to a Dry 3- or 6-month index if the preceding months were also below the median, providing an early warning of developing multi-month stress that would not be apparent from the 1-month index alone.

PEAC 4/8-inch Index

The Pacific ENSO Applications Climate Center (PEAC) 4/8-inch index applies absolute monthly precipitation thresholds to classify conditions in terms of potential adequacy for municipal and household water supply systems. The thresholds derive from long-standing operational guidance used by PEAC in their regional monitoring updates and reflect empirical relationships between monthly rainfall totals and drought impacts documented across the U.S.-affiliated Pacific Islands (PEAC, 2015). The classification criteria are:

- Red — Monthly precipitation less than 4 inches (101.6 mm): elevated risk to agricultural systems, rain catchment drawdown, and shallow groundwater depletion; wildfire risk increases substantially

- Yellow — Monthly precipitation 4.1–8 inches (101.6–203.2 mm): marginal conditions; may not fully sustain or replenish municipal catchment and surface water supply systems

- Dark Green — Monthly precipitation greater than 8.1 inches (203.2 mm): adequate rainfall to sustain and replenish municipal and home rain catchment systems, maintain groundwater recharge, and greatly reduce wildfire risk

These thresholds are applied uniformly across all nine locations and all calendar months, regardless of each site's climatological context. This means that the PEAC index and the Rainfall Index will diverge for locations where typical precipitation levels fall near the 4-inch or 8-inch thresholds. For example, at Majuro or Nauru during their characteristically drier months, precipitation may be classified as Typical by the Rainfall Index (near the historical median for that location and month) while simultaneously receiving less than 8 inches and thus receiving a Yellow PEAC classification. This divergence is informative rather than contradictory: it reflects the chronic, baseline condition of marginal water supply adequacy at such locations, which persists even during meteorologically typical months (Senay et al., 2023). The two indices are therefore complementary: the Rainfall Index characterizes relative anomaly with respect to local norms, while the PEAC index provides an absolute, operationally meaningful signal independent of local climate.

Developing the Monitoring Report Template

Chart Design and Visual Elements

The visual core of each monthly report is a composite graphic for each monitoring location consisting of a precipitation time-series chart and an adjacent summary table. The chart format was designed to communicate simultaneously the magnitude of current-year precipitation, the historical statistical context from the 25-year baseline, and the drought index classification for the most recent month -- all within a single, compact graphic suitable for rapid interpretation by both technical and non-technical audiences.

The precipitation chart (Figure 3) contains the following elements displayed on a monthly x-axis spanning January through December of the current year:

- Orange bars — Monthly IMERG Early Run precipitation totals (in millimeters) for the current reporting year, January through the most recently completed month. Months with no data (future months) are blank. Bar heights reflect mean precipitation across the monitoring polygon.

- Blue line — The long-term monthly median derived from the 2000–2024 IMERG baseline, plotted at each calendar month position. The median provides a stable reference for whether current precipitation is above or below the historical central tendency, and is more resistant to the influence of extreme events than the mean.

- Gray ribbon — The interquartile range (IQR; 25th to 75th percentile) of the 2000–2024 baseline for each calendar month, rendered as a semi-transparent filled band. The ribbon defines the zone within which the middle half of historical observations fell. Precipitation falling below the lower ribbon boundary was in the driest 25 percent of historical observations; precipitation above the upper boundary was in the wettest 25 percent. The ribbon provides rapid visual context for whether current-year bars are within, below, or above the typical historical range.

The y-axis is scaled dynamically for each location to the maximum of the historical Q3 and current-year precipitation values, with major gridlines and tick marks at 25-millimeter intervals. The x-axis displays abbreviated month-year labels (for example, Jan-2026).

Figure 4. Example precipitation monitoring chart for a single location, Kosrae in the FSM. Orange bars represent current-year monthly totals; the blue line is the long-term monthly median; the gray ribbon is the interquartile range of the 2000–2024 baseline. The summary table to the right reports the current-month precipitation, long-term median, and all three drought index classifications for the most recently completed month.

Summary Table and Index Display

Adjacent to each precipitation chart, a tabular summary presents six statistics for the most recently completed reporting month at that location:

- Current-month precipitation in millimeters (for example, February-2026 Precipitation)

- Long-term monthly median precipitation in millimeters from the 2000–2024 baseline

- 1-Month Rainfall Index (Drier / Dry / Typical / Wet / Wetter)

- 3-Month Rainfall Index (Drier / Dry / Typical / Wet / Wetter)

- 6-Month Rainfall Index (Drier / Dry / Typical / Wet / Wetter)

- PEAC 4/8-inch Index, displayed as a color-filled rectangle (Red / Yellow / Dark Green) without a text label, providing an immediate visual signal consistent with the color conventions in existing PEAC operational reporting products

The Rainfall Index categories for the 1-, 3-, and 6-month indices are expressed as text labels in the table rather than color fills, to ensure clarity for printed and grayscale reproduction. The PEAC index cell uses a color fill rather than text because PEAC color conventions have been in operational use for several decades and the colors themselves carry recognized meaning for disaster, water security, and food security managers in the Pacific (PEAC, 2015; Senay et al., 2023).

Automated 7-Day Precipitation Forecasting

An automated, end-to-end pipeline was developed in R to generate 7-day precipitation forecasts for nine Pacific island locations using output from the NOAA Global Forecast System (GFS). The pipeline acquires gridded forecast data, aggregates 6-hourly accumulations to daily totals, extracts island-level zonal statistics, produces a suite of visualization and tabular products, and distributes the results to stakeholders via email. The system is designed for unattended daily execution under the Windows Task Scheduler.

Input Data

GFS forecast data. The GFS is NOAA’s primary global deterministic numerical weather prediction model, producing forecasts four times daily (00Z, 06Z, 12Z, 18Z) on a 0.25° × 0.25° global grid (~25 km at the equator). The model employs the Finite-Volume Cubed-Sphere (FV3) dynamical core  (Harris et al., 2020) with scale-aware simplified Arakawa–Schubert deep convection (Han et al., 2017). Output is distributed in GRIB2 format. The pipeline uses the 6-hour accumulated total precipitation variable at forecast hours 6 through 168 (28 files per model run), accessed from the NOAA GFS archive on Amazon Web Services S3 (primary) with the NCEP NOMADS server as a fallback source.

Spatial reference data. Island boundaries are defined by a custom polygon shapefile in the WGS 84 geographic coordinate system (EPSG:4326). The shapefile contains dissolved features for each target location, used as zones for spatial averaging and as boundary overlays on map products.

Target locations. Nine locations across four Pacific nations are monitored: Majuro (Marshall Islands); Pohnpei, Kosrae, Weno, and Yap (Federated States of Micronesia); Noro, Munda, and the Noro–Munda Basin (Solomon Islands); and Nauru.

Processing Workflow

The automated processing pipeline implements a multi-stage workflow executing data acquisition, spatial processing, statistical analysis, visualization generation, and product distribution (Table 2). The workflow proceeds as follows:

- Forecast Run Selection: The system queries the NOMADS directory listing to identify available GFS model runs from the most recent three days. For each date, all four initialization cycles are tested in reverse order (18Z, 12Z, 06Z, 00Z). The most recent run containing all 28 required forecast files (hours 006–168 at 6-hour intervals) is selected. If no complete run is available, the partial run with the greatest number of available files is used and a warning is logged.

- Data Acquisition: Individual GRIB2 files are downloaded via HTTP GET requests using the httr package, with automatic retry logic (up to 5 attempts per file) and a 600-second timeout. AWS S3 is attempted first; if the download fails or returns a non-200 status, the NOMADS URL is tried. Files are cached in a temporary directory, and previously downloaded files with valid size (>1 MB) are reused to avoid redundant transfers.

- Temporal Aggregation: The 6-hourly precipitation accumulations are summed into daily totals by grouping four consecutive 6-hour forecast steps per day, producing up to 7 daily precipitation raster layers.

- Zonal Extraction: Mean areal precipitation for each target island is computed using terra::extract() with the dissolved shapefile polygons as extraction zones.

- Tabular and Statistical Output: Two CSV files are produced: (a) a wide-format summary table with islands as rows and forecast dates as columns, including a total precipitation column; and (b) a long-format table with one row per island per day, designed for statistical analysis. An HTML table is also generated with conditional cell formatting: daily precipitation values ≥50 mm and weekly totals ≥200 mm are highlighted with red background, serving as visual alerts for hazardous precipitation levels.

- Visualization Generation: Graphical products include a combined grouped bar chart (all islands) and individual island bar charts with country-contextualized titles. Regional precipitation maps are generated using the rasterVis levelplot function, cropped to the island bounding box plus a 0.75° buffer, with island boundary polygons overlaid. Individual island maps use a 0.25° buffer. Both regional and island-level daily map sequences are compiled into animated GIF files at 1 frame per second using the magick package.

- Archiving and Distribution: All output files (CSVs, PNGs, GIFs, README, log) are packaged into a ZIP archive. An HTML email is composed using the blastula package, embedding the combined bar chart, regional animation, and the alert-highlighted summary table directly in the email body. The ZIP archive and summary CSV are included as attachments. The email is sent via SMTP with authentication credentials stored as environment variables.

Table 2. Key Processing Parameters

| Parameter | Value |
| --- | --- |
| GFS spatial resolution | 0.25° (~25 km) |
| Forecast hours acquired | 6 through 168, at 6-hour intervals (28 files) |
| Daily aggregation | Sum of four consecutive 6-hour accumulation layers |
| Maximum forecast length | 7 days |
| Zonal statistic | Mean areal precipitation per polygon |
| Daily alert threshold | ≥50 mm (red cell highlighting) |
| Weekly alert threshold | ≥200 mm total (red cell highlighting) |
| Map output resolution | 150 DPI (PNG) |
| Animation frame rate | 1 frame per second (GIF) |
| Download retry attempts | 5 per file, with 600-second timeout |

Software Environment

The pipeline is implemented in R (version 4.0+) using the following core packages: terra for raster processing; sf for vector spatial operations; tidyverse (dplyr, tidyr, readr, ggplot2) for data manipulation and visualization; rasterVis with latticeExtra for spatial map rendering; magick for GIF animation assembly; httr and curl for HTTP data acquisition with retry logic; kableExtra for conditional HTML table formatting; and blastula for SMTP email composition and delivery. Package dependencies are managed through pacman, which installs missing packages automatically at runtime.

Quality Assurance

Automated checks are applied at multiple pipeline stages. Downloaded files are validated against a minimum file size threshold (>1 MB) to reject incomplete or corrupted GRIB2 transfers. The forecast run selection algorithm verifies completeness before accepting a model run and falls back to partial runs with a logged warning. Precipitation raster layers are checked for valid data presence prior to zonal extraction. All processing milestones are recorded to a timestamped log file for post-hoc auditing. Critical failures (e.g., no GRIB files downloaded, no precipitation data found) trigger script termination with descriptive error messages.

Informal Settlements Mapping

Pacific island cities face accelerating urbanization, climate exposure, and natural hazard risk, yet comprehensive, up-to-date building inventory data remain scarce across much of the region. This part of the report documents a deep-learning-based workflow developed to extract multi-temporal building footprints from very-high-resolution (VHR) commercial satellite imagery across six informal settlement sites in three Pacific nations: Papua New Guinea, Solomon Islands, and Fiji. The study sites were 8 Mile Settlement and Taurama Valley (Port Moresby, PNG), White River and Mbumburu (Honiara, Solomon Islands), and Tacirua and Wailekutu (Greater Suva, Fiji).  These sites were selected by ADB to represent a range of urban density, topographic complexity, and canopy cover conditions.

Imagery spanning three temporal epochs (approximately 2005, 2015, and 2025) was sourced from the QuickBird-2, WorldView-2, and WorldView-3 sensor archive. A seven-stage processing workflow was implemented, encompassing image co-registration, Gram-Schmidt pan-sharpening, 8-bit RGB composite preparation, deep learning instance segmentation using the Esri Mask R-CNN BuildingFootprintExtraction model, post-processing, and manual quality assurance through systematic digitizing. The Esri Africa model was applied at Solomon Islands and Fiji sites; the Esri USA model produced the best results at the particularly challenging PNG sites. Post-processing included coastal clipping, minimum area filtering, geometry repair, and polygon regularization. Systematic manual editing was performed to recover missed buildings and remove false positives.

Table 3 provides a summary of all study sites, their respective nations, survey epochs, and sensor sources. Survey epochs differ between nations due to image availability: PNG sites use 2010/2016/2025, Fiji sites use 2009/2014/2025, and Solomon Islands sites use 2006/2015/2025. Cross-site compound annual growth rate (CAGR) comparisons must explicitly account for these differing time spans.

Table 3. Study site summary - six informal settlement sites across three Pacific Island nations with survey epochs and sensor sources.

| Site ID | Settlement | Nation | City | Epoch 1 | Epoch 2 | Epoch 3 | Sensors |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PNG-1 | 8 Mile Settlement | Papua New Guinea | Port Moresby | ~2010 | ~2016 | 2025 | WV-2 / WV-2 / WV-3 |
| PNG-2 | Taurama Valley | Papua New Guinea | Port Moresby | ~2010 | ~2016 | 2025 | WV-2 / WV-2 / WV-3 |
| SOL-1 | White River | Solomon Islands | Honiara | ~2006 | ~2015 | 2025 | QB-2 / WV-3 / WV-3 |
| SOL-2 | Mbumburu | Solomon Islands | Honiara | ~2006 | ~2015 | 2025 | QB-2 / WV-3 / WV-3 |
| FIJ-1 | Tacirua | Fiji | Greater Suva | ~2009 | ~2014 | 2025 | QB-2 / WV-2 / WV-3 |
| FIJ-2 | Wailekutu | Fiji | Greater Suva | ~2009 | ~2014 | 2025 | QB-2 / WV-2 / WV-3 |

Data and Imagery

Satellite Imagery Sources

Three commercial VHR sensor platforms were used across the three temporal epochs. QuickBird-2 imagery was used for the earliest epoch (~2005/2009/2010), WorldView-2 for the intermediate epoch (~2015/2014/2016), and WorldView-3 for the most recent epoch (2025) at all sites. Table 4 summarizes the spatial resolution specifications of each sensor.

Table 4. Spatial resolution specifications for the three VHR sensor platforms used in this study

| Sensor | Panchromatic (m) | Multispectral (m) | MS Bands | Operational |
| --- | --- | --- | --- | --- |
| QuickBird-2 | 0.61 | 2.44 | 4-band (B, G, R, NIR) | 2001–2015 |  |
| WorldView-2 | 0.46 | 1.84 | 8-band MS | 2009–present |
| WorldView-3 | 0.31 | 1.24 | 8-band MS + SWIR | 2014–present |  |

All imagery was sourced from Vantor's Global Enhanced GEOINT Delivery Pro platform (GEGD Pro) commercial archive and delivered as orthorectified, surface-reflectance-corrected products. WorldView-3 imagery at 0.31 m panchromatic resolution sits comfortably within the 10–40 cm optimal input range of the Esri deep learning models; WorldView-2 at 0.46 m falls near the upper limit of this range and was resampled to 0.40 m before inference to improve model performance. QuickBird-2 at 0.61 m is marginally above the model's specified optimal range.

Study Area Boundaries

Settlement boundary delineations for all six sites were provided by the Asian Development Bank as polygon shapefiles. These boundaries define the spatial extent within which all building counts, density estimates, and coverage metrics are computed. Wailekutu is represented by two distinct polygons within a single merged shapefile and building footprints within both polygons are treated as a combined dataset for all metric calculations.

Image Co-registration

Accurate temporal change analysis requires sub-pixel positional alignment between multi-date images. Image co-registration was performed using the AROSICS automated co-registration library (Python, Miniforge environment) with the 2025 WorldView-3 scene as the positional reference and all earlier epochs as targets. Band 4 (near-infrared) was used as the matching channel due to its stronger structural contrast relative to visible bands in tropical environments.

Co-registration results differed by site. Most sites achieved sub-pixel accuracy globally, but AROSICS encountered memory failures and erroneous global shifts, leaving some scenes unresolved and needing improvement.

Building footprint extraction workflow

The building footprint extraction workflow consists of seven sequential stages, summarized in Table 5.

Table 5. Each stage is described in detail below.

| Stage | Operation | Key Parameters / Notes |
| --- | --- | --- |
| 1 | Image Co-registration | AROSICS (Python); Band 4 NIR reference; |
| 2 | Pan-sharpening | Gram-Schmidt algorithm; fuses PAN + all MS bands to sub-meter resolution |
| 3 | RGB Composite (8-bit) | Red-Green-Blue bands; linear stretch; rescale to 8-bit unsigned |
| 4 | Deep Learning Detection | Esri Mask R-CNN (Africa/USA model); sliding window inference |
| 5 | Post-processing | AOI clip, area filter, Repair Geometry, Regularize Building Footprints |
| 6 | Manual QA/QC | Systematic editing; add missed, remove false positives |
| 7 | Metric Computation | Building count, density, coverage, CAGR; R statistical analysis pipeline |

Image Preprocessing

Pan-sharpening

Pan-sharpening was performed in ArcGIS Pro (version 3.5.6, Image Analyst extension) using the Gram-Schmidt spectral sharpening algorithm, which is considered optimal for WorldView-2 and WorldView-3 multispectral data due to its minimal color distortion at full panchromatic resolution. The algorithm produces an output at the spatial resolution of the panchromatic band (0.31 m for WorldView-3; 0.40–0.46 m for WorldView-2 after resampling; 0.61 m for QuickBird-2) with spectral content from all multispectral bands.

RGB Composite

Following pan-sharpening, a three-band Red-Green-Blue (RGB) composite was prepared for model input. For both WorldView-2 and WorldView-3 sensors, the correct band assignments are: Red = Band 5, Green = Band 3, Blue = Band 2. This is a non-default ordering that must be explicitly specified; use of default band ordering (R=1, G=2, B=3) produces an incorrect false-color composite that substantially degrades model performance.

Deep Learning Building Detection

Model Selection

Two Esri pretrained Mask R-CNN models were evaluated across the study sites. The Africa Building Footprint Extraction model (BuildingFootprintExtraction_Africa.dlpk) was applied at Solomon Islands and Fiji sites. This model uses a ResNet-50 + Feature Pyramid Network backbone trained on sub-meter imagery from diverse African urban environments and achieves a reported average precision (AP) of 0.786 on its validation dataset. It was selected for tropical Pacific island applications based on similarity in building density, roofing material diversity, and informal settlement patterns between African and Melanesian urban typologies.

For the Papua New Guinea sites, all three available Esri regional models - Africa, Australia, and USA - were evaluated. The USA Building Footprint Extraction model produced the best detection results at the PNG sites, particularly at the challenging 8 Mile Settlement. This is attributed in part to the USA model's training on diverse suburban typologies including dry, low-vegetation landscapes analogous to parts of the dry season Port Moresby environment. Despite this improvement, PNG detection results still required extensive manual editing.

Detection Parameters

The Detect Objects Using Deep Learning tool in ArcGIS Pro was used for all inference runs. Table 6 summarizes the final validated detection parameters applied at each country group. These parameters reflect iterative testing and represent the configurations that balanced detection completeness against computational feasibility on CPU-only hardware.

Table 6. Deep learning detection parameters applied by country group. Lower confidence threshold at PNG sites was necessary to maximize recall under dense canopy cover.

| Parameter | PNG Sites | Solomon Islands Sites | Fiji Sites |
| --- | --- | --- | --- |
| Model | USA Building Footprint | Africa Building Footprint | Africa Building Footprint |  |
| Confidence threshold | 0.20 | 0.50 | 0.50 |
| Tile size (pixels) | 256 | 256 | 256 |  |
| Padding (pixels) | 32 | 32 | 32 |
| Batch size | 1 | 1 | 1 |  |
| Test Time Augmentation | True | True | True |
| Merge policy | mean | mean | mean |  |
| NMS overlap ratio | 0.10 | 0.10 | 0.10 |
| Processing mode | CPU | CPU | CPU |  |

A lower confidence threshold of 0.20 was used for PNG sites to maximize building recall in conditions where most structures are obscured by tropical tree canopy and the model assigns lower confidence scores to partially visible rooftops. The resulting increase in false positives was managed through post-processing and manual QA. A threshold of 0.50 was retained for Solomon Islands and Fiji sites where canopy occlusion was less severe and detection quality was substantially better at this standard threshold.

Test Time Augmentation (TTA) was enabled at all sites. TTA runs each image tile through multiple geometric transformations (horizontal and vertical flips, rotations) and averages the resulting prediction scores, improving detection of buildings with non-standard orientations and producing smoother, more accurate polygon boundaries. Enabling TTA increased processing time but produced measurably better polygon quality compared to single-pass inference. The tile size of 256 pixels, reduced from an initial 512 pixels, was required to avoid memory exhaustion errors on CPU-only hardware during inference over full-scene AOIs.

Post-Processing Pipeline

Raw detection outputs required a five-stage post-processing pipeline before building counts and metrics could be computed. The stages were applied in fixed sequence at all sites.

Stage 1 -  AOI/Coastal Clip

Raw detection outputs were clipped to the ADB-provided settlement boundary polygons using the Clip geoprocessing tool. This step removed ocean false positives, off-settlement detections, and artefacts outside the study area. For Wailekutu, which spans two boundary polygons, the union of both polygons was used as the clip geometry so that detections within either polygon were retained.

Stage 2 - Area Filter

Detections with polygon areas below 12 m² were removed. This threshold was established through visual inspection of the smallest credible building footprints across the study sites and removes sub-building fragments, sensor noise artefacts, and false positives from small reflective surfaces. The same threshold was applied consistently across all sites and epochs to ensure comparability.

Stage 3 - Repair Geometry

The Repair Geometry tool was applied after area filtering to resolve self-intersecting polygons, null geometries, and topology errors that can accumulate during rasterization and polygon conversion steps. This step is necessary before using the Regularize Building Footprints tool, as it helps prevent issues caused by corrupt geometry in the input feature class.

Stage 4 - Regularize Building Footprints

The Regularize Building Footprints tool was applied with the RIGHT_ANGLES AND DIAGONALS method and a tolerance of 1.5 m. This tool snaps building polygon edges to rectilinear and diagonal angular patterns consistent with constructed buildings, removing the jagged, over-detailed boundaries produced by the deep learning segmentation step. A tolerance of 0.3 m, tested in early runs, produced boundaries that were still visually rough and required subsequent manual correction; 1.5 m was validated as producing clean, architecturally realistic footprint geometry without over-simplification of building outlines.

Stage 5 - Manual QA/QC

Systematic manual editing was performed as the final post-processing stage. Three primary editing operations were performed: deletion of confirmed false positives, addition of manually digitized footprints for missed buildings (particularly those occluded by tree canopy), and splitting of merged multi-building polygons. Manual QA was applied to all sites and all epochs, with priority given to areas of known high false-positive density and confirmed missed-detection zones.

Urban Growth Metric Computation

Four urban growth indicators were derived for each site and epoch from the QA-validated building footprint datasets, using an R-based statistical analysis pipeline. All metrics were computed within the ADB-provided settlement boundary area.

- Building count: Total number of individual building polygon features after post-processing and QA.

- Settlement density: Building count divided by settlement boundary area in hectares (structures/ha), benchmarked against the ADB Pacific informal settlement norm of 15–25 structures/ha.

- Built-up coverage: Ratio of total building footprint area to settlement boundary area, expressed as a percentage.

- Compound Annual Growth Rate (CAGR):  Computed between consecutive epochs as CAGR = (End Count ÷ Start Count)^(1 ÷ n years) − 1, benchmarked against the ADB Melanesian urban expansion norm of 3–4%/yr.

Seismic Risk

The Earth is seismically active because its tectonic plates are in constant motion. Most seismic and volcanic activity occurs along plate boundaries, where friction can temporarily lock plates in place until accumulated stress is released suddenly as an earthquake. Many of the most active regions, particularly around the Pacific “Ring of Fire” are located where one plate is subducting beneath another (Figure 5). Human exposure to seismic hazards depends on both the level of tectonic activity at these boundaries and the proximity of populations and infrastructure.

Seismic hazards pose significant risks to coastal infrastructure, transportation networks, energy systems, and public safety. Strong ground shaking can damage ports, roads, bridges, and utilities, while tsunamis can inundate low-lying coastal zones where critical facilities and communities are concentrated. These impacts can isolate populations, disrupt essential services, and slow emergency response and long-term recovery.

The most destructive tsunamis in recent history have been triggered by underwater earthquakes, although landslides, volcanic activity, atmospheric disturbances, and meteorite impacts can also generate them (Grezio and others, 2017). The most common source remains large earthquakes near subduction zones, where one tectonic plate descends beneath another. Tsunami waves can travel across entire ocean basins, making exposure to open ocean and local elevation key determinants of risk. For low-lying atolls, tsunami hazards may exceed the threat posed by ground shaking.

Figure 5. Pacific Plate boundaries and relative motion. Source: USGS. Public Domain.

PAGER Earthquake Reporting

The U.S. Geological Survey’s Prompt Assessment of Global Earthquakes for Response (PAGER) system is designed to deliver rapid, automated estimates of an earthquake’s potential human and infrastructural impacts within minutes of its occurrence. Immediately after a significant event, PAGER evaluates the number of people exposed to various levels of ground shaking and identifies shaking intensities in affected cities. These assessments are accompanied by maps showing population distribution and modeled shaking levels, providing an early overview of potential impact zones. Each PAGER report includes region-specific commentary describing the vulnerability of local building stock and, when relevant, summaries of recent nearby earthquakes and their effects. The results are posted on the USGS Earthquake Program website and distributed in near real time to emergency responders, government agencies, and the media as a concise, one-page situational awareness product.

Both automated and manual updates refine these assessments in the hours following an earthquake, incorporating improved estimates of event location, magnitude, fault geometry, and firsthand shaking reports. The system integrates data from multiple USGS and Advanced National Seismic System (ANSS) tools. Ongoing research aims to expand PAGER’s capabilities beyond exposure estimates toward near real-time predictions of fatalities, injuries, and displaced populations, further enhancing its value for emergency response and humanitarian planning (Earle and others, 2009).

Summary

This report synthesizes recent advances in applying remote sensing to water resource assessment, drought monitoring, and the mapping of informal settlements in Pacific Island communities. Satellite derived observations, combined with quality assured building footprint datasets, support the evaluation of urban growth, water availability, and exposure to natural hazards. Surface water availability was assessed using remotely sensed imagery to delineate drainage basins and estimate freshwater resources. Historical precipitation data were analyzed to identify long term trends and seasonal variability from 1998 to current, revealing that although the islands generally maintain a positive water balance, irregular rainfall patterns can result in acute water shortages. Drought indices helped pinpoint periods of heightened water stress and identify vulnerable communities in need of targeted mitigation.

Informal settlement mapping relied on QA-validated building footprint datasets refined through manual review to correct false positives and separate merged structures. For each site and time period, indicators such as building count, settlement density, built-up area, and Compound Annual Growth Rate (CAGR) were calculated and benchmarked against regional norms to quantify patterns of urban expansion.

Seismic risk analysis incorporated tectonic plate boundary data and historical seismic records to identify areas of elevated hazard within the Pacific “Ring of Fire.” Assessments considered settlement proximity to plate boundaries, local topography, and the effectiveness of existing early warning systems to determine population exposure to earthquakes and tsunamis.

Collaboration between the U.S. Geological Survey (USGS) and the Asian Development Bank (ADB) enabled the effective use of remote sensing tools to address both water insecurity and natural hazard risks. Together, the findings illustrate how population growth, infrastructure development, and environmental pressures intersect, offering a framework for resilience planning across Pacific Island communities.

References Cited

ADB, 1965, Agreement establishing the Asian Development Bank signed at Manila, Dec. 4, 1965; entered into force Aug. 22, 1966, United Nations Treaty Series, vol. 608, p. 380.  Available at: Agreement Establishing the Asian Development Bank (ADB Charter). Accessed 29 January 2026.

Bijeesh, T.V. and K.N. Narasimhamurthy. 2020. Surface water detection and delineation using remote sensing images: a review of methods and algorithms. Sustainable Water Resources Management (2020) 6: 68. https://doi.org/10.1007/s40899-020-00425-4.

Che, Lusheng, Shuangshuang Li, and Xianfeng Liu. 2025. Improved surface water mapping using satellite remote sensing imagery based on optimization of the Otsu threshold and effective selection of remote-sensing water index. Journal of Hydrology 654 (2025) 132771. https://doi.org/10.1016/j.jhydrol.2025.132771.

Grezio, A., Babeyko, A., Baptista, M. A., Behrens, J., Costa, A., Davies, G., Geist, E. L., Glimsdal, S., González, F. I., Griffin, J., Harbitz, C. B., and others. (2017). Probabilistic tsunami hazard analysis: Multiple sources and global applications. Reviews of Geophysics, 55, 1158–1198. https://doi.org/10.1002/2017RG000579

Hou, Jiawei, Albert I.J.M. van Dijk, and Hylke E. Beck. 2020. Global satellite-based gaging and the influence of river morphology on its application. Remote Sensing of Environment 239 (2020) 111629. https://doi.org/10.1016/j.rse.2019.111629.

Lin, Yaling and Shunqiao Song. 2026. Monitoring surface water in floodplains by satellites: Progress, challenges, and perspectives. Journal of Hydrology, 664 B, January 2026 134458. https://doi.org/10.1016/j.jhydrol.2025.134458.

Earle, P. S., Wald, D. J., Jaiswal, K., Allen, T. I., Hearne, M. G., Marano, K., Hotovec, A. J., and Fee, J. (2009). Prompt Assessment of Global Earthquakes for Response (PAGER): A system for rapidly determining the impact of earthquakes worldwide. U.S. Geological Survey Open-File Report 20091131, iii + 15 pp. https://doi.org/10.3133/ofr20091131