National Geospatial Program

Prepared in cooperation with the Asian Development Bank

Increasing Water Security on Pacific Islands Through Increased Data Availability and Understanding—Volume 2, Federated States of Micronesia

By Gabriel B. Senay,1 Francis K. Dwomoh,2 Stefanie Kagone,2 Tomas Smieszek1, and Matthew E. Andersen1

1U.S. Geological Survey.

2ASRC Federal, contractor to the U.S. Geological Survey (USGS) Earth Resources Observations and Science (EROS) Center, Sioux Falls, SD 57198, USA. Work performed under USGS Contract [140G0124D0001].

Any use of trade, firm, or product names is for descriptive purposes only and does not imply endorsement by the U.S. Government.

Although this information product, for the most part, is in the public domain, it also may contain copyrighted materials as noted in the text. Permission to reproduce copyrighted items must be secured from the copyright owner.

This draft manuscript is distributed solely for purposes of courtesy review and comments received will be addressed and treated as appropriate to ensure there is no conflict of interest. Its content is deliberative and predecisional, so it must not be disclosed or released by reviewers. Because the manuscript has not yet been approved for publication by the U.S. Geological Survey (USGS), it does not represent any official USGS finding or policy.

Acknowledgments

This work was conducted in collaboration with the Strengthening Resilient and Sustainable Urban and Water Service Delivery in the Pacific project of the Water and Urban Development Sector of the Asian Development Bank (ADB). ADB provided financial support from some USGS technical salaries and associated administrative costs, including some of the travel necessary to administer and manage the work.

Outline

Acknowledgments	2

Introduction	5

Purpose and Scope	6

Federated States of Micronesia (FSM)	7

Island of Kosrae	12

Precipitation	14

Evapotranspiration	17

Water Balance	20

Drought Monitoring	22

Automated 7-Day Precipitation Forecasting	25

Seismicity	35

Island of Pohnpei	35

Precipitation	37

Evapotranspiration	39

Water Balance	42

Drought Monitoring	44

Seismicity	46

Island of Yap	47

Precipitation	49

Evapotranspiration	51

Water Balance	54

Drought Monitoring	57

Seismicity	60

City of Weno in the Chuuk Lagoon	61

Precipitation	62

Evapotranspiration	65

Water Balance	68

Drought Monitoring	71

Seismicity	74

Discussion	74

Conclusion	77

References	78

Figures

Figure 1. Map of Western Pacific Ocean Island Nations. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.	8

Figure 2. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.	10

Figure 3. Map of Kosrae. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.	13

Figure 4. Monthly precipitation (P) for Kosrae (1998 – 2025) from IMERG.	16

Figure 5. Annual precipitation (P) for Kosrae (1998 – 2024) from IMERG.	17

Figure 6. Mean monthly precipitation (P) and actual ET (ETa) in mm for Korsrae (2013-2025).	18

Figure 7. Annual actual ET (ETa) for Kosrae in mm for years 2013 – 2025 using SSEBop model.	19

Figure 8. Monthly relative net water availability (P – ETa) for Kosrae (2012 – 2025) in mm.	20

Figure 9. Annual Water Budget for Kosrae (2013-2024).	21

Figure 10. Quintile-based Rainfall Index time series for Kosrae showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows.	23

Figure 11. Combined 7-day GFS precipitation forecast bar chart for the four FSM monitoring locations, March 10–16, 2026. The dashed red line indicates the 50-mm daily alert threshold.	28

Figure 12. GFS 7-day precipitation forecast bar chart for Yap, March 10–16, 2026. Bars show daily accumulated precipitation in millimeters.	29

Figure 13. GFS 7-day precipitation forecast bar chart for Weno (Chuuk), March 10–16, 2026.	30

Figure 14. GFS 7-day precipitation forecast bar chart for Pohnpei, March 10–16, 2026.	31

Figure 15. GFS 7-day precipitation forecast bar chart for Kosrae, March 10–16, 2026.	32

Figure 16. Map of Pohnpei. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.	36

Figure 17. Monthly precipitation for years 1998 - 2025 in mm.	38

Figure 18. Annual precipitation for years 1998 - 2024 in mm.	39

Figure 19. Mean monthly precipitation (P) and actual ET (ETa) in mm for Pohnpei (2013-2025).	40

Figure 20. Annual actual ET for years 2013 - 2025 in mm.	41

Figure 21. Monthly net water availability for years 2012 - 2025 in mm.	43

Figure 22. Net water availability per year in mm.	44

Figure 23. Quintile-based Rainfall Index time series for Pohnpei showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows.	45

Figure 24. Map of Yap. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.	48

Figure 25. Monthly precipitation for years 1998 - 2025 in mm.	50

Figure 26. Annual precipitation for years 1998 - 2024 in mm.	51

Figure 27. Mean monthly precipitation (P) and actual ET (ETa) in mm for Yap (2013-2025).	52

Figure 28. Annual actual ET for years 2013 - 2025 in mm.	53

Figure 29. Monthly net water availability for years 2012 - 2025 in mm.	55

Figure 30. Net water availability per year in mm.	56

Figure 31. Quintile-based Rainfall Index time series for Yap showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows. Bars represent observed precipitation (or rolling sums) color-coded by quintile category. The dashed line shows the baseline median for each calendar month.	58

Figure 32. Map of the Island Weno with city Weno in the Chuuk Lagoon. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.	62

Figure 33. Monthly precipitation for years 1998 - 2025 in mm.	64

Figure 34. Annual precipitation for years 1998 - 2024 in mm.	65

Figure 35. Mean monthly precipitation (P) and actual ET (ETa) in mm for Weno (2013-2025).	66

Figure 36. Annual actual ET for years 2013 - 2025 in mm.	67

Figure 37. Monthly net water availability for years 2012 - 2025 in mm.	69

Figure 38. Net water availability per year in mm.	70

Figure 39. Quintile-based Rainfall Index time series for Weno (Chuuk) showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows.	72

Tables

International System of Units to U.S. customary units

Table 1. IMERG Precipitation Summary (1998–2024) for Kosrae

Table 2. Summary of notable drought episodes at Kosrae.

Table 3. Alert thresholds applied to GFS precipitation forecasts.

Table 4. GFS 7-day precipitation forecast (mm) for FSM study locations, March 10–16, 2026. GFS model run 20260310, 12z cycle. No daily or weekly alert thresholds were exceeded.

Table 5. Mean IMERG Precipitation Statistics (1998–2024) for Pohnpei

Table 6. Summary of notable drought episodes at Pohnpei.

Table 7. Mean IMERG precipitation statistics (1998–2024) for Yap

Table 8. Summary of notable drought episodes at Yap, defined as three or more consecutive months classified as Drier or Dry at the 1-month Rainfall Index.

Table 9. Mean IMERG precipitation statistics (1998–2024) for Weno

Table 10. Summary of notable drought episodes at Weno (Chuuk).

Conversion Factors

International System of Units to U.S. customary units

| Multiply | By | To obtain |
| --- | --- | --- |
| Length |  |  |
| millimeter (mm) | 0.03937 | inch (in.) |
| meter (m) | 3.281 | foot (ft) |
| kilometer (km) | 0.6214 | mile (mi) |
| Area |  |  |
| square meter (m2) | 0.0002471 | acre |
| hectare (ha) | 2.471 | acre |
| square hectometer (hm2) | 2.471 | acre |
| square kilometer (km2) | 247.1 | acre |
| square centimeter (cm2) | 0.001076 | square foot (ft2) |
| square meter (m2) | 10.76 | square foot (ft2) |
| square centimeter (cm2) | 0.1550 | square inch (in2) |
| square hectometer (hm2) | 0.003861 | section (640 acres or 1 square mile) |
| hectare (ha) | 0.003861 | square mile (mi2) |
| square kilometer (km2) | 0.3861 | square mile (mi2) |

Abbreviations

ADB	Asian Development Bank

FSM	Federated States of Micronesia

USGS	U.S. Geological Survey

ET	Evapotranspiration

P	Precipitation

ENSO	El Niño–Southern Oscillation

Introduction

Freshwater is critically important for life across the Pacific islands. Utilities, resource managers, and the public increasingly require reliable hydrologic information to support water resource planning, reduce the impacts of droughts, and improve the long-term security of freshwater supplies. The Asian Development Bank (ADB), whose mission includes strengthening economic resilience and supporting sustainable development across Asia and the Pacific (ADB, 1965), recognizes that dependable access to safe water is fundamental to public health and economic stability.

If water managers better understand the timing, magnitude, and variability of freshwater availability, they can more effectively allocate resources, plan infrastructure improvements, and anticipate periods of deficit. Potential development actions, such as expanding water storage capacity, hardening supply systems, or improving watershed protection, depend on having consistent and accessible hydrologic information. Satellite based datasets provide an efficient means of meeting these needs across the region’s many small, remote, and data sparse islands.

To address these challenges, ADB and the U.S. Geological Survey (USGS) partnered to evaluate how Earth observation products can support water resource management across the Federated States of Micronesia (FSM). The collaboration focuses on analyzing long-term satellite datasets for precipitation, evapotranspiration (ET), seasonal rainfall variability, drought monitoring, and short-term precipitation forecasting indicators. The methods used to generate these datasets are detailed in Volume 1 – Overview of this series, while Volume 2 applies those methods to the four priority population centers in FSM of Kosrae, Pohnpei, Yap, and Weno in the Chuuk Lagoon.

Purpose and Scope

This report presents the results of a collaborative effort by the USGS and the ADB to assess the applicability of remotely sensed Earth observation data for improving understanding of water resource conditions across the Federated States of Micronesia. The work supports regional efforts to address persistent water availability challenges on Pacific islands, where freshwater supplies can be limited for extended periods.

This volume documents the analytical components of the study and provides results for four priority population centers: Kosrae, Pohnpei, Yap, and the city of Weno in the Chuuk Lagoon. The scope of the report includes satellite-based assessments of precipitation, evapotranspiration, seasonal rainfall variability, drought monitoring indicators, and automated 7-day precipitation forecasting. Methods used to generate these datasets are presented in Volume 1 of this series; the present volume focuses on applying those methods within FSM and summarizing the resulting water availability information.

Each chapter describes the datasets used, outlines key processing steps, and presents the derived products for the respective study areas. The information provided is intended to assist water managers, utilities, and planners in evaluating hydrologic conditions, identifying periods of relative water surplus or shortage, and supporting decisions related to water conservation, infrastructure planning, and resource management.

Federated States of Micronesia (FSM)

Beginning in 2024, the USGS entered a collaborative relationship with ADB to assess the utility of various remotely sensed datasets for improving water resource management within the FSM, which spans nearly 2,700 kilometers across the western Pacific between roughly 1° and 10° north latitude, encompassing more than 600 islands with substantial variation in hydrologic characteristics (Figure 1).

Figure 1. Map of Western Pacific Ocean Island Nations. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Western Pacific Ocean island nations. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

High volcanic islands such as Kosrae and Pohnpei support dense cloud forests, steep catchments, and perennial stream networks that respond rapidly to intense rainfall events (Raynor, 1995). In contrast, the low-lying atolls of Yap State and the Chuuk Lagoon rely primarily on groundwater from thin freshwater lenses that are highly sensitive to changes in precipitation, evapotranspiration, and sea level fluctuations (Werner et al., 2017). This broad geographic distribution across approximately 3 million km² of ocean exposes FSM to large spatial and temporal gradients in rainfall associated with the Intertropical Convergence Zone (ICZ), trade wind patterns, and ENSO driven (El Niño–Southern Oscillation) drought cycles which are key factors limiting water reliability for communities with minimal surface water or storage capacity. A regional map of mean precipitation over the last 10 years highlights the increased precipitation amounts across the FSM (Figure 2).

Figure 2. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

Alt. text. Graphical representation of mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

In consultation with FSM water managers, ADB identified Kosrae, Pohnpei, Yap, and the city of Weno in the Chuuk Lagoon as priority locations for expanding precipitation (P), actual evapotranspiration (ETa), water balance, and drought monitoring datasets to reflect on areas where hydrologic vulnerability, population density, and exposure to climate and geologic variability are greatest. The water balance or net water availability is computed as difference between P and ETa (P- ETa)) at monthly and annual scales. Although change in storage can be considered negligible at annual time scales, the monthly water balance does not take into account for storage changes in the system and thus should only be consulted as relative indicators of P and ETa.

The drought monitoring system uses monthly precipitation estimates from the IMERG Early Run dataset and evaluates those estimates against a historical baseline (1998–2025) using two complementary drought indices. The first is a quintile-based Rainfall Index that classifies precipitation conditions such as Drier, Dry, Typical, Wet, or Wetter relative to the historical distribution for each specific location and calendar month, computed at 1-month, 3-month, and 6-month accumulation windows. The second is the Pacific ENSO Applications Climate Center (PEAC) 4/8-inch (10.2 cm/20.4 cm) index, which applies fixed absolute monthly precipitation thresholds to flag potential risk to water supply systems.

According to USGS seismic data, the FSM is considered a region of low to moderate seismicity, experiencing infrequent damaging earthquakes and relatively limited population exposure. Despite this, FSM is situated within the broader, tectonically active western Pacific, where significant regional earthquakes can still occur, and the westernmost islands of Yap state are at greatest risk from seismic activity. The states occupy a complex tectonic setting influenced by the interactions among the Pacific Ocean, Philippine Sea, and Caroline Plates. Although the Caroline Plate itself moves relatively slowly, its boundary regions are affected by nearby subduction zones and transform faults, which occasionally generate regional seismic activity. These tectonic processes not only shape the long-term morphology of the islands and influence watershed evolution on volcanic islands but also contribute to hazards such as landslides and coastal subsidence. Such hazards can impact freshwater infrastructure and groundwater storage (U.S. Geological Survey, 2021).

The Federated States of Micronesia’s diverse island environments present unique hydrologic, climatic, and geologic challenges, which require tailored approaches for water resource monitoring and management. The following sections provide focused analyses for each island, starting with Kosrae, to address their specific characteristics and vulnerabilities.

Island of Kosrae

Kosrae, the easternmost state of the Federated States of Micronesia, is a high volcanic island located near 5°N and 163°E and is the most topographically elevated of the FSM study sites (Figure 3).

Figure 3. Map of Kosrae. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A simplified topographic map showing the island of Kosrae. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

The island is characterized by steep, densely forested slopes, deeply incised stream valleys, and a central mountainous interior that rises to more than 600 meters, creating strong orographic enhancement of rainfall (Van der Brug, 1984). As a result, Kosrae receives some of the highest annual precipitation in the region, supporting perennial streamflow, extensive wetland areas, and saturated soils that respond rapidly to storm events. Freshwater availability is generally high compared to the other islands, though limited storage capacity and short watershed travel times can make the island vulnerable to short‑term droughts. Kosrae’s population is concentrated primarily in coastal communities and relies on a combination of surface‑water intakes and small groundwater systems that are sensitive to changes in rainfall, land use, and sea‑level conditions. Geologically, Kosrae sits near the boundary of the slowly moving Caroline Plate, whose interactions with the adjacent Pacific Plate contribute to regional seismicity and the long‑term uplift and erosion patterns that shape the island’s steep volcanic landscape. Although major earthquakes are infrequent locally, regional tectonic activity influences slope stability, coastal processes, and the resilience of water‑resource infrastructure.

Precipitation

Precipitation (P) is the primary source of freshwater across Pacific Island environments, because islands lack the buffering capacity of large watersheds or reservoirs (Tribble, 2008). Therefore, understanding spatial and temporal rainfall variability is essential for evaluating water availability, assessing drought risk, and guiding sustainable management of hydrologic resources. Satellite derived precipitation datasets provide a consistent means of monitoring these patterns across remote and data sparse island settings.

Precipitation patterns for Kosrae, and the other islands, were derived using the Integrated Multi-satellitE Retrievals for GPM (IMERG) precipitation dataset (Huffman and others, 2019). After the data was acquired, the island average for every day from 1 January 1998 to 31 September 2025 was created and can be accessed from (USGS Data release).

Kosrae has a humid, high-precipitation tropical climate dominated by orographic rainfall, with extremely wet conditions year-round but still susceptible to short drought periods. The mean annual total precipitation is 4063 mm/yr (Table 1) and the mean monthly precipitation is 339 mm. The average minimum precipitation in a month is never below 120mm for the 27-year period which is expected for the island’s climate while the mean maximum precipitation in a month is 520mm.

Table 1. Mean IMERG Precipitation Summary (1998–2024) for Kosrae

| Statistic | Precipitation total (mm) |
| --- | --- |
| Mean annual | 4063 |
| Mean monthly mean | 339 |
| Mean monthly minimum | 120 |
| Mean monthly maximum | 520 |

Kosrae receives substantial precipitation throughout the year, with monthly rainfall totals typically ranging from roughly 300 to more than 600 millimeters, reflecting the island’s persistently wet tropical climate (Leeling, 1996). Although rainfall occurs in all seasons, a distinct pattern is evident, with enhanced precipitation commonly observed during April through July and again during November and December. Several months exceed 700 mm, such as April 2019 (746 mm) and May 2021 (849 mm), and an exceptional maximum of more than 1,000 mm occurred in December 2001, indicating the potential for intense, short‑duration wet seasons. Despite generally high year‑round rainfall, occasional short dry spells are linked to regional climate variability; for example, February 2010 recorded only about 23 mm of precipitation, representing one of the driest months.

Figure 4. Monthly precipitation (P) for Kosrae (1998 – 2025) from IMERG.

Alt. text. Line graph showing the monthly precipitation for years 1998 - 2025 in mm.

Annual precipitation totals vary substantially throughout the period of record, with the highest annual value occurring in 2021 at approximately 5,683 mm, followed by several other notably wet years including 2020 (5,061 mm), 2022 (5,341 mm), 2023 (4,768 mm), and 2024 (4,442 mm), whereas the lowest recent annual total was recorded in 2016 at 3,259 mm (Figure 5). The island has experienced several very wet years recently (2020–2022) relative to the 1998–2024 mean. The last five years (2020–2024) average ≈5,059 mm/yr, which is ~24–25% wetter than the long‑term mean of 4,063 mm/yr.

Figure 5. Annual precipitation (P) for Kosrae (1998 – 2024) from IMERG.

Alt. text. Bar chart showing Annual precipitation (P) for Kosrae (1998 – 2024) from IMERG.

From a water management standpoint, this pattern supports reliable surface water availability, but planning should still account for episodic dry months and storage or turbidity challenges during peak rain events.

Evapotranspiration

Because precipitation provides the dominant water input to Kosrae’s hydrologic system, understanding how much of this water is subsequently lost to the atmosphere is essential for evaluating freshwater availability. Evapotranspiration (ET) is the combined water loss to the atmosphere from evaporation (soil, water, and surfaces) and transpiration (plants) (DeJonge and others, 2025). It reflects the interaction of available energy, humidity, wind, and vegetation conditions. Actual ET (ETa), as estimated by the SSEBop model (Senay and others, 2020), represents the portion of potential ET that occurs given surface and atmospheric conditions.

In Kosrae the ET exhibits modest variability compared to precipitation (Figure 6). The mean monthly precipitation pattern is strongly seasonal, with enhanced rainfall during April through July and again in November and December, reaching its highest amount in April with 423 mm. In contrast, the mean monthly actual ET remains rather stable throughout the year, reaching its lowest in January (90 mm) and its highest in October (108 mm), a difference of less than 20 mm across the year. The relatively narrow range of monthly ET values reflects Kosrae’s energy limited environment, where persistent cloud cover and high humidity constrain evaporative demand despite abundant rainfall. Thus, while precipitation varies widely from month to month, actual ET remains comparatively stable.

Figure 6. Mean monthly precipitation (P) and actual ET (ETa) in mm for Kosrae (2013-2025).

Alt. text. Area chart showing the mean monthly precipitation in mm with blue hatch lines and actual ET in mm in green (solid).

Annual ETa totals are similarly stable. From 2013 through 2025, ETa varies only between approximately 1,137 and 1,245 mm per year, indicating that year-to-year changes in atmospheric water loss are small compared to the much larger variability in precipitation (Figure 7).

Figure 7. Annual actual ET (ETa) for Kosrae in mm for years 2013 – 2025 using SSEBop model.

Alt. text. Bar chart with Annual actual ET (ETa) for Kosrae in mm for years 2013 – 2025 using SSEBop model.

This stability, combined with consistently high rainfall, results in a large and persistent surplus of water available for runoff, groundwater recharge, and storage. Because ET responds primarily to energy limitations rather than moisture limitations on Kosrae, these annual values remain relatively constant even in drier years. As a result, interannual differences in water availability are driven almost entirely by precipitation variability rather than by changes in atmospheric demand. This predictable ET regime enhances the island’s hydrologic reliability, supporting perennial streamflow and maintaining saturated soils; however, Kosrae could still be sensitive to large reductions in rainfall during regional drought events.

Water Balance

The water balance or net water availability refers to the water surplus available after accounting for atmospheric losses. Monthly net water is consistently positive across Kosrae, reflecting a humid tropical climate where precipitation frequently exceeds ET. ET typically ranges near ~95–105 mm per month, whereas monthly precipitation commonly falls between ~300–600 mm, yielding surpluses of several hundred millimeters in many months (Figure 8). Extremely wet months with large surpluses are April 2019 (657 mm) and May 2021 (758 mm). Lower surplus months are March/April 2016 and November/December 2018.

Figure 8. Monthly relative net water availability (P – ETa) for Kosrae (2012 – 2025) in mm.

Alt. text. Bar chart showing monthly net water availability for years 2012 - 2025 in mm.

At the annual scale the net water availability ranges from 2060 mm in 2016 to 4544 mm in 2021 (Figure 9). Across the last five years (2020–2024), the mean annual net water is approximately 3,890 mm/yr underscoring a large and persistent water surplus available for streamflow, storage, and groundwater recharge even as year‑to‑year precipitation fluctuates.

Figure 9. Net annual water availability in mm for Kosrae from 2013-2024. Precipitation (P) data is based on IMERG ( ) and evapotranspiration (ET) data is based on the SSEBop (operational Simplified Surface Energy Balance) model. The difference between P and ETa  is the available water in mm.

Alt. text. Horizontal bar chart showing net water availability per year in mm for 2013 to 2024 starting with 2024 at the top.

The combination of high monthly surpluses and strong annual net water implies that water availability is robust for Kosrae under typical conditions. However, the observed variability (for example, 2016 as a lower surplus year) highlights the value of monitoring and planning for short dry spells and managing turbidity or flood related impacts during exceptionally wet months.

Drought Monitoring

Rainfall Index

The Kosrae Rainfall Index record (Figure 10) shows Drier classifications in 61 of 314 months (19 percent) and Dry in 64 months (20 percent) at the 1-month timescale. At the 6-month timescale, Drier (60 months) and Dry (60 months) together account for 39 percent of observations, slightly lower than the other FSM sites, consistent with the greater buffering capacity of Kosrae’s high and uniform rainfall.

Figure 10. Quintile-based Rainfall Index time series for Kosrae showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows.

Notable Drought Episodes

The most prolonged drought at Kosrae occurred from August 2010 through March 2011, when 8 consecutive months were classified as Drier or Dry, with 6 in the Drier category (Table 2). At the 3-month window, 6 of these months were classified as Drier, and 7 at the 6-month window, indicating a sustained multi-timescale deficit of exceptional duration for this wet location. No PEAC-Red months occurred, as even the minimum monthly precipitation during the episode (121.0 millimeters) exceeded the 4-inch (10.2 cm) threshold.

The 2015–2016 regional drought produced 7 consecutive months of Drier/Dry conditions at Kosrae (October 2015 through April 2016), with 5 Drier months at the 1-month timescale and 1 PEAC-Red month (minimum precipitation 66.0 millimeters). Earlier episodes include March through July 2001 (5 months, 3 Drier, 1 PEAC-Red, minimum 72.0 millimeters) and January through June 2007 (6 months, 2 Drier, no PEAC-Red). More recently, October through December 2018 showed 3 consecutive Drier months with Drier classifications at the 3-month window.

Table 2. Summary of notable drought episodes at Kosrae.

| Episode | Duration (months) | Drier Months (1-mo) | PEAC-Red Months | Min Precip. (mm) |
| --- | --- | --- | --- | --- |
| Aug 2010 – Mar 2011 | 8 | 6 | 0 | 121.0 |
| Oct 2015 – Apr 2016 | 7 | 5 | 1 | 66.0 |
| Jan 2007 – Jun 2007 | 6 | 2 | 0 | 143.0 |
| Mar 2001 – Jul 2001 | 5 | 3 | 1 | 72.0 |
| Jun 2019 – Oct 2019 | 5 | 3 | 0 | 208.0 |
| Oct 2018 – Dec 2018 | 3 | 3 | 0 | 192.0 |

Automated 7-Day Precipitation Forecasting

This section of the report documents the 7-day precipitation forecasting system for the four FSM monitoring locations and presents the forecast for March 10–16, 2026 as a demonstration of the operational output format and content. Detailed descriptions of the data sources, processing pipeline, and forecasting methodology are provided in Volume 1 of this report series. The present volume focuses on the system design, output products, and an illustrative forecast example.

Forecasting System Overview

The forecasting methodology is described in detail in Volume 1 of this report series. In brief, the system retrieves daily precipitation forecasts from the NOAA Global Forecast System (GFS) at 0.25° spatial resolution for the 7-day forecast horizon. Total accumulated precipitation fields are downloaded from the NOAA NOMADS server in GRIB2 format, and daily totals are computed by summing the four 6-hourly accumulations within each 24-hour period. For each monitoring location, zonal statistics are extracted using a polygon shapefile, producing a 7-day daily precipitation time series. The processing pipeline is implemented in R using the terra, sf, tidyverse, ggplot2, and magick packages.

The system generates individual and combined bar charts, animated GIF maps of forecast spatial distribution, and summary CSV tables for each forecast cycle, all packaged into a ZIP archive for distribution. Alert thresholds of 50 mm per day and 200 mm per 7-day total are applied to flag potential high-precipitation events (Table 3). Forecast products are compiled into HTML report format and distributed to designated recipients via automated email, enabling water managers to receive updated forecasts without manual intervention.

Table 3. Alert thresholds applied to GFS precipitation forecasts.

| Threshold | Value | Significance |
| --- | --- | --- |
| Daily precipitation | ≥ 50 mm | Potential flood risk; heavy rainfall day |
| 7-day total | ≥ 200 mm | Sustained heavy precipitation over forecast period |

Demonstration Forecast: March 10–16, 2026

The following section presents the GFS precipitation forecast for March 10–16, 2026 (GFS model run 20260310, 12z cycle) as a demonstration of the operational output. This forecast period falls within the FSM dry season, when Yap in particular is climatologically vulnerable to low rainfall. The forecast results are presented as a combined overview followed by site-specific analysis.

Forecast Overview

Table 4 presents the daily forecast precipitation for all four FSM locations for March 10–16, 2026. Seven-day forecast totals ranged from 43.2 mm at Pohnpei to 64.5 mm at Kosrae. No daily values exceeded the 50-mm alert threshold, and no 7-day totals approached the 200-mm threshold. The highest single-day forecast was 26.9 millimeters at Kosrae on Wednesday, March 11. All four sites show a broadly similar pattern of heavier rainfall at the beginning of the period tapering to lighter conditions by mid-week, with a secondary pulse at some sites toward the weekend.

Table 4. GFS 7-day precipitation forecast (mm) for FSM study locations, March 10–16, 2026. GFS model run 20260310, 12z cycle. No daily or weekly alert thresholds were exceeded.

| Location | Tue 10 Mar | Wed 11 Mar | Thu 12 Mar | Fri 13 Mar | Sat 14 Mar | Sun 15 Mar | Mon 16 Mar | 7-Day Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yap | 17.7 | 16.9 | 6.7 | 9.5 | 4.0 | 8.5 | 0.3 | 63.6 |
| Weno | 18.5 | 2.3 | 0.0 | 2.9 | 0.9 | 5.8 | 16.7 | 47.1 |
| Pohnpei | 23.2 | 5.8 | 3.9 | 3.6 | 0.3 | 6.4 | 0.0 | 43.2 |
| Kosrae | 10.6 | 26.9 | 2.8 | 8.0 | 5.4 | 10.2 | 0.6 | 64.5 |

Figure 11 presents the combined bar chart for the four FSM locations, showing the day-by-day evolution of forecast precipitation and facilitating cross-site comparison. The highest daily totals are concentrated on the first two days (March 10–11) across most sites, with a secondary pulse at Weno on the final day.

Figure 11. Combined 7-day GFS precipitation forecast bar chart for the four FSM monitoring locations, March 10–16, 2026. The dashed red line indicates the 50-mm daily alert threshold.

Yap

The Yap forecast for March 10–16 shows a 7-day total of 63.6 mm (Figure 12), the second-highest among the FSM sites. Rainfall is concentrated in the first two days (17.7 mm on March 10 and 16.9 mm on March 11), tapering to moderate levels mid-week and near-zero on the final day (0.3 mm on March 16). No daily values exceed the 50-mm threshold. In the context of Yap’s dry-season climatology (February median of 96 mm), a weekly total of 63.6 mm would represent a meaningful but not exceptional contribution to monthly accumulation. Given that the drought monitoring record shows Yap is currently in an extended period of below-normal precipitation (the Rainfall Index was classified as Drier or Dry for recent months), this forecast of moderate rainfall would be insufficient on its own to reverse the ongoing deficit but may provide temporary relief to catchment systems.

Figure 12. GFS 7-day precipitation forecast bar chart for Yap, March 10–16, 2026. Bars show daily accumulated precipitation in millimeters.

Weno (Chuuk)

The Weno forecast shows a 7-day total of 47.1 mm (Figure 13), characterized by an asymmetric distribution with 18.5 mm on March 10 followed by several days of very low precipitation (0.0 to 2.9 mm, March 11–14), then a secondary pulse of 16.7 mm on the final day (March 16). This pattern of episodic rainfall interspersed with dry days is typical of the trade-wind convective regime that governs Weno’s precipitation. The 7-day total of 47.1 mm is the second-lowest among the FSM sites and represents a relatively dry forecast for a location with a baseline March median of approximately 150 mm.

Figure 13. GFS 7-day precipitation forecast bar chart for Weno (Chuuk), March 10–16, 2026.

Pohnpei

The Pohnpei forecast shows the lowest 7-day total among the FSM sites at 43.2 millimeters (Figure 14). Nearly all of the expected precipitation is concentrated on the first day (23.2 millimeters on March 10), with the remaining 6 days producing only 0.0 to 6.4 millimeters each. For Pohnpei, which normally receives year-round heavy rainfall, a 7-day total this low is noteworthy. However, it should be interpreted with the caveat that GFS forecasts at this resolution may underestimate convective precipitation in high-rainfall orographic environments such as Pohnpei’s volcanic terrain. The forecast does not trigger either the daily or weekly alert threshold.

Figure 14. GFS 7-day precipitation forecast bar chart for Pohnpei, March 10–16, 2026.

Kosrae

The Kosrae forecast shows the highest 7-day total among the FSM sites at 64.5 mm (Figure 15). The distribution is bimodal, with a moderate first day (10.6 mm on March 10) followed by the peak of the period on March 11 (26.9 mm), a mid-week lull, and a secondary moderate-rainfall day on March 15 (10.2 mm). The 26.9-mm forecast for March 11 represents the highest single-day value in this forecast cycle across the FSM sites. While no alert thresholds are exceeded, the Kosrae forecast suggests more active convective conditions than at the other three sites during this period.

Figure 15. GFS 7-day precipitation forecast bar chart for Kosrae, March 10–16, 2026.

Forecast Utility and Interpretation

The March 10–16, 2026 forecast demonstrates the type of near-term guidance that the GFS-based system provides to water managers across the FSM. During this forecast period, all four sites received moderate to low predicted rainfall with no alert-level events, a finding that is operationally relevant for managers at drought-affected locations such as Yap, where extended below-normal conditions have been documented in the companion drought monitoring volume. A forecast of 63.6 millimeters over 7 days at Yap, while not negligible, signals to water managers that the coming week is unlikely to produce rainfall sufficient to substantially replenish depleted catchment reserves, reinforcing the need for continued conservation measures.

The forecast is most useful when interpreted alongside the drought monitoring results. A 7-day forecast showing no alert-level precipitation at a site currently experiencing Drier conditions at the 3-month and 6-month Rainfall Index windows confirms that the near-term outlook does not offer relief from the ongoing deficit. Conversely, a forecast showing daily totals exceeding 50 millimeters at such a site would signal a potential recovery event worthy of operational attention. This complementary relationship between backward-looking monitoring and forward-looking forecasting is the central design principle of the integrated system.

Operational Workflow

The forecasting system is designed for routine operational deployment on a schedule aligned with the GFS model update cycle. Each forecast run retrieves the most recent GFS data, processes it for all monitoring locations, generates the standard suite of output products (bar charts, animation maps, summary tables, and README), packages the outputs into a distributable archive, and transmits the package to designated recipients via automated email. The entire processing chain completes in approximately 30 minutes, enabling same-day delivery of updated forecasts. The system has been implemented to support weekly or more frequent forecast distribution, depending on stakeholder needs and the current drought status of the monitored locations.

This section documented the 7-day GFS precipitation forecasting system developed for four study locations in the Federated States of Micronesia and presented the March 10–16, 2026 forecast as a demonstration of operational output. The principal findings are as follows.

The forecasting system retrieves NOAA GFS forecast data at 0.25° resolution, extracts daily accumulated precipitation for each monitoring location, applies operational alert thresholds, and generates a standardized suite of output products including bar charts, spatial animations, summary tables, and a packaged distribution archive. The processing pipeline completes in approximately 30 minutes and is designed for automated, routine deployment.

The March 10–16, 2026 demonstration forecast showed 7-day totals ranging from 43.2 mm at Pohnpei to 64.5 mm at Kosrae, with no daily or weekly alert thresholds exceeded. This moderate-to-low forecast is operationally relevant for drought-affected locations, as it signals to water managers that the near-term outlook does not include substantial rainfall relief.

The forecasting system fills a critical gap in water management across the FSM, where sparse ground-based weather station networks and limited access to numerical weather prediction products have historically left many communities without reliable short-term rainfall outlooks. When used in conjunction with the IMERG-based drought monitoring system, the combined backward-looking and forward-looking capabilities provide a comprehensive precipitation intelligence framework for Pacific Island water supply management.

Seismicity

Kosrae is a deeply eroded basaltic volcanic edifice (shield‑type) with rugged relief and narrow coastal lowlands (Rehman and others, 2013, USGS, 2015). It lies well inside the Pacific Plate, far from the nearest active circum‑Pacific subduction margins (Yap–Palau–Mariana trench–arc systems), which are thousands of kilometers to the west and northwest; for example, straight‑line distance from Kosrae to Yap is on the order of ~2,780 km. This intraplate setting contributes to a low local seismic hazard on Kosrae, and regional catalogs show sparse instrumental activity in its vicinity compared with locations near the trenches. When felt shaking does occur, it is typically from distant regional events that have propagated across oceanic paths, or from teleseisms that may prompt marine advisories (e.g., the July 30, 2025 tsunami warning for all FSM states following a distant M 8.7 event). Overall, perceptible shaking on Kosrae is infrequent, and damaging ground motions are uncommon; nevertheless, routine preparedness remains prudent given the island’s broader location within the Ring of Fire and its reliance on coastal infrastructure.

Island of Pohnpei

Pohnpei, located near 6.9°N and 158.2°E, is the largest and wettest island in the Federated States of Micronesia, with a rugged volcanic interior rising above 750 meters (Mink and others, 1992) (Figure 16).

Figure 16. Map of Pohnpei. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A simplified topographic map showing the island of Pohnpei. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

Its steep watersheds, cloud forests, and extensive river networks generate abundant surface water, making Pohnpei one of the region’s most hydrologically productive islands. Despite high rainfall, water availability varies seasonally, and communities rely heavily on surface‑water intakes that can be affected by turbidity during storms. The island’s population is concentrated along the coastal plain surrounding Kolonia and neighboring municipalities, where groundwater is used to supplement surface sources. Tectonically, Pohnpei rests on the Caroline Plate and experiences low to moderate seismicity from interactions with the nearby Pacific Plate. These tectonic forces, combined with intense rainfall, contribute to slope instability, erosion, and sedimentation that influence water quality and watershed management.

Precipitation

Pohnpei has a humid, high precipitation tropical climate dominated by orographic rainfall generated by its rugged volcanic interior with wet conditions throughout the year but still susceptible to short drought periods. The mean annual total precipitation is 3,104 mm/yr, the mean monthly precipitation is 259 mm, and the mean of each year’s minimum and maximum monthly precipitation are 96 mm and 420 mm, respectively (Table 5).

Table 5. Mean IMERG Precipitation Statistics (1998–2024) for Pohnpei

| Statistic | Precipitation (mm) |
| --- | --- |
| Mean annual | 3104 |
| Mean monthly mean | 259 |
| Mean monthly minimum | 96 |
| Mean monthly maximum | 420 |

Pohnpei receives substantial precipitation throughout the year, with monthly totals commonly ranging from about 200 to 450 mm, reflecting the island’s persistently wet tropical climate (Figure 17). Several months exceed 500 mm, including May 2015 (595 mm), July 2018 (565 mm), November 2020 (587 mm), and October 2022 (524 mm), and a rare extreme of >700 mm occurred in July 1998 (757 mm). Conversely, occasional short dry spells linked to regional climate variability can occur; for example, March 1998 (1.25 mm) and April 1999 (2.68 mm) rank among the driest months.

Figure 17. Monthly precipitation for years 1998 - 2025 in mm.

Alt. text. Line graph showing the monthly precipitation for years 1998 - 2025 in mm.

Annual precipitation totals varied substantially across the period of record (Figure 18). The highest annual value occurred in 2022 (4,405 mm), with several other notably wet years including 2018 (4,361 mm), 2019 (3,754 mm), 2021 (3,848 mm), and 2023 (3,865 mm), whereas the lowest annual total was recorded in 1998. The last five years (2020–2024) average 3,892 mm/yr, which is ~25% wetter than the long term- mean of 3,104 mm/yr, underscoring recent multi-year wet conditions.

Figure 18. Annual precipitation for years 1998 - 2024 in mm.

Alt. text. Bar chart showing annual precipitation for years 1998 to 2024 in mm.

From a water management standpoint, this pattern supports reliable surface water availability, but planning should still account for episodic dry months and storage/turbidity challenges during peak rain events.

Evapotranspiration

Like Kosrae, Pohnpei exhibits an energy‑limited hydrologic regime, where abundant cloud cover, high humidity, and dense vegetation constrain the amount of energy available for atmospheric water loss. As a result, monthly actual ET remains relatively stable throughout the year, despite large seasonal swings in precipitation. ET values fall within a narrow band of 90–115 mm per month (Figure 19), varying far less than rainfall, which varies from 176 mm in February to 351 mm in July. The modest variability in ET highlights that available energy, rather than soil moisture, dictates ET, even during wetter months when soils are consistently saturated.

Figure 19. Mean monthly precipitation (P) and actual ET (ETa) in mm for Pohnpei (2013-2025).

Alt. text. Area chart showing the mean monthly precipitation (P) and actual ET (ETa) in mm for Pohnpei (2013-2025) with blue hatch lines and actual ET in mm in green (solid).

Annual ET totals show a similar degree of stability (Figure 20). From 2013 through 2025, actual ET varies only between approximately 1,140 and 1,230 mm per year, indicating that year‑to‑year changes in atmospheric water loss are small compared to the much larger variability in precipitation. This consistency reflects the climatic controls on ET.

Figure 20. Annual actual ET for years 2013 - 2025 in mm.

Alt. text. Bar chart with annual actual ET for years 2013 - 2025 in mm.

Because ET changes so little from month to month or year to year, interannual differences in Pohnpei’s water availability are driven almost entirely by precipitation variability rather than changes in evaporative demand. Even during years with reduced rainfall, ET remains relatively constant, meaning that declines in precipitation directly translate into reduced net water inputs. Conversely, during exceptionally wet years, nearly all additional rainfall becomes available for runoff, groundwater recharge, or storage because ET does not increase proportionally.

This stable ET regime contributes to Pohnpei’s abundant freshwater resources, including perennial streams, extensive wetlands, and high baseflow while also emphasizing the island’s sensitivity to extended dry periods. During such events, even modest reductions in precipitation can reduce water inputs substantially because ET demand continues at near‑normal levels. Accordingly, understanding the predictable behavior of ET is essential for effective water‑resource planning across the island.

Water Balance

The water balance on Pohnpei reflects the interaction between exceptionally high precipitation, relatively stable evapotranspiration, and the island’s substantial capacity for surface‑water generation through its steep volcanic watersheds and dense vegetation cover. Because actual ET varies only modestly throughout the year, net water availability is driven almost entirely by fluctuations in rainfall. As a result, Pohnpei consistently exhibits large monthly and annual water surpluses, punctuated by occasional but notable declines during regional drought events.

Monthly net water availability ranges from -121 mm to more than 500 mm in March 2018, with several months exceeding 400 mm producing therefore large surpluses (Figure 21). In contrast, the driest months, including February 2016 and March 2020 generate negative net water availability. These dry extremes predominantly occur during strong ENSO events, which temporarily suppress rainfall across the region.

Figure 21. Monthly net water availability for years 2012 - 2025 in mm.

Alt. text. Bar chart showing monthly net water availability for years 2012 - 2025 in mm.

During most months of the year, however, Pohnpei experiences substantial positive water balances. Precipitation routinely exceeds actual ET by several hundred millimeters, potentially contributing directly to increased soil moisture, perennial streamflow, sustained baseflow, and widespread groundwater recharge. Nearly all excess rainfall becomes available for hydrologic replenishment, supporting the island’s extensive freshwater resources. During dry periods, such as the ENSO‑driven drought of 2016, rainfall declines sharply while ET continues at near‑normal levels, leading to short‑term reductions in net water availability. Although Pohnpei’s overall hydrologic system is resilient, these events can still reduce streamflow, lower reservoir or intake yields, and increase turbidity susceptibility when rainfall abruptly resumes. However, unlike the low‑lying atolls of Yap State or the Chuuk Lagoon, Pohnpei’s larger watersheds and deeper volcanic aquifers could provide buffering capacity that helps moderate the impacts of multi‑week dry spells.

Interannual variability reflects this same pattern (Figure 22). Wet years include 2018 (4,361 mm), 2022 (4,405 mm), and 2023 (3,865 mm) and produced very large annual surpluses. Drier years such as 2013 (1434 mm), though still wetter than many islands in the region, demonstrate the degree to which precipitation anomalies directly influence annual water availability. Because ET remains relatively stable from year to year, differences in annual net water are driven almost entirely by precipitation variability, emphasizing the importance of understanding rainfall patterns and ENSO dynamics for long‑term water‑resource planning.

Figure 22. Net water availability per year in mm.

Alt. text. Horizontal bar chart showing net water availability per year in mm for 2013 to 2024 starting with 2024 at the top.

Overall, Pohnpei’s water balance highlights the island’s strong hydrologic resilience, substantial natural water storage, and consistently positive annual surplus. At the same time, dependence on surface‑water intakes makes the island sensitive to turbidity during intense rainfall and to short‑lived deficits during drought conditions. Understanding these dynamics is essential for guiding watershed protection, storage strategies, and infrastructure planning across the island.

Drought Monitoring

Rainfall Index

The Pohnpei Rainfall Index time series (Figure 23) shows Drier classifications in 63 of 314 months (20 percent) and Dry in 64 months (20 percent) at the 1-month timescale. The PEAC index classified only 10 months as Red and 60 as Yellow. These are substantially fewer than Yap or Weno, reflecting the high absolute rainfall at this site. At the 6-month timescale, Drier (65 months) and Dry (64 months) together comprise 41 percent of observations, indicating that cumulative deficits do occur despite the high baseline rainfall.

Figure 23. Quintile-based Rainfall Index time series for Pohnpei showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows.

Notable Drought Episodes

The most significant drought at Pohnpei occurred from February through May 2016, when four consecutive months were classified as Drier or Dry, with 3 in the Drier category. The 3-month and 6-month windows both showed Drier classifications for all 4 months. The PEAC index classified 2 months as Red during this period, with a minimum monthly precipitation of 44.0 mm, which is a very low value for this normally wet location. This event is part of the same 2015–2016 regional drought observed across the FSM.

Additional episodes include February through May 2006 (4 months, 2 Drier at 1-month, Drier persisting through 3-month and 6-month windows); May through August 2013 (4 months, 2 Drier, 4 Drier at 3-month, 4 Drier at 6-month); and January through March 2020 (3 months, all 3 Drier, 2 PEAC-Red, minimum 87.2 mm). Compared to Yap and Weno, Pohnpei’s drought episodes are generally shorter in duration and are accompanied by fewer PEAC-Red months, reflecting the higher absolute rainfall baseline.

Table 6. Summary of notable drought episodes at Pohnpei.

| Episode | Duration (months) | Drier Months (1-mo) | PEAC-Red Months | Min Precip. (mm) |
| --- | --- | --- | --- | --- |
| Feb 2016 – May 2016 | 4 | 3 | 2 | 44.0 |
| Feb 2006 – May 2006 | 4 | 2 | 0 | 141.2 |
| May 2013 – Aug 2013 | 4 | 2 | 0 | 140.8 |
| Jan 2020 – Mar 2020 | 3 | 3 | 2 | 87.2 |
| Jun 2007 – Sep 2007 | 4 | 3 | 0 | 190.8 |
| Oct 2015 – Dec 2015 | 3 | 3 | 0 | 155.2 |

Seismicity

Pohnpei Island, the eroded remnant of a basaltic shield volcano within the Caroline Islands, lies on interior oceanic lithosphere well away from the nearest active plate boundaries of the circum‑Pacific system (Spengler and others, 1991; Spengler and others, 2012; Zong and others, 2020). The closest subduction margins are to the northwest along the Yap–Palau–Mariana trench–arc system, far offshore from Pohnpei’s location, which contributes to a generally low local seismic hazard on the island. Recent regional assessments and historical catalogs indicate few instrumental earthquakes within several hundred kilometers of Pohnpei; for example, the strongest event reported in the past decade was about M 4.6 on March 24, 2025, located roughly 160 km east of Palikir at shallow depth, and earlier records note only sparse moderate events since 1900. Over such distances, attenuation of seismic waves across oceanic paths typically limits felt intensity on Pohnpei, and there are no mapped active faults or documented sequences of damaging, locally generated earthquakes on the island. A 2024 engineering hazard review similarly characterizes Pohnpei’s seismic exposure as low and provides probabilistic and deterministic ground‑motion estimates for planning purposes, noting no island‑specific liquefaction zones or tsunami sources identified by prior investigators. Accordingly, perceptible shaking on Pohnpei is infrequent, and damaging ground motions are uncommon; routine awareness and preparedness remain prudent given the island’s regional proximity to active subduction systems and its broader western Pacific context within the Ring of Fire.

Island of Yap

Yap State consists of four high continental‑type islands located near 9.3°N and 138.1°E, along with numerous low‑lying atolls (Figure 24).

Figure 24. Map of Yap. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the island Yap. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

The high islands feature rolling hills, thin soils, and short watersheds that support small streams during wet periods but provide limited perennial flow. Freshwater availability on Yap relies heavily on shallow groundwater lenses and rainwater harvesting, making the island particularly vulnerable to ENSO‑driven droughts (Jenson and others, 2008, Samson and Ruegorong, 2021). Most residents live in small coastal villages where groundwater salinization and limited storage capacity pose chronic challenges. Tectonically, Yap lies near the boundary of the Pacific and Philippine Sea Plates, adjacent to the Yap Trench, a major subduction zone. Although strong earthquakes are uncommon on the main islands, regional tectonic activity shapes long‑term coastal geomorphology and contributes to hazards relevant to water infrastructure and groundwater sustainability.

Precipitation

Yap has a tropical monsoon influenced climate with marked wet and dry seasons and substantially lower rainfall than the high volcanic islands of Kosrae and Pohnpei (Aranug, 2016). Unlike the steep, cloud forest environments of those islands, Yap’s topography is more subdued, producing weaker orographic enhancement of rainfall. As a result, although Yap receives abundant precipitation during the wet season, it is more vulnerable to extended dry periods, especially during El Niño events. Longterm IMERG summaries indicate that Yap’s mean annual precipitation is 2,809 mm/yr, the mean monthly precipitation is 234 mm, and the mean of the minimum and maximum monthly precipitation per year is 56 mm and 575 mm, respectively (Table 7).

Table 7. Mean IMERG precipitation statistics (1998–2024) for Yap

| Statistic | Precipitation (mm) |
| --- | --- |
| Mean annual | 2809 |
| Mean monthly mean | 234 |
| Mean monthly minimum | 56 |
| Mean monthly maximum | 575 |

Yap’s monthly precipitation varies substantially, with wet season totals commonly exceeding 300–400 mm and dry season months frequently falling below 100 mm (Figure 25). The years from 1998 to 2024 show several notably wet months exceeding 500 mm, including August 2002 (818 mm), September 2002 (586 mm), September 2007 (582 mm), October 2022 (897 mm), August 2023 (676 mm), and July 2023 (644 mm). In contrast, Yap experiences some very dry months, often below 20 mm, such as March 1998 (0.38 mm), February 2019 (13.5 mm), and February 2024 (30 mm), reflecting Yap’s sensitivity to ENSO driven drought conditions.

Figure 25. Monthly precipitation for years 1998 - 2025 in mm.

Alt. text. Line graph showing the monthly precipitation for years 1998 - 2025 in mm.

Annual precipitation totals also vary widely (Figure 26). The wettest year in the IMERG record is 2022 (4405 mm), closely followed by 2018 (4361 mm), 2015 (3730 mm), and 2011 (3665 mm). The driest year was 1998 (2142 mm). The most recent five-year period (2020–2024) averages 3,892 mm/yr, which is ~39% wetter than the long-term mean of 2,809 mm/yr, reflecting a relatively wet recent period compared to earlier parts of the record.

Figure 26. Annual precipitation for years 1998 - 2024 in mm.

Alt. text. Bar chart showing annual precipitation for years 1998 to 2024 in mm.

From a water-management perspective, Yap’s combination of a strong dry season, shallow groundwater systems, and pronounced ENSO sensitivity highlights the importance of drought monitoring, rainwater harvesting reliability, and strategic storage planning to mitigate periodic freshwater shortages.

Evapotranspiration

Yap’s ET regime reflects the lower elevation on the island, more canopy structure and stronger exposure to seasonal shifts in solar radiation and humidity (Hatfield and Prueger, 2010).

As a result, Yap experiences little ET variability throughout the year (Figure 27).  ET values fall within a narrow band of 90-107 mm per month, varying far less than rainfall, which varies from 109 mm in February to 382 mm in September. During wetter months, high humidity, frequent cloud cover, and saturated soils limit evaporative demand, keeping actual ET relatively subdued. During the dry season, however, clearer skies and reduced rainfall increase atmospheric demand, allowing ET to rise even as soil moisture becomes more limiting.

Figure 27. Mean monthly precipitation (P) and actual ET (ETa) in mm for Yap (2013-2025).

Alt. text. Area chart showing the mean monthly precipitation (P) and actual ET (ETa) in mm for Yap (2013-2025) with blue hatch lines and actual ET in mm in green (solid).

Annual ET totals on Yap remain comparatively stable from year to year, reflecting the influence of large‑scale atmospheric controls rather than local moisture availability alone (Figure 28).

Figure 28. Annual actual ET for years 2013 - 2025 in mm.

Alt. text. Bar chart with annual actual ET for years 2013 - 2025 in mm.

The consistency of ET means that year‑to‑year differences in Yap’s hydrologic balance are driven primarily by precipitation fluctuations, not by changes in evaporative losses. In wet years, additional rainfall contributes directly to groundwater lens recharge and increased surface‑water availability because ET does not increase proportionally. In dry years, however, ET continues at near‑normal levels, drawing down soil moisture and accelerating the depletion of thin freshwater lenses.

This behavior underscores Yap’s heightened sensitivity to rainfall deficits. Because ET remains relatively steady even during drought, decreases in precipitation translate more directly into reduced groundwater storage and increased salinity risk. As a result, understanding both the stability of ET and the variability of precipitation is critical for managing Yap’s water resources, planning storage capacity, and anticipating the onset and severity of ENSO‑driven drought conditions.

Water Balance

The water balance on Yap reflects the interaction between a strong seasonal precipitation regime, comparatively stable evapotranspiration, and the island’s limited capacity to store freshwater in its thin groundwater lenses and small watershed systems (Shade and others, 1992). Because actual ET varies far less than rainfall, net water availability (precipitation minus actual ET) is controlled primarily by fluctuations in precipitation rather than by changes in atmospheric demand. As a result, Yap’s hydrologic system alternates between periods of substantial water surplus during the wet season and pronounced deficits during the dry season (Figure 29). Extremely wet months with large surpluses occur when monsoon flows or tropical disturbances bring intense rainfall, such as in October 2022 (800 mm) or September 2012 (702 mm) producing several hundred millimeters of net water in a single month. Conversely, very low‑surplus or deficit months commonly occur during January–April in El Niño years, including February–March 2019, and January–February 2024, when rainfall can briefly fall below 20–30 mm for the month. These extremes illustrate Yap’s strong sensitivity to large‑scale climate oscillations.

Figure 29. Monthly net water availability for years 2012 - 2025 in mm.

Alt. text. Bar chart showing monthly net water availability for years 2012 - 2025 in mm.

During the wet season, frequent rainfall and high humidity create consistently positive monthly water balances. In these months, precipitation routinely exceeds ET by several hundred millimeters, contributing to increased soil moisture, intermittent streamflow, and recharge of the island’s fragile freshwater lenses. Because ET does not rise proportionally during wetter periods, nearly all excess rainfall becomes available for hydrologic recharge, making these seasonal pulses essential for sustaining groundwater reserves and household storage systems during later dry months. In contrast, the dry season and ENSO‑related drought periods result in sharp reductions in net water availability. Even as rainfall declines dramatically, ET continues at near‑normal levels, steadily depleting soil moisture and thinning groundwater lenses. This sustained atmospheric demand, combined with limited natural storage capacity, leads to rapid reductions in freshwater availability, increasing the risk of salinity intrusion and placing added pressure on rainwater‑harvesting systems. Yap’s small catchments, shallow lenses, and dependence on household storage amplify the severity of these seasonal shortfalls. Interannual variability further shapes Yap’s water balance (Figure 30). Wet years, such as 2014, 2017, 2022, with strong precipitation anomalies produce large annual surpluses that help maintain groundwater quality and improve water security across communities. Dry years, such as 2015, and 2019, can result in extended periods of minimal or negative net water availability, creating substantial stress on both natural water systems and community infrastructure.

Figure 30. Net water availability per year in mm.

Alt. text. Horizontal bar chart showing net water availability per year in mm for 2013 to 2024 starting with 2024 at the top.

Because ET is relatively steady from year to year, these deficits arise almost entirely from precipitation shortages, underscoring Yap’s dependence on reliable rainfall for freshwater security.

Overall, Yap’s water balance demonstrates that the island’s hydrologic resilience hinges on effective rainwater capture, careful groundwater stewardship, and proactive drought‑preparedness planning. The stability of ET simplifies long‑term water‑balance interpretation, but it also means that reductions in precipitation translate directly into reduced freshwater availability. Understanding these dynamics is essential for supporting sustainable water supply, anticipating drought impacts, and guiding investments in storage and climate‑resilient infrastructure.

Drought Monitoring

Rainfall Index

The 1-month Rainfall Index time series for Yap (Figure 31A) shows substantial month-to-month variability, with frequent Drier classifications (63 of 314 months, 20 percent) and Dry classifications (64 months, 20 percent), yielding a combined below-Typical frequency of 40 percent. Wetter classifications occurred in 61 months (19 percent), indicating that above-normal precipitation months are nearly as common as below-normal months, which is consistent with the expected quintile distribution of approximately 20 percent per category. The departures from exact 20-percent splits arise because the baseline period (2000–2024, 25 years) produces only 25 observations per calendar month for computing quintile thresholds.

The 3-month and 6-month windows (Figure 31 B, C) provide smoothed perspectives on cumulative precipitation patterns. At the 6-month timescale, Drier classifications (65 months) and Dry classifications (66 months) together account for 42 percent of all observations, indicating that sustained multi-month deficits are a recurring feature at Yap. The PEAC 4/8-inch index classified 58 months (18 percent) as Red (below 4 inches), the highest proportion among the four FSM sites, and 81 months (26 percent) as Yellow.

Figure 31. Quintile-based Rainfall Index time series for Yap showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows. Bars represent observed precipitation (or rolling sums) color-coded by quintile category. The dashed line shows the baseline median for each calendar month.

Notable Drought Episodes

The most severe sustained drought at Yap in the 25-year record occurred from October 2015 through September 2016. During this period, the 1-month Rainfall Index classified 7 consecutive months (October 2015 through April 2016) as Drier or Dry, with 6 of those months in the Drier category. All 7 of those months also received a Drier classification at the 3-month window, and 6 months at the 6-month window. The PEAC index classified 6 months as Red during this period, with a minimum monthly precipitation of just 4.0 millimeters. A subsequent 4-month Drier/Dry period from June through September 2016 extended the drought impact.

A second prolonged episode from January 2019 through October 2020 included two distinct clusters of consecutive Drier/Dry months: February through May 2019 (4 months, minimum precipitation 11.5 millimeters, 3 PEAC-Red months) and January through April 2020 (4 months, minimum 24.5 millimeters, 4 PEAC-Red months), interspersed by only brief recovery. At the 6-month window, persistent Drier and Dry classifications spanned much of this period, indicating sustained cumulative deficits. Additional notable events include October 2002 through March 2003 (6 consecutive Drier/Dry months, 3 PEAC-Red) and February through June 2024 (5 months, minimum 15.5 millimeters, 3 PEAC-Red).

Table 8. Summary of notable drought episodes at Yap, defined as three or more consecutive months classified as Drier or Dry at the 1-month Rainfall Index.

| Episode | Duration (months) | Drier Months (1-mo) | PEAC-Red Months | Min Precip. (mm) |
| --- | --- | --- | --- | --- |
| Oct 2015 – Apr 2016 | 7 | 6 | 6 | 4.0 |
| Feb 2019 – May 2019 | 4 | 3 | 3 | 11.5 |
| Jan 2020 – Apr 2020 | 4 | 3 | 4 | 24.5 |
| Oct 2002 – Mar 2003 | 6 | 1 | 3 | 28.5 |
| Feb 2024 – Jun 2024 | 5 | 4 | 3 | 15.5 |
| Feb 2013 – May 2013 | 4 | 1 | 3 | 39.5 |

Seismicity

The Yap Islands, the extreme western state of the Federated States of Micronesia, experience earthquakes because they are adjacent to the Yap Trench, where the Pacific plate is being subducted, part of the Ring of Fire. For example, M 5.4 and M 5.9 earthquakes were felt in the Yap Islands in October 2025 (USGS, 2025a; USGS, 2025b).

The Philippine Sea plate is bordered by the larger Pacific and Eurasia plates and the smaller Sunda plate. The Philippine Sea plate is unusual in that its borders are nearly all zones of plate convergence. The Pacific plate is subducted into the mantle, south of Japan, beneath the Izu-Bonin and Mariana island arcs, which extend more than 3,000 km along the eastern margin of the Philippine Sea plate. This subduction zone is characterized by rapid plate convergence and high-level seismicity extending to depths of over 600 km. Despite this extensive zone of plate convergence, the plate interface has been associated with few great (M>8.0) ‘megathrust’ earthquakes. This low seismic energy release is thought to result from weak coupling along the plate interface (Scholz and Campos, 1995). These convergent plate margins are also associated with unusual zones of back-arc extension (along with resulting seismic activity) that decouple the volcanic island arcs from the remainder of the Philippine Sea Plate (Karig et al., 1978; Klaus et al., 1992).

South of the Mariana arc, the Pacific plate is subducted beneath the Yap Islands along the Yap trench. The long zone of Pacific plate subduction at the eastern margin of the Philippine Sea Plate is responsible for the generation of the deep Izu-Bonin, Mariana, and Yap trenches as well as parallel chains of islands and volcanoes, typical of circum-pacific island arcs. Similarly, the northwestern margin of the Philippine Sea plate is subducting beneath the Eurasia plate along a convergent zone, extending from southern Honshu Island of Japan to the northeastern coast of Taiwan, manifested by the Ryukyu Islands and the Nansei-Shoto (Ryukyu) trench.

City of Weno in the Chuuk Lagoon

Weno, the most populated island in the Chuuk Lagoon, lies near 7.5°N and 151.9°E and is part of a large, volcanic‑origin island complex now dominated by low‑lying karst and lagoonal systems (Figure 32).

Figure 32. Map of the Island Weno with city Weno in the Chuuk Lagoon. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Chuuk Lagoon including the islands of Weno, Tonoas, Uman, Fefan, Uatschaluk, and Tol Island. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

The island has limited surface water, with most residents relying on rainwater harvesting and thin, fragile groundwater lenses that are sensitive to contamination, overuse, and saltwater intrusion (U.S. Geological Survey, 2018). Hydrologically, Weno is among the most vulnerable locations in FSM, with droughts causing rapid declines in freshwater availability (Bailey and Jenson, 2011; Werner and others, 2017). The island’s dense population places additional pressure on limited water resources and aging distribution systems. Tectonically, Weno sits on the Caroline Plate but is influenced by regional seismicity associated with surrounding plate boundaries, including the nearby subduction zones to the north and west. Earthquakes can exacerbate coastal erosion and infrastructure damage, indirectly affecting water‑supply reliability.

Precipitation

Weno, located in the Chuuk Lagoon, has a tropical maritime climate with high humidity, abundant rainfall, and relatively weak orographic enhancement compared to the high volcanic islands of Kosrae and Pohnpei (Climate-Data.org, 2026). This results in consistently wet conditions but also a strong dependence on rainwater harvesting and limited shallow groundwater resources. Long-term IMERG summaries indicate that Weno’s mean annual precipitation is 2,817 mm/yr, the mean monthly precipitation is 235 mm, and the mean minimum and maximum monthly precipitation per year are 61 mm and 533 mm, respectively (Table 9).

Table 9. Mean IMERG precipitation statistics (1998–2024) for Weno

| Statistic | Precipitation (mm) |
| --- | --- |
| Mean annual | 2817 |
| Mean monthly mean | 235 |
| Mean monthly minimum | 61 |
| Mean monthly maximum | 533 |

Weno receives substantial rainfall year round, but monthly totals vary widely (Figure 33). Wet season months commonly exceed 300–400 mm, and several months show extreme rainfall above 500 mm, including April 2004 (853 mm), August 2000 (766 mm), November 2009 (542 mm), May 2023 (538 mm), and October 2022 (443 mm). In contrast, the island periodically experiences dry season months falling near or below 20–30 mm, such as March 2020 (48 mm), February 2019 (78 mm), February 2024 (201 mm but preceded by very low January totals), and multiple early record months below 15 mm, underscoring Weno’s vulnerability to short duration droughts.

Figure 33. Monthly precipitation for years 1998 - 2025 in mm.

Alt. text. Line graph showing the monthly precipitation for years 1998 - 2025 in mm.

Annual precipitation totals show substantial interannual variability (Figure 34). The wettest year in the record is 2000 (4,176 mm), followed by 2023 (3842 mm), and 2003 (3729 mm), whereas the driest year was 1998 (1,898 mm), a year strongly influenced by El Niño related drought across the western Pacific. The most recent five year period (2020–2024) averages 3,172 mm/yr, which is ~13% wetter than the long-term mean of 2,817 mm/yr, reflecting a generally wetter recent period.

Figure 34. Annual precipitation for years 1998 - 2024 in mm.

Alt. text. Bar chart showing annual precipitation for years 1998 to 2024 in mm.

From a water-management standpoint, Weno’s frequent reliance on rainwater harvesting, thin freshwater lenses, and increased turbidity during heavy rainfall events highlight the critical importance of monitoring rainfall variability, preparing for short droughts, and maintaining resilient storage capacity.

Evapotranspiration

On Weno, understanding ET is essential for assessing how much rainfall remains available for storage, groundwater recharge, and daily household supply. Because Weno is low lying and experiences fewer topographic controls on cloud formation than the high islands of Kosrae or Pohnpei, its climate is shaped by strong maritime influences, consistent humidity, and a warm boundary layer. These conditions create an energy-limited to mildly water-limited ET regime, depending on season. ET values fall within a narrow band of 96-106 mm per month, varying far less than rainfall, which varies from 189 mm in February to 317 mm in September. During the wet season from July to October, high humidity, frequent cloud cover, and saturated soils constrain ET much like on the higher islands (Figure 35). In the drier months from January to March, however, clearer skies and reduced rainfall increase evaporative demand, allowing ET to rise somewhat even as soil moisture declines.

Figure 35. Mean monthly precipitation (P) and actual ET (ETa) in mm for Weno (2013-2025).

Alt. text. Area chart showing the mean monthly precipitation (P) and actual ET (ETa) in mm for Weno (2013-2025) with blue hatch lines and actual ET in mm in green (solid).

Despite this seasonal contrast, annual actual ET totals on Weno remain relatively stable, reflecting the consistent nature of its tropical maritime climate and the limited variation in solar energy, temperature, and vegetation cover from year to year. Annual ET totals from 2013 through 2025 fall between approximately 1,241 and 1,302 mm per year (Figure 36).

Figure 36. Annual actual ET for years 2013 - 2025 in mm.

Alt. text. Bar chart with annual actual ET for years 2013 - 2025 in mm.

As a result, year-to-year differences in freshwater availability on Weno are driven overwhelmingly by precipitation variability rather than changes in ET. When rainfall is abundant, ET does not increase proportionally, meaning that excess precipitation can contribute directly to groundwater lens recharge, surface runoff to storage tanks, or increased soil moisture. Conversely, during prolonged dry periods or ENSO driven droughts, ET continues at near normal rates, steadily drawing down soil moisture and groundwater reserves even as rainfall inputs diminish. This dynamic highlights a critical vulnerability for Weno: because ET is steady but precipitation is highly variable, any reduction in rainfall translates quickly into reduced net water availability. Thin freshwater lenses and limited storage capacity respond rapidly to this deficit, underscoring the importance of drought preparedness, water use efficiency, and protected rainwater catchment infrastructure. Understanding the stability of ET thus provides essential context for interpreting Weno’s precipitation patterns, water balance behavior, and the degree of stress placed on local freshwater systems during dry periods.

Water Balance

The water balance or net water availability reflects the interaction between substantial but highly variable precipitation, relatively stable evapotranspiration, and the island’s limited capacity to store freshwater. Because actual ET varies much less than rainfall, the net water availability (P minus actual ET) is driven almost entirely by fluctuations in precipitation rather than changes in atmospheric demand. As a result, Weno’s hydrologic system is characterized by periods of pronounced surplus during wet months and rapid deficits during dry spells (Figure 37). Extremely wet months with large surpluses are February 2014 (517 mm) and May 2021 (460 mm). Lower surplus months are commonly in February/March with the year 2016 and 2020 showing dry spells.

Figure 37. Monthly net water availability for years 2012 - 2025 in mm.

Alt. text. Bar chart showing monthly net water availability for years 2012 - 2025 in mm.

During the wet season, frequent rainfall and high humidity produce consistently positive monthly water balances. In these months, precipitation commonly exceeds evapotranspiration by several hundred millimeters, contributing directly to increased soil moisture, temporary surface runoff, and recharge of the island’s thin freshwater lenses. Because ET does not rise proportionally during wetter periods, nearly all excess rainfall becomes available for water‑resource replenishment. These episodic surpluses are critical for sustaining groundwater supplies that communities rely on during drier months. In contrast, during the dry season or during ENSO‑related drought events, Weno experiences substantial reductions in net water availability. Even as rainfall declines sharply, ET continues at near‑normal levels, steadily drawing water from soils and groundwater lenses. This mismatch between input and atmospheric demand leads to rapid lens thinning, heightened salinity intrusion, and increased pressure on rainwater‑harvesting systems. Weno’s small landmass, limited storage infrastructure, and high population density amplify these effects, making the island particularly sensitive to multi‑week and multi‑month rainfall deficits. Interannual variability further reinforces this pattern (Figure 38). Wet years with strong precipitation anomalies yield large annual surpluses, such as 2023, supporting broader freshwater availability across the island. Dry years, such as 2016 and 2020, however, can produce extended periods of negative or near‑zero net water, stressing both natural and human‑built systems.

Figure 38. Net water availability per year in mm.

Alt. text. Horizontal bar chart showing net water availability per year in mm for 2013 to 2024 starting with 2024 at the top.

Because ET remains comparatively steady from year to year, these deficits arise almost entirely from reductions in rainfall, underscoring the island’s reliance on consistent precipitation for freshwater security.

Overall, Weno’s water balance demonstrates that the island’s hydrologic resilience depends heavily on effective rainwater capture, careful groundwater‑lens management, and proactive drought‑preparedness strategies. The stability of ET simplifies long‑term water‑balance interpretation, but it also means that any substantial reduction in rainfall translates directly into reduced freshwater availability. Understanding these dynamics is essential for supporting sustainable water supply, anticipating drought impacts, and guiding infrastructure planning for the island’s communities.

Drought Monitoring

Rainfall Index

The Weno Rainfall Index record (Figure 39) shows Drier classifications in 66 of 314 months (21 percent) and Dry in 60 months (19 percent) at the 1-month timescale. The PEAC index classified 20 months as Red and 90 as Yellow. At the 6-month timescale, 68 months were Drier and 61 were Dry, indicating that cumulative deficits occur with similar frequency to single-month shortfalls, though different months may be involved.

Figure 39. Quintile-based Rainfall Index time series for Weno (Chuuk) showing (A) 1-month, (B) 3-month, and (C) 6-month accumulation windows.

Notable Drought Episodes

The most prolonged drought at Weno occurred from February through October 2016, when 9 consecutive months were classified as Drier or Dry at the 1-month timescale, with 7 months in the Drier category. All 9 months also received Drier classifications at both the 3-month and 6-month windows, indicating severe cumulative deficits across all timescales. The PEAC index classified 2 months as Red during this period, with a minimum monthly precipitation of 23.0 millimeters. This event mirrors the 2015–2016 drought observed at Yap and across the broader Pacific.

Additional significant episodes include February through May 2010 (4 months, 3 Drier at 1-month, minimum 14.0 millimeters, 1 PEAC-Red); a 6-month Drier/Dry period from April through September 2013; and February through April 2020 (3 months, all 3 Drier at 1-month, 3 PEAC-Red, minimum 60.0 millimeters). The most recent below-normal period, May through July 2025, showed 3 consecutive Drier/Dry months with Drier classifications propagating through the 3-month and 6-month windows.

Table 10. Summary of notable drought episodes at Weno (Chuuk).

| Episode | Duration (months) | Drier Months (1-mo) | PEAC-Red Months | Min Precip. (mm) |
| --- | --- | --- | --- | --- |
| Feb 2016 – Oct 2016 | 9 | 7 | 2 | 23.0 |
| Feb 2010 – May 2010 | 4 | 3 | 1 | 14.0 |
| Apr 2013 – Sep 2013 | 6 | 2 | 0 | 160.0 |
| Feb 2020 – Apr 2020 | 3 | 3 | 3 | 60.0 |
| Dec 2008 – Mar 2009 | 4 | 2 | 1 | 75.0 |
| Oct 2015 – Dec 2015 | 3 | 3 | 0 | 107.0 |

Seismicity

The City of Weno sits within the protected confines of Chuuk Lagoon, well inland of the nearest active plate‑boundary faults and oceanic trenches. The lagoon’s islands are located on interior oceanic lithosphere far from the principal circum‑Pacific subduction systems; the closest plate‑boundary features lie to the west and northwest along the Yap–Palau–Mariana trench–arc system (hundreds of kilometers from Weno), where the Pacific Plate interacts with the Philippine Sea Plate and the Australian Plate. As a result, local seismic hazard at Weno is generally low, and there are no mapped active faults or documented sequences of damaging, locally generated earthquakes within the lagoon itself (Rong and others, 2012). Regional seismicity associated with the Yap and Mariana systems does occur and can occasionally be detected in Chuuk, typically as weak or moderate shaking from events located hundreds of kilometers offshore. One example is a M 4.9 earthquake on May 24, 2025 west of Chuuk that was located ~430 km from Weno. Deep and intermediate‑depth earthquakes are common along the broader western Pacific subduction zones, but over these distances, wave attenuation and oceanic path effects typically limit felt intensity on Chuuk’s islands. Consequently, perceptible shaking in Weno is infrequent, and damaging ground motions are uncommon; nonetheless, routine awareness and preparedness are appropriate given the region’s proximity to active subduction systems and the wider Ring of Fire context.

Discussion

The water balance across the four study sites in the FSM is generally positive, meaning that annual precipitation consistently exceeds evapotranspiration. This favorable balance is crucial for sustaining freshwater supplies on the islands and atolls. However, the region's monsoon climate results in considerable variability in precipitation delivery throughout the year, making the islands vulnerable to water shortages during episodic droughts. Drought monitoring has revealed both the frequency and severity of such events. The 2015–2016 drought stands out as the most severe and widespread, affecting all four study sites. At Yap, for instance, seven consecutive months were classified as Drier or Dry at the 1-month timescale, with six months marked as PEAC-Red, indicating acute water stress. This event persisted across all sites, with multi-timescale Drier classifications propagating through the 3-month and 6-month windows, underscoring the susceptibility of each island to large-scale climate anomalies.

The application of two drought indices—the PEAC 4/8-inch (10.2 cm/ 20.4 cm) index and a quintile-based Rainfall Index—offers complementary perspectives. The PEAC index exposes a pronounced east-west gradient in absolute water supply risk: for example, 18 percent of months at Yap fell below the 4-inch (10.2 cm) Red threshold, compared to only 1 percent at Kosrae. In contrast, the Rainfall Index shows that below-normal conditions occur with similar frequency at all sites (about 40 percent of months classified as Drier or Dry). This suggests that even the wettest locations are not immune to relative precipitation deficits that can affect ecosystems, groundwater recharge, and water infrastructure.

Employing a multi-timescale approach proved effective in distinguishing transient precipitation shortfalls from developing droughts. The 6-month monitoring window identified extended events like Kosrae’s eight-month drought in 2010–2011 and Weno’s nine-month drought in 2016, episodes that single-month analyses might overlook. This method allows for improved characterization of drought recurrence patterns and supports proactive water management and infrastructure planning.

The 7-day GFS precipitation forecasting system retrieves NOAA GFS forecast data at 0.25° resolution, extracts daily accumulated precipitation for each monitoring location, applies operational alert thresholds, and generates a standardized suite of output products including bar charts, spatial animations, summary tables, and a packaged distribution archive. The processing pipeline completes in approximately 30 minutes and is designed for automated, routine deployment.

The March 10–16, 2026 demonstration forecast showed 7-day totals ranging from 43.2 mm at Pohnpei to 64.5 mm at Kosrae, with no daily or weekly alert thresholds exceeded. This moderate-to-low forecast is operationally relevant for drought-affected locations, as it signals to water managers that the near-term outlook does not include substantial rainfall relief.

The forecasting system fills a critical gap in water management across the FSM, where sparse ground-based weather station networks and limited access to numerical weather prediction products have historically left many communities without reliable short-term rainfall outlooks. When used in conjunction with the IMERG-based drought monitoring system, the combined backward-looking and forward-looking capabilities provide a comprehensive precipitation intelligence framework for Pacific Island water supply management.

In summary, while the annual water balance at all four study sites is typically positive, the islands remain vulnerable to significant droughts due to the variability of monsoon-driven rainfall. Continued monitoring and the use of complementary drought indices are essential for safeguarding water resources and informing climate adaptation strategies across the region.

Conclusion

The islands and atolls of the Federated States of Micronesia generally have a positive annual water balance as precipitation is typically greater than evapotranspiration. However, the monsoon climate results in precipitation being delivered to the islands varies greatly with time, and residents could face acute water shortages during episodic regional drought events.

Regarding drought monitoring, the 2015–2016 drought was the most severe and widespread event recorded during the 25-year monitoring period. At Yap, 7 consecutive months were classified as Drier or Dry at the 1-month timescale with 6 PEAC-Red months, while the event persisted at all four sites with multi-timescale Drier classifications propagating through the 3-month and 6-month windows. This region-wide event underscores the vulnerability of all FSM states to large-scale climate forcing.

The two indices provide complementary perspectives on drought conditions. The PEAC 4/8-inch (10.2 cm/ 20.4 cm) index reveals a strong east-west gradient in absolute water supply risk: 18 percent of months at Yap fell below the 4-inch (10.2 cm) Red threshold compared to only 1 percent at Kosrae. However, the quintile-based Rainfall Index shows that all four sites experience below-normal conditions with comparable frequency (approximately 40 percent of months classified as Drier or Dry), indicating that even the wettest sites are not immune to relative precipitation deficits that may affect ecosystem health, groundwater recharge, and water infrastructure planning.

The multi-timescale approach proved effective for distinguishing between transient precipitation deficits and developing sustained drought. The 6-month window identifies events such as Kosrae’s 8-month 2010–2011 drought and Weno’s 9-month 2016 drought that would not be apparent from single-month monitoring alone.

Continued monitoring using this framework will extend the observational record and improve the characterization of drought recurrence patterns at each location. The results presented in this volume, together with the detailed methods in Volume 1, provide an operational foundation for satellite-based drought monitoring in the FSM that can support water supply management, infrastructure planning, and climate adaptation decision-making.

References

ADB, 1965, Agreement establishing the Asian Development Bank signed at Manila, Dec. 4, 1965; entered into force Aug. 22, 1966, United Nations Treaty Series, vol. 608, p. 380. Available at: Agreement Establishing the Asian Development Bank (ADB Charter). Accessed 29 January 2026.

Bailey, R.T., and Jenson, J.W., 2011, Groundwater resources analysis of atoll islands in the

Federated States of Micronesia using an algebraic model: University of Guam, Water and Environmental Research Institute Technical Report 134, vii 73 p.

Climate-Data.org, 2026, Climate Chuuk (Federated States of Micronesia): Climate-Data.org, accessed April 12, 2026, at https://en.climate-data.org/oceania/federated-states-of-micronesia/chuuk-2649/

David Aranug, 2016, Climate variability and change of the Federated States of Micronesia: National Weather Service–Micronesia Operational Hydrologic Network presentation, accessed April 11, 2026, at http://www.globalislands.net/userfiles/FSM16.pdf

DeJonge, K.C., Allen, R.G., Kilic, A., Thorp, K.R., Kukal, M., Marek, G., Altenhofen, J., Amatya, D.M., Blankenau, P., Datta, S., Grabow, G., Hashem, A., Kisekka, I., Kjaersgaard, J., Marek, T., Peters, T., Porter, D., Reba, M., Rudnick, D., Senay, G., Sharma, V., Sridhar, V., Sun, G., Taghvaeian, S., Trezza, R., and Trout, T., 2025, Evapotranspiration terminology and definitions: U.S. Geological Survey Publication, accessed April 11, 2026, at https://www.usgs.gov/publications/evapotranspiration-terminology-and-definitions.

Hatfield, J.L., and Prueger, J.H., 2010, Spatial and temporal variation in evapotranspiration: IntechOpen, 56 p., https://doi.org/10.5772/54221

Hijmans, R.J., 2024, terra—Spatial data analysis: Comprehensive R Archive Network (CRAN), R package, accessed 2024, at https://CRAN.R-project.org/package=terra.

Huffman, G.J., E. F. Stocker, D. T. Bolvin, E. J. Nelkin, and J. Tan, 2019, GPM IMERG Final Precipitation L3 1 Day 0.1°×0.1° V07 (GPM_3IMERGDF), Goddard Earth Sciences Data and Information Services Center (GES DISC), accessed April 10, 2026, https://doi.org/10.5067/GPM/IMERGDF/DAY/07

Huffman, G.J., Bolvin, D.T., Braithwaite, D., Hsu, K.L., Joyce, R.J., Kidd, C., Nelkin, E.J., Sorooshian, S., Stocker, E.F., Tan, J., Wolff, D.B., and Xie, P., 2020, Integrated Multi-satellite Retrievals for the Global Precipitation Measurement (GPM) Mission (IMERG), in Levizzani, V., Kidd, C., Kirschbaum, D.B., Kummerow, C.D., Nakamura, K., and Turk, F.J., eds., Satellite Precipitation Measurement: Advances in Global Change Research, v. 67, Springer, Cham, p. 343–353, https://doi.org/10.1007/978-3-030-24568-9_19.

Jenson, J.W., et al., 2008, Sustainable conjunctive use of groundwater and rain-catchment systems on Yap State atoll communities, Water and Environmental Research Institute of the Western Pacific (WERI), University of Guam report.

Karig, D. E., Anderson, R. N., and Bibee, L. D., 1978, Characteristics of back-arc spreading in the Mariana Trough: Journal of Geophysical Research, v. 83, p. 1213–1226, doi:10.1029/JB083iB03p01213.

Klaus, A., Taylor, B., Moore, G., Okamura, Y., and Murakami, F., 1992, Multichannel seismic and swath-mapping investigations of back-arc rifting in the Izu-Bonin Arc (abstract), in Abstracts, 1992 Japan Earth and Planetary Science Joint Meeting, p. 325.

Leeling, D., 1996, Kosrae temperature and precipitation (rainfall) dataset, 1954–1995: College of Micronesia–FSM, digital dataset.

Mink, J.F., Murabayashi, E.T., and Lau, L.S., 1992, Geology and hydrogeology of the island of Pohnpei, Federated States of Micronesia: Hawaii Water Resources Research Center, Technical Report No. 189, 148 p.

National Aeronautics and Space Administration, 2025, IMERG—Integrated Multi-satellitE Retrievals for GPM: National Aeronautics and Space Administration digital data, accessed March 2025, at https://gpm.nasa.gov/data/imerg.

Pacific ENSO Applications Climate Center [PEAC], 2015, 3rd quarter, 2015: Pacific ENSO Update, v. 21, no. 3, accessed 2023, at https://www.weather.gov/media/peac/PEU/PEU_v21_n3.pdf.

Pebesma, E., 2018, Simple features for R—Standardized support for spatial vector data: The R Journal, v. 10, no. 1, p. 439–446, https://doi.org/10.32614/RJ-2018-009.

R Core Team, 2024, R—A language and environment for statistical computing: Vienna, Austria, R Foundation for Statistical Computing, https://www.R-project.org/.

Rong, Y., Park, J., Duggan, D., Mahdyiar, M., and Bazzurro, P., 2012, Probabilistic seismic hazard assessment for Pacific Island Countries: in Proceedings of the 15th World Conference on Earthquake Engineering, Lisbon, Portugal, Paper 2521, 10 p.

Raynor, B., 1995, Montane cloud forests in Micronesia: Status and future management, in Hamilton, L.S., Juvik, J.O., and Scatena, F.N., eds., Tropical Montane Cloud Forests: Ecological Studies, v. 110, p. 274–283, Springer-Verlag, New York.

Rehman, H. U., Nakaya, H., and Kawai, K., 2013, Geological origin of the volcanic islands of the Caroline Group in the Federated States of Micronesia, western Pacific: South Pacific Studies, v. 33, no. 2, p. 101–118.

Samson, F., and Ruegorong, Y., 2021, Watershed-based assessment of food security and biodiversity on Yap—Yap State Forestry and the Institute of Pacific Islands Forestry, Hilo, Hawaii, report 12–1G-11052021–223.

Scholz, C. H., and Campos, J., 1995, On the mechanism of seismic decoupling and back-arc spreading at subduction zones: Journal of Geophysical Research – Solid Earth, v. 100, no. B11, p. 22,103–22,115, doi:10.1029/95JB01869

Senay, G.B., Helweg, D.A., Kagone, S., Taylor, J.B., Cecere, T., Eastmond, T., Koch, A., Nelson, K., and Rufus, L., 2023, Developing satellite-estimated precipitation monthly reports for selected locations in the Republic of the Marshall Islands: U.S. Geological Survey Data Report 1181, 7 p., https://doi.org/10.3133/dr1181.

Senay, G.B., Kagone, S., and Velpuri, N.M., 2020, Operational global actual evapotranspiration using the SSEBop model—USGS data release: U.S. Geological Survey Data Release, DOI:10.5066/P9OUVUUI.

Shade, P.J., Anthony, S.S., and Takasaki, K.J., 1992, Ground-water resources reconnaissance of the Yap main islands, Federated States of Micronesia: U.S. Geological Survey Water-Resources Investigations Report 90-4074, vi, 71 p., https://doi.org/10.3133/wri904074

Spengler, S. R., Peterson, F. L., and Mink, J. F., 1991, Geology and hydrogeology of the island of Pohnpei, Federated States of Micronesia (Water Resources Research Center Technical Report No. 189, University of Hawaii at Manoa): University of Hawaii, Water Resources Research Center, 79 p.

Spengler, S. R., Spencer, K. J., Mahoney, J. J., and Goles, G. G., 2012, Geology and geochemical evolution of lavas on the island of Pohnpei, Federated States of Micronesia: University of Hawaii and University of Oregon report, 16 p.

Tribble, G.W., 2008, Ground water on tropical Pacific Islands—understanding a vital resource: U.S. Geological Survey Circular 1312, 35 p.

USGS Data release

U.S. Geological Survey, 2021 (updated), Landslides—Subduction Zone Science, accessed April 11, 2026, at https://www.usgs.gov/special-topics/subduction-zone-science/science/landslides

U.S. Geological Survey, 2015, USGS 10-m Digital Elevation Model (DEM): FSM: Kosrae (Digital dataset): U.S. Geological Survey, accessed via Pacific Islands Ocean Observing System, May 11, 2015, at http://pacioos.org/metadata/usgs_dem_10m_kosrae.html.

U.S. Geological Survey, 2018, Saltwater contamination of freshwater resources could make many atoll islands uninhabitable in decades: U.S. Geological Survey News Release, April 25, 2018, accessed April 12, 2026, at https://www.usgs.gov/news/saltwater-contamination-freshwater-resources-could-make-many-atoll-islands-uninhabitable

U.S. Geological Survey, 2025a, M 5.9 – State of Yap, Federated States of Micronesia (2025-10-13): USGS Earthquake Catalog, retrieved Apr. 17, 2026, from https://earthquake.usgs.gov/earthquakes/eventpage/us6000rgul

U.S. Geological Survey, 2025b, M 5.4 – South of the Fiji Islands (2025-10-31): USGS Earthquake Catalog, retrieved Apr. 17, 2026, from https://earthquake.usgs.gov/earthquakes/eventpage/us6000rkpc

Van der Brug, O., 1984, Water resources of Kosrae, Caroline Islands: U.S. Geological Survey Water-Resources Investigations Report 83-4161, 143 p., https://doi.org/10.3133/wri834161.

Werner, A.D., Sharp, H.K., Galvis, S.C., Post, V.E.A., and Sinclair, P., 2017, Hydrogeology and management of freshwater lenses on atoll islands: Review of current knowledge and research needs: Journal of Hydrology, v. 550, p. 111–125.

Wickham, H., Averick, M., Bryan, J., Chang, W., McGowan, L.D., François, R., Grolemund, G., Hayes, A., Henry, L., Hester, J., Kuhn, M., Pedersen, T.L., Miller, E., Bache, S.M., Müller, K., Ooms, J., Robinson, D., Seidel, D.P., Spinu, V., Takahashi, K., Vaughan, D., Wilke, C., Woo, K., and Yutani, H., 2019, Welcome to the Tidyverse: Journal of Open Source Software, v. 4, no. 43, p. 1686, https://doi.org/10.21105/joss.01686.

Zong, T., Li, Z.-G., Dong, Y.-H., Li, X.-P., and Zhu, J.-H., 2020, Geochemical constraints on mantle melting and magma genesis at Pohnpei Island, Micronesia, Minerals, v. 10, no. 9, 816, doi:10.3390/min10090816.