National Geospatial Program

Prepared in cooperation with the Asian Development Bank

Increasing Water Security on Pacific Islands Through Increased Data Availability and Understanding—Volume 3, Republic of Fiji

By Gabriel B. Senay,1 Francis K. Dwomoh,2 Stefanie Kagone,2 Tomas Smieszek1, and Matthew E. Andersen1

1U.S. Geological Survey.

2ASRC Federal, contractor to the U.S. Geological Survey (USGS) Earth Resources Observations and Science (EROS) Center, Sioux Falls, SD 57198, USA. Work performed under USGS Contract [140G0124D0001].

Any use of trade, firm, or product names is for descriptive purposes only and does not imply endorsement by the U.S. Government.

Although this information product, for the most part, is in the public domain, it also may contain copyrighted materials as noted in the text. Permission to reproduce copyrighted items must be secured from the copyright owner.

This draft manuscript is distributed solely for purposes of courtesy review and comments received will be addressed and treated as appropriate to ensure there is no conflict of interest. Its content is deliberative and predecisional, so it must not be disclosed or released by reviewers. Because the manuscript has not yet been approved for publication by the U.S. Geological Survey (USGS), it does not represent any official USGS finding or policy.

Acknowledgments

This work was conducted in collaboration with the Strengthening Resilient and Sustainable Urban and Water Service Delivery in the Pacific project of the Water and Urban Development Sector of the Asian Development Bank (ADB). ADB provided financial support from some USGS technical salaries and associated administrative costs, including some of the travel necessary to administer and manage the work.

Outline

Acknowledgments	1

Introduction	5

Purpose and Scope	6

Republic of Fiji	7

Island of Viti Levu including capital city of Suva	11

Precipitation	13

Evapotranspiration	18

Water Balance	20

Informal Settlement Mapping	23

Seismicity	39

Discussion	40

Conclusion	42

References Cited	43

Figures

Figure 1. Map of Western Pacific Ocean Island Nations. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Western Pacific Ocean island nations. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

Figure 2. A simplified topographic map of the Republic of Fiji. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Republic of Fiji. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

Figure 3. Viti Levu Island with capital Suva. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing Viti Levu Island with capital Suva. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

Figure 4. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

Figure 5. Monthly precipitation for the Suva watershed for years 1998 - 2025 in mm.

Figure 6. Annual precipitation for years 1998 – 2024 in mm.

Figure 7. Mean monthly precipitation (P) and actual ET (ETa) in mm for Viti Levu (2013-2025).

Figure 8. Annual actual ET for years 2013 - 2025 in mm.

Figure 9. Monthly net water availability for years 2012 - 2025 in mm.

Figure 10. Net water availability per year in mm.

Figure 11. Building count growth for both Fiji study sites across the three survey epochs. Note the near-flat trajectory at Tacirua between 2009 and 2014, followed by substantial growth in the 2014–2025 interval.

Figure 12. Absolute change in building count by survey period for both Fiji study sites. The contrast between the two periods at Tacirua is the defining feature of the Fiji dataset.

Figure 13. Compound annual growth rates for both Fiji study sites by survey period. Blue shaded band = ADB Melanesian benchmark (3–4%/yr). The near-zero 2009–2014 rate at Tacirua contrasts markedly with above-benchmark growth in 2014–2025.

Figure 14. Settlement density (structures per hectare) for both Fiji study sites across all survey epochs. Yellow band = ADB Pacific norm (15–25 str/ha). Both sites remain well below the norm across all three epochs.

Figure 15. Built-up coverage ratio (%) for both Fiji study sites by survey epoch. Both sites show monotonically increasing trajectories, with the most rapid increase at Tacirua in the 2014–2025 interval.

Figure 16. Median building footprint area over time for both Fiji study sites. Bubble size is proportional to total building count. The large median footprint sizes relative to PNG and Solomon Islands are a distinctive characteristic of the Fiji sites.

Tables

International System of Units to U.S. customary units

Table 1. Average IMERG Precipitation Statistics (1998–2024) for Viti Levu

Table 2. Fiji study site summary. Settlement areas derived from ADB-provided boundary shapefiles

Table 3. Building count summary for Fiji study sites across three survey epochs.

Table 4. Compound annual growth rate (CAGR) by period for Fiji study sites. ADB Melanesian benchmark: 3–4%/yr.

Table 5. Settlement density by epoch for Fiji study sites. Pacific norm: 15–25 str/ha.

Table 6. Built-up coverage ratio and total building footprint area by epoch for Fiji study sites.

Table 7. Building footprint size statistics by epoch for Fiji study sites.

Conversion Factors

International System of Units to U.S. customary units

| Multiply | By | To obtain |
| --- | --- | --- |
| Length |  |  |
| millimeter (mm) | 0.03937 | inch (in.) |
| meter (m) | 3.281 | foot (ft) |
| kilometer (km) | 0.6214 | mile (mi) |
| Area |  |  |
| square meter (m2) | 0.0002471 | acre |
| hectare (ha) | 2.471 | acre |
| square hectometer (hm2) | 2.471 | acre |
| square kilometer (km2) | 247.1 | acre |
| square centimeter (cm2) | 0.001076 | square foot (ft2) |
| square meter (m2) | 10.76 | square foot (ft2) |
| square centimeter (cm2) | 0.1550 | square inch (in2) |
| square hectometer (hm2) | 0.003861 | section (640 acres or 1 square mile) |
| hectare (ha) | 0.003861 | square mile (mi2) |
| square kilometer (km2) | 0.3861 | square mile (mi2) |

Abbreviations

ADB	Asian Development Bank

FSM	Federated States of Micronesia

USGS	U.S. Geological Survey

ET	Evapotranspiration

P	Precipitation

ENSO	El Niño–Southern Oscillation

VHR 	Very-high-resolution

Introduction

Freshwater is critically important for sustaining life, supporting communities, and enabling economic activities across the Pacific islands. On Viti Levu, Fiji’s largest and most populated island, reliable hydrologic information is increasingly essential for utilities, resource managers, and the public to plan for growing water demands, reduce the impacts of droughts and climate variability, and strengthen long-term water security. The Asian Development Bank (ADB), whose mission includes supporting sustainable development and building climate resilience across Asia and the Pacific (ADB, 1965), recognizes that dependable access to safe water underpins public health, environmental stability, and economic growth.

Improved understanding of the timing, magnitude, and variability of freshwater availability enables water managers on Viti Levu to make informed decisions about resource allocation, infrastructure investment, and preparation for periods of deficit. Potential development actions, such as expanding water storage, enhancing supply system resilience, and protecting watersheds, depend on consistent, accessible hydrologic information. Satellite-based datasets offer an efficient and scalable way to meet these needs across Fiji’s diverse and often remote catchments.

To help address these challenges, ADB and the U.S. Geological Survey (USGS) partnered to evaluate and demonstrate how Earth observation products can support water resource management in Fiji, with a focus on Viti Levu including the capital city of Suva. This collaboration analyzes long-term satellite datasets for precipitation, evapotranspiration (ET), and seasonal rainfall patterns. In addition, multi-temporal building footprint extraction was carried out at two informal settlement sites, Tacirua and Wailekutu, in the Greater Suva Urban Area to quantify settlement growth over approximately two decades. This analysis is directly relevant to the hydrological work because the rapid, unplanned expansion of informal communities is increasing water demand and altering catchment characteristics in areas where reticulated supply has not kept pace with growth, and understanding the pace and location of that expansion provides an empirical basis for targeting water infrastructure investment alongside the precipitation and drought monitoring components of this study. The methods used to develop these datasets are detailed in Volume 1 – Overview of this series, while this volume applies those methods to the hydrologic and climate conditions relevant to Viti Levu.

Purpose and Scope

This report presents the results of a collaborative effort by the USGS and ADB to assess the applicability of remotely sensed Earth-observation data for improving understanding of water-resource conditions across the Republic of Fiji. The work supports regional efforts to address persistent water-availability challenges on Pacific islands, where freshwater supplies can be limited for extended periods.

This volume documents the analytical components of the study and provides results for the Republic of Fiji including the Island of Viti Levu and the city of Suva as the capital and most populous city. The scope of the report includes satellite based assessments of precipitation, evapotranspiration, and informal settlements mapping and settlement growth analysis. Methods used to generate these datasets are presented in Volume 1 of this series; the present volume focuses on applying those methods within Fiji and summarizing the resulting water availability information.

Each chapter describes the datasets used, outlines key processing steps, and presents the derived products for the respective study area. The information provided is intended to assist water managers, utilities, and planners in evaluating hydrologic conditions, identifying periods of relative water surplus or shortage, and supporting decisions related to water conservation, infrastructure planning, and resource management.

Republic of Fiji

Beginning in 2024, the USGS entered a collaborative relationship with ADB to assess the utility of various remotely sensed datasets for improving water resource management within the Republic of Fiji, an archipelago situated in the southwest Pacific and composed of more than 300 islands (U.S. Geological Survey, 2026) distributed between approximately 15° and 22° south latitude (Figure 1).

Figure 1. Map of Western Pacific Ocean Island Nations. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Western Pacific Ocean island nations. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

Fiji’s two largest islands—Viti Levu and Vanua Levu—contain most of the nation’s land area, population, and surface-water resources, while numerous smaller islands exhibit a wide range of hydroclimatic and geologic conditions (Figure 2).

Figure 2. A simplified topographic map of the Republic of Fiji. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Republic of Fiji. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

High volcanic islands such as Viti Levu and Vanua Levu support steep forested watersheds, relief-affected precipitation regimes, and extensive perennial river networks that respond quickly to tropical rainfall systems and cyclonic storms (Kostaschuk and others, 2001). In contrast, Fiji’s low-lying outer islands, many of which are carbonate or reef derived, rely more heavily on shallow freshwater lenses based on groundwater sources that are sensitive to changes in rainfall, evapotranspiration, storm surge, and sea level variability (White and Falkland, 2010). This distribution across multiple island types exposes Fiji to strong spatial gradients in precipitation driven by the South Pacific Convergence Zone, Tradewind rainfall patterns, tropical cyclone activity, and ENSO related drought cycles which are factors that strongly influence water availability, especially for communities with limited surface water storage or groundwater reserves.

Fiji also lies within an active tectonic environment shaped by interactions among the IndoAustralian and Pacific Plates (Hamburger and others, 1988). Although large earthquakes are less frequent than in some neighboring regions, tectonic uplift, volcanic history, and localized faulting continue to influence long-term island morphology, watershed evolution, and hazards such as landslides or coastal instability that can affect drinking water infrastructure and aquifer systems. In consultation with Fiji’s national and local water authorities, ADB identified Viti Levu as a priority area for expanding precipitation, evapotranspiration, water balance, and drought monitoring datasets, reflecting locations where hydrologic vulnerability, population density, and exposure to climatic and geologic variability are greatest.

Island of Viti Levu including capital city of Suva

Suva is located on the southeastern coast of Viti Levu near 18°S and 178°E and is the capital and largest urban center in the Republic of Fiji.

Figure 3. Viti Levu Island with capital Suva. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing Viti Levu Island with capital Suva. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

The city occupies a series of low hills and coastal terraces that overlook Suva Harbour and Laucala Bay, forming one of the most protected deep-water ports in the region. Suva’s built environment extends across roughly 26 square kilometers, with residential and commercial districts situated on narrow coastal plains backed by steep, forested slopes characteristic of Viti Levu’s volcanic terrain.

As Fiji’s administrative and economic hub, Suva supports a population exceeding 90,000 within the city and more than 185,000 across the larger metropolitan area (Fiji Bureau of Statistics, 2017). Population density is highest along the coastal perimeter, where public facilities, government buildings, and transportation networks are concentrated. The region experiences a humid tropical climate influenced by the South Pacific Convergence Zone, with high annual rainfall and pronounced wet-season peaks. These conditions contribute to frequent localized flooding, rapid runoff from surrounding uplands, and variable water-supply pressures during prolonged dry intervals.

Suva’s water-resource challenges reflect its physiographic setting, reliance on surface-water catchments originating in the adjacent highlands, and sensitivity to extreme rainfall events, tropical cyclones, and periodic droughts. The combination of steep watershed gradients, expanding urban development, and limited land area for storage infrastructure underscores the need for improved precipitation monitoring, evapotranspiration estimates, and drought-risk assessments for areas supported by the remotely sensed datasets analyzed in this volume.

Precipitation

Precipitation is the primary source of freshwater across Pacific Island environments, because islands lack the buffering capacity of large watersheds or reservoirs (Tribble, 2008). Therefore, understanding spatial and temporal rainfall variability is essential for evaluating water availability, assessing drought risk, and guiding sustainable management of hydrologic resources. Satellite derived precipitation datasets provide a consistent means of monitoring these patterns across remote and data-sparse island settings (Figure 4).

Figure 4. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

Alt. text. Graphical representation of mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

Precipitation patterns for Viti Levu, and the other islands, were derived using the Integrated Multi-satellitE Retrievals for GPM (IMERG) precipitation dataset (Huffman et al., 2019). After the data was acquired, the island average for every day from 1. January 1998 to 31. September 2025 was created and can be accessed from (USGS Data release).

Viti Levu has a humid tropical climate influenced by both orographic rainfall and seasonal weather patterns, with wet conditions persisting throughout the year but still experiencing periods of relatively reduced rainfall. The mean annual total precipitation is 2180 mm and the mean monthly precipitation is 182 mm. Over the period analyzed, the mean minimum monthly precipitation is 58 mm, indicating that even during the driest months the island still receives notable rainfall, while the mean maximum monthly precipitation reaches 322 mm.

Table 1. Mean IMERG Precipitation Statistics (1998–2024) for Viti Levu

| Statistic | Precipitation (mm) |
| --- | --- |
| Mean annual | 2180 |
| Mean monthly mean | 182 |
| Mean monthly minimum | 58 |
| Mean monthly maximum | 322 |

Viti Levu receives moderate to high precipitation throughout the year, with monthly rainfall totals generally ranging from about 60 to more than 300 millimeters, consistent with the island’s humid tropical climate. Although rainfall occurs year-round, a seasonal pattern is typically present, with enhanced precipitation commonly observed during the months of October through April (Figure 5). Several months exceed 300 millimeters, such as January 2009 (800 mm), March 2024 (738 mm), and March 2020 (658 mm) indicating the potential for intense, short‑duration wet seasons due to the influence of regional weather systems and orographic effects across the island’s mountainous interior. Conversely, drier months occur regularly, with the lowest monthly totals averaging around 58 millimeters, indicating that short dry spells can develop despite the island’s generally moist climate; for example, August 2018 recorded only about 3 mm of precipitation, representing one of the driest months in recent years.

Figure 5. Monthly precipitation for the Suva watershed for years 1998 - 2025 in mm.

Alt. text. Line graph showing the monthly precipitation for years 1998 - 2025 in mm.

Annual precipitation on Viti Levu varies from year to year, but the overall range is more moderate compared to the extremely wet islands of the central Pacific. Annual totals generally cluster around the long-term mean of approximately 2180 mm/yr, with individual years fluctuating between notably wetter and drier conditions (Figure 6). Several years exceed 2000 mm including in 2012 (3018 mm), 2000 (2842 mm) and most recently 2024 (2744 mm) during periods influenced by strong regional climate anomalies or enhanced convective activity, while the driest years fall below 1500 mm, such as 2015 or 1998, reflecting the island’s sensitivity to synoptic-scale weather patterns and El Niño–Southern Oscillation–related variability. Although the most recent years show modest departures from the long-term average, no sustained multi-year trend in extreme wetness is evident, and interannual variability remains within the typical historical range for the island.

Figure 6. Annual precipitation for years 1998 – 2024 in mm.

Alt. text. Bar chart showing annual precipitation for years 1998 to 2024 in mm.

From a water-resource perspective, this level of variability supports generally reliable surface-water availability, as even the lower end of the annual rainfall range (around 1500 mm) typically provides sufficient input for most management needs. Although year-to-year swings can reach 100%, such variability is normal for Fiji’s strongly seasonal and interannually dynamic climate, with nearly a century of data showing high interannual fluctuation but no long-term trend (Weir and others, 2014). Even so, management strategies should continue to account for occasional dry months by incorporating measures such as seasonal storage planning, controlled reservoir releases, or temporary demand-reduction protocols. During wetter periods, managers may also need to address increases in turbidity or short-duration runoff peaks through practices like enhanced sediment monitoring, adaptive intake operations, or upstream erosion-control interventions. This helps ensure system robustness even when rainfall varies substantially in relative terms but remains adequate in absolute amounts.

Evapotranspiration

Because precipitation provides the dominant water input to Viti Levu’s hydrologic system, understanding how much of this water is subsequently lost to the atmosphere is essential for evaluating freshwater availability. Evapotranspiration (ET) represents the combined atmospheric water loss from evaporation (soil, water, and surfaces) and transpiration (vegetation) (DeJonge and others, 2025), and reflects the interaction of available solar energy, humidity, wind, and land cover. Actual ET, as estimated by the SSEBop model (Senay and others, 2020), represents the portion of potential ET that is realized given the surface and atmospheric conditions.

On Viti Levu, ET exhibits moderate seasonal variability but remains more stable than precipitation throughout the year (Figure 7). The island’s rainfall pattern is strongly seasonal, with wet-season peaks commonly occurring from October through April, reaching its highest average amount in March with 322 mm. The dry season brings markedly reduced precipitation when the lowest average amount falls in July with 58 mm. In contrast, monthly actual ET varies within a narrower range, governed primarily by changes in solar radiation and humidity rather than by moisture availability, reaching its lowest in May/June (71 mm) and its highest in December/January (141 mm). ET typically reaches its lowest values during the wettest months, when high humidity and greater cloud cover limit evaporative demand, and increases during the drier, sunnier months of the year. This pattern reflects Viti Levu’s partially energy limited environment, where both humidity and cloud cover modulate the surface energy balance.

Figure 7. Mean monthly precipitation (P) and actual ET (ETa) in mm for Viti Levu (2013-2025).

Alt. text. Area chart showing the mean monthly precipitation (P) with blue hatch lines and the mean actual ET (ETa) in mm in solid green for Viti Levu (2013-2025).

Annual ET totals show similarly limited variability. Across the available record (2013–2025), actual ET varies only modestly from 1262 mm in 2020 to 1305 mm in 2023 (Figure 8), with annual totals remaining within a relatively narrow band compared to the much larger interannual fluctuations in precipitation. This consistency underscores the energy‑driven nature of ET on Viti Levu and reinforces the dominant role of precipitation in shaping hydrologic surplus or deficit.

Figure 8. Annual actual ET for years 2013 - 2025 in mm.

Alt. text. Bar chart with annual actual ET for years 2013 - 2025 in mm.

The stability of ET, combined with Viti Levu’s moderate to high rainfall, results in a generally reliable hydrologic surplus that supports perennial streamflow, recharge, and storage. However, because ET remains comparatively steady while precipitation can vary substantially, hydrologic conditions on the island remain highly sensitive to reductions in rainfall during regional drought events. Water-resource planning should therefore continue to account for seasonal shifts in atmospheric demand, dry-season precipitation deficits, and the potential need for storage or turbidity management during intense wet-season storms.

Water Balance

The water balance, or net water availability, represents the surplus of water remaining after subtracting actual ET from precipitation (P) at monthly and annual scales. The water balance is computed as difference between P and ETa (P- ETa) at monthly and annual scales.  Although change in storage can be considered negligible at annual time scales, the monthly water balance does not take into account storage changes in the system and thus should only be consulted as a relative indicator of P and ETa.

On Viti Levu, monthly net water is positive during most of the year, though with more variability than on extremely wet Pacific islands. Net monthly values range from large wet-season surpluses, such as January 2019 (538 mm), April 2020 (310 mm), January 2021 (288 mm), December 2021 (180 mm), January 2022 (617 mm), and February 2022 (289 mm), to negative values that occur during the drier months when ET exceeds precipitation. The largest deficits include July 2015 (–92 mm), August 2015 (–113 mm), August 2016 (–97 mm), October 2017 (–87 mm), August 2018 (–87 mm), and February 2020 (–1.5 mm) (Figure 9). These patterns reflect Viti Levu’s strong seasonal cycle: wet-season months (typically October–April) commonly produce surpluses exceeding 100–300 mm, while dry-season months (May–September) often show reduced surpluses or temporary deficits.

Figure 9. Monthly net water availability for years 2012 - 2025 in mm.

Alt. text. Bar chart showing monthly net water availability for years 2012 - 2025 in mm.

At the annual scale, Viti Levu’s net water availability ranges from a low of approximately 120 mm in 2015 to a high of approximately 1,468 mm in 2022 (Figure 10). Other notable annual surpluses include 2013 (766 mm), 2016 (1,285 mm), 2018 (1,227 mm), 2019 (1,017 mm), and 2024 (1,474 mm). Across the most recent five-year period (2020–2024), the mean annual net water is approximately 1,135 mm/yr, indicating a consistent but more moderate surplus than that of extremely high‑rainfall islands.

Figure 10. Net water availability per year in mm.

Alt. text. Horizontal bar chart showing net water availability per year in mm for 2013 to 2024 starting with 2024 at the top.

Overall, the combination of strong wet season surpluses and stable ET supports reliable annual freshwater availability for Viti Levu. The occurrence of negative monthly values during the dry season highlights the importance of monitoring short term deficits, ensuring adequate storage, and preparing for turbidity or flood related impacts during intense wet season events.

Informal Settlement Mapping

Fiji is the most urbanized state in Melanesia, with more than 50 per cent of its national population now living in urban areas, a proportion projected to increase to approximately 60 per cent within the coming decade (Phillips & Keen, 2016). Suva, the national capital and principal commercial city, is the focal point of this urbanization, anchoring the Greater Suva Urban Area (GSUA) on the southeastern coast of Viti Levu. The GSUA encompasses Suva City, Lami Town, Nasinu Town, and Nausori Town, and is home to well over 200,000 residents, representing the country’s most densely settled urban agglomeration (UN-Habitat, 2012).

Urban growth in greater Suva has produced a significant and expanding informal settlement sector. UN-Habitat (2012) documented the challenges of rapid expansion in the GSUA, noting the proliferation of informal communities in peri-urban corridors where reticulated water supply, sewerage, and road access have not kept pace with settlement growth. Phillips and Keen (2016) identified approximately 72 informal settlements in the greater Suva area earmarked for upgrading under the government’s National Housing Policy and characterized the urban governance challenge as one of managing exclusion, inequality, and irregular land access within a highly constrained land market. The land tenure context in Fiji is distinctive: around 87 per cent of the country’s land area is classified as iTaukei (indigenous Fijian) communal land, administered through the iTaukei Land Trust Board (ILTB), with a further allocation between state and freehold categories. Within the GSUA, where much land was historically alienated during the colonial period, land access for informal settlers typically involves informal arrangements on state land or negotiated use of peri-urban iTaukei customary land, arrangements that Kiddle (2010) documented as providing a form of de facto security that influences housing investment behavior even in the absence of formal tenure.

Two informal settlement sites in the eastern peri-urban fringe of greater Suva were included in the building footprint study conducted in collaboration with the Strengthening Resilient and Sustainable Urban and Water Service Delivery in the Pacific project of the Water and Urban Development Sector of the ADB: Tacirua and Wailekutu. Both sites are in the Nasinu Town corridor, an area that the Greater Suva Urban Profile identified as experiencing some of the highest population and settlement growth rates in the GSUA (UN-Habitat, 2012). Kiddle and Hay (2017) highlighted the Suva peri-urban corridor as presenting both significant infrastructure challenges and, given Fiji’s comparatively well-developed governance capacity relative to other Pacific Island nations, genuine opportunities for planned settlement upgrading.

This section presents the datasets used, key processing steps, and derived analytical products for both Fiji study sites. All building count figures are preliminary estimates based on deep learning extraction and partial manual quality assurance. These results are submitted as proof-of-concept estimates to the ADB project. Table 2 summarizes the two Fiji study sites (Tacirua and Wailekutu), their boundary areas, survey epochs, and imagery sources.

Table 2. Fiji study site summary. Settlement areas derived from ADB-provided boundary shapefiles

| Site | Settlement Area (ha) | Epoch 1 | Epoch 2 | Epoch 3 | Imagery |
| --- | --- | --- | --- | --- | --- |
| Tacirua | 239.55 | ~2009 | ~2014 | 2025 | QB-2 / WV-2 / WV-3 |
| Wailekutu | 84.16 | ~2009 | ~2014 | 2025 | QB-2 / WV-2 / WV-3 |

Tacirua is a substantial peri-urban settlement in the Nasinu Town area on the eastern outskirts of the Greater Suva Urban Area. The settlement’s boundary area of 239.55 hectares is by far the largest of any of the six ADB project sites, reflecting its character as an expansive peri-urban corridor community rather than a compact inner-city settlement. The boundary encompasses a mix of hilly terrain, valley floors, and gentle slopes that transition from established residential development closer to the Suva–Nausori Highway corridor into more sparsely occupied hillslope areas. The Greater Suva Urban Profile specifically identified Tacirua East as one of the areas experiencing significant recent housing development and infrastructure investment pressure in the GSUA, including active upgrading of water supply connections (UN-Habitat, 2012). The settlement is predominantly accessed via the Suva–Nausori corridor, and its location provides relatively good access to both Suva City and the Nausori Town employment centers compared to more isolated peri-urban communities.

Building typologies at Tacirua reflect Fiji’s more developed construction context relative to other study countries in this project. Median building footprint areas at Tacirua across all three epochs (119–123 m²) are substantially larger than those recorded at the Port Moresby and Honiara sites, consistent with the generally larger building stock observed in Fijian informal settlements compared to their Melanesian counterparts. This may partly reflect the more regulated building environment in Fiji and the prevalence of concrete block or timber-frame construction with sheet metal roofing that produces relatively large, well-defined structural footprints detectable by the deep learning model.

Wailekutu is a smaller peri-urban settlement also in the Greater Suva eastern corridor, occupying a boundary area of 84.16 hectares. Compared to Tacirua, Wailekutu has a substantially smaller building stock across all survey epochs, with 45 structures in 2009 rising to 80 in 2025, and corresponding site-wide densities well below those of any other project site. This settlement occupies a more peripheral location, characterized by relatively dispersed residential structures on hillslope terrain with limited formal road access to the main corridor. The low overall building density and small absolute counts suggest that Wailekutu represents an early-stage or slowly developing informal settlement at the peri-urban fringe, where land availability may not be the primary constraint on growth so much as accessibility, services, and the viability of daily commuting to employment centers.

Informal land access in the eastern Suva peri-urban area typically involves a combination of state land occupancy and negotiated use of peri-urban iTaukei land through vakavanua arrangements, involving informal agreements between settlers and customary landowners that provide a degree of practical security but limited legal protection (Kiddle, 2010). The tenure uncertainty associated with such arrangements can discourage durable building investment and may contribute to the relatively modest building stock observed at Wailekutu, where rapid infill is unlikely without clearer pathways to tenure security.

The study utilized very-high-resolution commercial satellite imagery from the Maxar archive, specifically QuickBird-2, WorldView-2, and WorldView-3 sensors, to analyze building footprints and settlement changes across three epochs (2009, 2014, and 2025) in the Tacirua and Wailekutu sites. Key processing steps included orthorectification of imagery and application of a deep learning model to detect and quantify structural footprints, enabling comparative analysis of peri-urban development over time.

Results are presented for both Fiji study sites across the three survey epochs. All figures are preliminary estimates subject to revision. Growth metrics use compound annual growth rates (CAGR), defined as:

CAGR = (End Count ÷ Start Count)^(1 ÷ n years) − 1

Results are benchmarked against the ADB Melanesian urban expansion norm of 3–4 per cent per year for informal settlement growth (ADB, 2012) and the Pacific density norm of 15–25 structures per hectare.

The building counts increased at both Fiji study sites across the full study period, but with markedly different trajectories between the two survey intervals. Table 3 presents the full building count record.

Table 3. Building count summary for Fiji study sites across three survey epochs.

| Site | Year | Building Count | Δ from prev. epoch | Boundary Area (ha) |
| --- | --- | --- | --- | --- |
| Tacirua | 2009 | 496 | — | 239.55 |
| Tacirua | 2014 | 602 | +106 | 239.55 |
| Tacirua | 2025 | 838 | +236 | 239.55 |
| Wailekutu | 2009 | 45 | — | 84.16 |
| Wailekutu | 2014 | 54 | +9 | 84.16 |
| Wailekutu | 2025 | 80 | +26 | 84.16 |

Figure 11 presents the building count trajectories for both sites. At Tacirua, building counts increased from 496 in 2009 to 602 in 2014 and 838 in 2025, with additions of 106 structures in the first interval and 236 in the second. At Wailekutu, the trajectory shows modest but consistent growth throughout: nine structures added in 2009–2014 and a further 26 in 2014–2025. While the absolute counts at Wailekutu are far smaller than at Tacirua, the proportional growth rates across the two periods are broadly similar to those expected for a maturing peri-urban settlement.

Figure 11. Building  count growth for both Fiji study sites across the three survey epochs. Tacirua and Wailekutu both experienced steady structural growth over the years, with notable increases in each period.

Absolute Change in Building Count by Period

Figure 12 presents the absolute change in building count between consecutive survey epochs. The 2009–2014 interval added 106 structures at Tacirua and nine at Wailekutu. The 2014–2025 interval added 236 at Tacirua and 26 at Wailekutu. Growth at Tacirua is distributed across both inter-survey periods, with the larger absolute addition in the 2014–2025 interval reflecting the longer elapsed time (eleven versus five years). The continued expansion of Tacirua across both periods is consistent with the sustained in-migration and informal housing investment characteristic of the eastern Suva corridor.

Figure 12. Absolute change in building count by survey period for both Fiji study sites Tacirua demonstrated ongoing growth in building counts during both survey intervals, while Wailekutu exhibited steady progress throughout the same periods.

Compound Annual Growth Rates

Table 4 presents the CAGR for each survey interval at both Fiji sites, benchmarked against the ADB Melanesian norm of 3–4 per cent per year.

Table 4. Compound annual growth rate (CAGR) by period for Fiji study sites. ADB Melanesian benchmark: 3–4%/yr.

| Site | Period | Start Count | End Count | Years | CAGR (%/yr) | vs. ADB Benchmark |
| --- | --- | --- | --- | --- | --- | --- |
| Tacirua | 2009–2014 | 496 | 602 | 5 | 3.95% | Within (3–4%/yr) |
| Tacirua | 2014–2025 | 602 | 838 | 11 | 3.05% | Within (3–4%/yr) |
| Wailekutu | 2009–2014 | 45 | 54 | 5 | 3.71% | Within (3–4%/yr) |
| Wailekutu | 2014–2025 | 54 | 80 | 11 | 3.64% | Within (3–4%/yr) |

The Fiji CAGR results present an analytically distinctive pattern. Wailekutu shows consistent growth within the ADB benchmark across both periods — 3.71% per year in 2009–2014 and 3.64% per year in 2014–2025. This is the only site across all six ADB project locations to exhibit within-benchmark growth in both survey intervals, suggesting a comparatively stable and gradual settlement expansion dynamic. The two-period consistency at Wailekutu provides a degree of methodological confidence that the CAGR reflects genuine settlement growth rather than artefacts from changing sensor resolution.

Tacirua shows moderate growth across both intervals. The 2009–2014 CAGR of 3.95% per year falls within the ADB benchmark, and the 2014–2025 CAGR of 3.05% per year similarly sits within the 3–4% range. Both periods show consistent, benchmark-aligned expansion consistent with the broader peri-urban growth pattern of the eastern Suva corridor. Given that QuickBird-2 (0.61 m) was used for 2009 and WorldView-2 (0.46 m) for 2014, some degree of sensor-related detection variation between those two epochs cannot be ruled out, though this does not materially affect the overall interpretation.

All CAGR figures should be treated as indicative estimates, particularly for the 2009–2014 interval where sensor resolution differences between epochs are greatest. The 2014–2025 interval, with both epochs using Maxar sensors within the model’s optimal range, provides the most methodologically reliable CAGR comparison for both sites.

Figure 13. Compound annual growth rates for both Fiji study sites by survey period. Blue shaded band = ADB Melanesian benchmark (3–4%/yr). Both Tacirua and Wailekutu show within-benchmark growth across both survey intervals.

Settlement Density

Settlement density at both Fiji sites is substantially lower than at the PNG and Solomon Islands sites and remains well below the Pacific informal settlement norm of 15–25 structures per hectare across all survey epochs. Table 5 and Figure 14 present density values for both sites.

Table 5. Settlement density by epoch for Fiji study sites. Pacific norm: 15–25 str/ha.

| Site | Year | Building Count | Area (ha) | Density (str/ha) | vs. Pacific Norm (15–25/ha) |
| --- | --- | --- | --- | --- | --- |
| Tacirua | 2009 | 496 | 239.55 | 2.07 | Below norm |
| Tacirua | 2014 | 602 | 239.55 | 2.51 | Below norm |
| Tacirua | 2025 | 838 | 239.55 | 3.50 | Below norm |
| Wailekutu | 2009 | 45 | 84.16 | 0.53 | Below norm |
| Wailekutu | 2014 | 54 | 84.16 | 0.64 | Below norm |
| Wailekutu | 2025 | 80 | 84.16 | 0.95 | Below norm |

Tacirua reached a density of 3.50 str/ha by 2025, which, while representing a 69 per cent increase from 2.07 str/ha in 2009, is still well below the Pacific norm lower bound of 15 str/ha. Wailekutu at 0.95 str/ha in 2025 is extremely sparse by any urban standard. Two factors help contextualize these low density figures. First, both boundary areas are large relative to the occupied settlement footprint: Tacirua’s 239.55 ha boundary is the largest of any project site, and much of this area likely comprises undeveloped hillslope terrain within the delineated boundary. Density figures computed over the full boundary area therefore substantially understate the density of the actively occupied zones. Second, the large individual building footprints at both sites (median 95–120 m²) mean that even relatively modest structure counts translate into meaningful coverage of the occupied areas.

The low site-wide density figures should not be interpreted as indicating that infrastructure investment is not urgently needed. Connell (2011) notes that Pacific urban growth frequently concentrates in poorly serviced peri-urban corridors precisely because the formal urban core lacks the capacity to absorb in-migrants, and that the physical expansion of such corridors tends to outrun the provision of basic services regardless of site-wide density averages.

Figure 14. Settlement density (structures per hectare) for both Fiji study sites across all survey epochs. Yellow band = ADB Pacific norm (15–25 str/ha). Both sites remain well below the norm across all three epochs.

Built-up Coverage Ratio

Built-up coverage provides a measure of urban intensity that integrates building count with individual footprint size. Table 6 and Figure 15 present coverage trajectories for both Fiji sites.

Table 6. Built-up coverage ratio and total building footprint area by epoch for Fiji study sites.

| Site | Year | Building Count | Total Building Area (m²) | Coverage (%) |
| --- | --- | --- | --- | --- |
| Tacirua | 2009 | 496 | 65,637 | 2.74% |
| Tacirua | 2014 | 602 | 79,052 | 3.30% |
| Tacirua | 2025 | 838 | 116,900 | 4.88% |
| Wailekutu | 2009 | 45 | 4,545 | 0.54% |
| Wailekutu | 2014 | 54 | 6,985 | 0.83% |
| Wailekutu | 2025 | 80 | 9,089 | 1.08% |

Coverage trajectories at both sites show monotonically increasing patterns, consistent with continued settlement growth across the study period. At Tacirua, coverage increased from 2.74% in 2009 to 3.30% in 2014 and then to 4.88% in 2025. The 2009–2014 increase of 0.56 percentage points reflects the 106 structures added in that interval; the subsequent rise to 4.88% in 2025 reflects the further 236 structures added in the 2014–2025 period. The large boundary area of 239.55 ha means that even substantial absolute building count additions translate into modest site-wide coverage changes.

At Wailekutu, coverage increased consistently from 0.54% to 0.83% to 1.08% across the three epochs. The low absolute coverage values are consistent with the sparse, dispersed settlement character described in Section 2.2. Total building footprint area at Wailekutu grew from 4,545 m² to 9,089 m² over the study period, a doubling of total covered area, though these areas remain very small in absolute terms relative to the 84.16 ha boundary. A key observation for both sites is that the large median building footprint size (95–123 m²) means that even individual new structures contribute meaningfully to total covered area, so coverage metrics at these sites are relatively sensitive to the addition of even a small number of buildings.

Figure 15. Built-up coverage ratio (%) for both Fiji study sites by survey epoch. Both sites show monotonically increasing trajectories across all three epochs, consistent with sustained settlement growth throughout the study period.

Median Building Size

Median building footprint area at the Fiji sites is notably high, and provides insight into the distinct building typology of the Greater Suva peri-urban informal settlement environment. Table 7 and Figure 16 present the building size statistics.

Table 7. Building footprint size statistics by epoch for Fiji study sites.

| Site | Year | Median Building (m²) | Mean Building (m²) | Minimum (m²) | Maximum (m²) |
| --- | --- | --- | --- | --- | --- |
| Tacirua | 2009 | 119.54 | 132.12 | 26.90 | 558.82 |
| Tacirua | 2014 | 115.10 | 138.11 | 0.00 | 693.29 |
| Tacirua | 2025 | 119.55 | 139.53 | 0.66 | 598.44 |
| Wailekutu | 2009 | 95.49 | 101.66 | 35.81 | 255.13 |
| Wailekutu | 2014 | 120.00 | 129.54 | 0.27 | 321.89 |
| Wailekutu | 2025 | 103.77 | 113.58 | 0.03 | 383.02 |

At Tacirua, the median building size shows modest variation across the three epochs: 119.54 m² in 2009, 115.10 m² in 2014, and 119.55 m² in 2025, reflecting a broadly consistent building typology across the study period. The mean footprint area, by contrast, shows a gradual increase from 132.12 m² to 139.53 m², indicating that while the typical (median) structure has not grown in size, the distribution has become slightly right-skewed over time, with more large structures contributing to the rising mean. The presence of a zero minimum footprint value in 2014 likely reflects a very small post-processing artefact, a near-zero area polygon not fully eliminated by the 12 m² area filter and does not affect the median or mean statistics materially.

At Wailekutu, the median building size shows more variation: 95.49 m² in 2009, 120.00 m² in 2014, and 103.77 m² in 2025. Given the very small sample sizes involved (45, 54, and 80 structures respectively) these fluctuations should be interpreted cautiously; the apparent increase in 2014 and partial reversion in 2025 may partly reflect sampling variation rather than a genuine change in building typology. Nevertheless, the consistently higher values at both Fiji sites relative to the PNG and Solomon Islands sites (where medians typically ranged from 45 to 103 m²) are a genuine reflection of Fiji’s somewhat different informal settlement construction context, where buildings tend to be larger and use more durable materials than their counterparts in Port Moresby or Honiara.

Figure 16. Median building footprint area over time for both Fiji study sites. Bubble size is proportional to total building count.

The results for the two Fiji study sites present a picture that is analytically distinct from the other project countries, characterized by lower absolute densities, larger individual building footprints, and consistent within-benchmark growth rates. Both Tacirua and Wailekutu show growth trajectories that fall within the ADB Melanesian norm across both survey intervals, in contrast to the above-benchmark early-period growth observed at all four PNG and Solomon Islands sites.

The most notable finding from a planning perspective is the continued and consistent expansion at Tacirua across both survey periods, with 106 structures added in 2009–2014 (CAGR 3.95%/yr) and 236 in 2014–2025 (CAGR 3.05%/yr). This sustained growth reflects the ongoing attraction of the eastern Suva corridor to in-migrants. The ADB and the Fiji government have made significant investments in water supply and infrastructure in this corridor; if these investments have improved living conditions and accessibility, they may have reinforced the area’s appeal as a destination for informal settlement , a well-documented dynamic in informal settlement upgrading interventions noted by Kiddle and Hay (2017) for the broader Suva peri-urban context.

The consistent within-benchmark growth at Wailekutu is the only instance across the six project sites where a settlement-maintained growth rates within the ADB norm across two consecutive intervals. This may reflect the site’s peripheral character and limited accessibility: growth appears constrained not by physical saturation, with only 80 structures on 84 ha, the site is far from capacity, but by the deterrent effects of poor connectivity, uncertain tenure, and limited services. If ADB and government infrastructure investments reach this corridor, growth rates could increase substantially. The planning implication is that Wailekutu, despite its current low density, represents a site of latent settlement expansion pressure that warrants monitoring.

The large boundary area of Tacirua (approximately 240 ha, the largest of any project site) means that site-wide density and coverage metrics persistently understate the intensity of occupation within the actively built-up zones. Infrastructure planning that relies on site-wide averages will underestimate the per-capita demand for water, sanitation, and road access in the densest parts of the settlement. A sub-boundary zonal analysis would provide more operationally useful density estimates, though such analysis is beyond the scope of this preliminary study.

Seismicity

The Fiji Islands lie within one of the most complex and seismically active regions of the southwest Pacific, situated between the converging Pacific and Australian Plates (Hamburger and others, 1988; Rahiman, 2009). Unlike intraplate islands such as Nauru, Fiji occupies a broad zone of active tectonism characterized by back‑arc spreading, transform faulting, and subduction processes associated with the Tonga–Kermadec and Vanuatu trenches. As a result, the region experiences frequent moderate‑to‑strong earthquakes, many occurring at intermediate depths within the subducting Pacific Plate. Although most events do not produce severe damage, felt earthquakes are common across the islands, and strong shaking is possible from both local and regional sources. Deep‑focus earthquakes—some exceeding magnitude 6 (Richter Scale)—are also characteristic of the region and, while typically less damaging, may be widely felt due to their depth and broad wave propagation. Because of its tectonic setting, the Fiji area maintains a persistent level of seismic hazard, and the population regularly experiences perceptible ground shaking.

Discussion

Understanding water availability in Fiji requires considering the balance between precipitation inputs, evapotranspiration losses, and the resulting runoff and recharge that sustain local water supply systems. Fiji’s humid tropical climate typically delivers substantial annual rainfall but pronounced seasonal and interannual variability affects how much of this water is converted to usable surface and groundwater resources. Actual evapotranspiration remains high year-round due to warm temperatures and dense vegetation, creating periods—especially during drier months—when ET demand approaches or exceeds available moisture, thereby reducing effective runoff and streamflow. These climate-driven fluctuations shape the hydrologic baseline against which settlement growth, water infrastructure performance, and resource pressures must be interpreted, underscoring the need for integrated assessments that link climatic drivers to local water demand and supply conditions.

The integration of remote sensing techniques for water assessment in Fiji provides valuable insights into the dynamic interplay between settlement expansion and resource availability. High-resolution imagery and geospatial analysis have enabled the accurate delineation of settlement boundaries and the monitoring of changes in building footprints over time, which directly inform estimates of population growth and density. The temporal mapping of settlement growth at Tacirua and Wailekutu underscores significant shifts in population distribution, with Tacirua experiencing a marked acceleration in informal settlement expansion between 2014 and 2025, linked to improved infrastructure and water supply investments. These findings highlight the importance of regularly updating water resource assessments to reflect rapidly changing settlement patterns, as increased population density can exert substantial pressure on local water infrastructure and accessibility.

The stable median building size observed at Tacirua, despite rising mean footprint area, suggests an evolving distribution where new, larger structures contribute to overall growth, but the typical dwelling remains unchanged. In contrast, Wailekutu’s more variable median building size and consistently within-benchmark growth rates point to latent expansion potential constrained by limited services and connectivity. Remote sensing data reveal that both sites possess relatively large individual building footprints compared to other project countries, a factor that may influence per-capita water demand and complicate infrastructure planning. Site-wide averages, as derived from remote sensing, risk underestimating the true intensity of occupation and resource needs in the densest zones, emphasizing the need for sub-boundary or zonal analyses to guide effective water management strategies.

Overall, the use of settlement mapping through remote sensing not only improves the accuracy of population estimates but also enhances the capacity for targeted water resource planning. As informal settlements continue to expand and densify, particularly in response to infrastructure upgrades, adaptive water management approaches that integrate up-to-date geospatial data will be essential to meet the evolving needs of these communities and mitigate the risks of under-provision or overuse in high-growth corridors.

Conclusion

Effective water management in the Fiji Islands and surrounding Pacific regions requires a nuanced understanding of both environmental and social dynamics. While the prevailing monsoon climate generally supports a positive water balance and replenishes vital groundwater resources, episodic droughts and uneven precipitation patterns present ongoing challenges. The rapid expansion and densification of informal settlements, as observed at Tacirua and Wailekutu, further intensify pressure on water infrastructure and highlight the risk of under-provision in the most densely populated areas. Remote sensing and geospatial analysis have proven essential for monitoring settlement growth and informing adaptive resource planning, but site-wide averages may obscure the true demand in high-occupancy zones. To ensure resilient access to potable water, integrated strategies must combine regular updates of population and density estimates, improved infrastructure investments, and flexible management approaches that anticipate both seasonal abundance and temporary scarcity. By prioritizing targeted analyses and infrastructure upgrades, communities can better safeguard water security against the dual pressures of climatic variability and ongoing settlement expansion.

References Cited

ADB, 1965, Agreement establishing the Asian Development Bank signed at Manila, Dec. 4, 1965; entered into force Aug. 22, 1966, United Nations Treaty Series, vol. 608, p. 380. Available at: Agreement Establishing the Asian Development Bank (ADB Charter). Accessed 29 January 2026.

Asian Development Bank, 2012, The State of Pacific Towns and Cities: Urbanization in ADB’s Pacific Developing Member Countries. Pacific Studies Series. Manila: ADB. Available at https://www.adb.org/publications/state-pacific-towns-and-cities-urbanization-adbs-pacific-developing-member-countries

Connell, J. (2011). Elephants in the Pacific? Pacific urbanisation and its discontents. Asia Pacific Viewpoint, 52(2), 121–135. https://doi.org/10.1111/j.1467-8373.2011.01445.x

DeJonge, K.C., Allen, R.G., Kilic, A., Thorp, K.R., Kukal, M., Marek, G., Altenhofen, J., Amatya, D.M., Blankenau, P., Datta, S., Grabow, G., Hashem, A., Kisekka, I., Kjaersgaard, J., Marek, T., Peters, T., Porter, D., Reba, M., Rudnick, D., Senay, G., Sharma, V., Sridhar, V., Sun, G., Taghvaeian, S., Trezza, R., and Trout, T., 2025, Evapotranspiration terminology and definitions: U.S. Geological Survey Publication, accessed April 11, 2026, at https://www.usgs.gov/publications/evapotranspiration-terminology-and-definitions.

Fiji Bureau of Statistics. 2017. 2017 Population and Housing Census: Major Urban Areas — Suva. In Population and Housing Census 2017 Dashboard. Suva, Fiji. Retrieved April 2026, https://www.statsfiji.gov.fj/

Hamburger, M. W., Everingham, I. B., Isacks, B. L., & Barazangi, M. 1988. Active tectonism within the Fiji platform, southwest Pacific. Geology, 16(3), 237–241. https://doi.org/10.1130/0091-7613(1988)016<0237:ATWTFP>2.3.CO;2

Huffman, G.J., E. F. Stocker, D. T. Bolvin, E. J. Nelkin, and J. Tan, 2019, GPM IMERG Final Precipitation L3 1 Day 0.1°×0.1° V07 (GPM_3IMERGDF), Goddard Earth Sciences Data and Information Services Center (GES DISC), accessed April 10, 2026, https://doi.org/10.5067/GPM/IMERGDF/DAY/07

He, K., Gkioxari, G., Dollár, P., & Girshick, R. (2017). Mask R-CNN. Proceedings of the IEEE International Conference on Computer Vision (ICCV), 2017, 2961–2969. https://doi.org/10.1109/ICCV.2017.322

Jones, P. (2016). The Emergence of Pacific Urban Villages: Urbanization Trends in the Pacific Islands. Pacific Studies Series. Manila: Asian Development Bank.

Kiddle, G.L. (2010). Perceived security of tenure and housing consolidation in informal settlements: Case studies from urban Fiji. Pacific Economic Bulletin, 25(3), 193–214.

Kiddle, G.L., & Hay, I. (2017). Informal settlement upgrading: Lessons from Suva and Honiara. Development Bulletin, 78, 25–29.

Kostaschuk, R., Terry, J., & Raj, R. 2001. Tropical cyclones and floods in Fiji. Hydrological Sciences Journal, 46(3), 435–450. https://doi.org/10.1080/02626660109492837

Lu, D., Mausel, P., Brondízio, E., & Moran, E. (2004). Change detection techniques. International Journal of Remote Sensing, 25(12), 2365–2401. https://doi.org/10.1080/0143116031000139863

Phillips, T., & Keen, M. (2016). Sharing the City: Urban Growth and Governance in Suva, Fiji. SSGM Discussion Paper 2016/6. Canberra: Australian National University. Available at https://openresearch-repository.anu.edu.au

Rahiman, T.I.H., and Pettinga, J.R., 2009, Fracture lineaments, fault mesh formation and seismicity—Towards a seismotectonic model for Viti Levu, Fiji: Bulletin of the New Zealand Society for Earthquake Engineering, v. 42, no. 1, p. 63–72.

Scheffler, D., Hollstein, A., Diedrich, H., Segl, K., & Hostert, P. (2017). AROSICS: An automated and robust open-source image co-registration software for multi-sensor satellite data. Remote Sensing, 9(7), 676. https://doi.org/10.3390/rs9070676

Senay, G.B., Kagone, S., and Velpuri, N.M., 2020, Operational global actual evapotranspiration using the SSEBop model—USGS data release: U.S. Geological Survey Data Release, DOI:10.5066/P9OUVUUI.

Tribble, G.W., 2008, Ground water on tropical Pacific Islands—understanding a vital resource: U.S. Geological Survey Circular 1312, 35 p.

UN-Habitat (2012). Fiji: Greater Suva Urban Profile. Nairobi: United Nations Human Settlements Programme. Available at https://unhabitat.org/sites/default/files/documents/2019-06/fiji_greater_suva_urban_profile.pdf

USGS Data release

U.S. Geological Survey. 2026. Geography of Fiji: Volcanic Archipelago, Islands, and South Pacific Climate. Retrieved April 2026, from USGS publications server.

Weir, T., Stephens, M., and Kumar, R., 2014, Rainfall trends in Fiji: International Journal of Climatology, v. 34, no. 5, p. 1501–1510, doi:10.1002/joc.3779

White, I., & Falkland, T. 2010. Management of freshwater lenses on small Pacific islands. Hydrogeology Journal, 18(1), 227–246. https://doi.org/10.1007/s10040-009-0525-0