National Geospatial Program

Prepared in cooperation with the Asian Development Bank

Increasing Water Security on Pacific Islands Through Increased Data Availability and Understanding—Volume 5, Papua New Guinea

By Gabriel B. Senay,1 Francis K. Dwomoh,2 Stefanie Kagone,2 Tomas Smieszek1, and Matthew E. Andersen1

1U.S. Geological Survey.

2ASRC Federal, contractor to the U.S. Geological Survey (USGS) Earth Resources Observations and Science (EROS) Center, Sioux Falls, SD 57198, USA. Work performed under USGS Contract [140G0124D0001].

Any use of trade, firm, or product names is for descriptive purposes only and does not imply endorsement by the U.S. Government.

Although this information product, for the most part, is in the public domain, it also may contain copyrighted materials as noted in the text. Permission to reproduce copyrighted items must be secured from the copyright owner.

This draft manuscript is distributed solely for purposes of courtesy review and comments received will be addressed and treated as appropriate to ensure there is no conflict of interest. Its content is deliberative and predecisional, so it must not be disclosed or released by reviewers. Because the manuscript has not yet been approved for publication by the U.S. Geological Survey (USGS), it does not represent any official USGS finding or policy.

Acknowledgments

This work was conducted in collaboration with the Strengthening Resilient and Sustainable Urban and Water Service Delivery in the Pacific project of the Water and Urban Development Sector of the Asian Development Bank (ADB). ADB provided financial support from some USGS technical salaries and associated administrative costs, including some of the travel necessary to administer and manage the work.

Outline

Acknowledgments

Introduction

Purpose and Scope

Papua New Guinea

City of Port Moresby

Precipitation

Evapotranspiration

Water Balance

Informal Settlement Mapping

Seismicity

Discussion

Conclusion

References Cited

Figures

Figure 1. Map of Western Pacific Ocean Island Nations. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Western Pacific Ocean island nations. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

Figure 2. Map of Papua New Guinea. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing Papua New Guinea. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

Figure 3. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

Figure 4. Monthly precipitation for the Port Moresby for years 1998 - 2025 in mm.

Figure 5. Annual precipitation for years 1998 - 2024 in mm.

Figure 6. Mean monthly precipitation and actual ET in mm.

Figure 7. Annual actual ET for years 2013 - 2025 in mm.

Figure 8. Monthly net water availability for years 2012 - 2025 in mm.

Figure 9. Net water availability per year in mm.

Figure 10. Building count growth for both Papua New Guinea study sites across the three survey epochs. Values annotated at each data point.

Figure 11. Absolute change in building count by survey period across all study sites. The 2016–2025 period produced the largest single-site addition in the entire study at Taurama Valley (+1,902 structures).

Figure 12. Compound annual growth rates for both Papua New Guinea study sites by survey period. Blue shaded band = ADB Melanesian benchmark (3–4%/yr). Red = exceeds; amber = below; green = within benchmark.

Figure 13. Settlement density (structures per hectare) for both Papua New Guinea study sites across all survey epochs. Yellow band = ADB Pacific norm (15–25 str/ha).

Figure 14. Built-up coverage ratio (%) for both Papua New Guinea study sites by survey epoch. Coverage reflects the proportion of the settlement boundary area occupied by building footprints.

Figure 15. Median building footprint area over time for both Papua New Guinea study sites. Bubble size is proportional to total building count.

Tables

U.S. customary units to International System of Units

International System of Units to U.S. customary units

Table 1. Mean IMERG Precipitation Statistics (1998–2024) for Port Moresby Basin

Table 2. Papua New Guinea study site summary. Settlement areas are derived from ADB-provided boundary shapefiles

Table 3. Building count summary for Papua New Guinea study sites across three survey epochs

Table 4. Compound annual growth rate (CAGR) by period for Papua New Guinea study sites. ADB Melanesian benchmark: 3–4%/yr

Table 5. Settlement density by epoch for Papua New Guinea study sites

Table 6. Built-up coverage ratio by epoch for Papua New Guinea study sites

Table 7. Building footprint size statistics by epoch for Papua New Guinea study sites

Conversion Factors

U.S. customary units to International System of Units

| Multiply | By | To obtain |
| --- | --- | --- |
| Length |  |  |
| inch (in.) | 2.54 | centimeter (cm) |
| inch (in.) | 25.4 | millimeter (mm) |
| foot (ft) | 0.3048 | meter (m) |
| mile (mi) | 1.609 | kilometer (km) |
| mile, nautical (nmi) | 1.852 | kilometer (km) |
| yard (yd) | 0.9144 | meter (m) |
| Area |  |  |
| acre | 4,047 | square meter (m2) |
| acre | 0.4047 | hectare (ha) |
| acre | 0.4047 | square hectometer (hm2) |
| acre | 0.004047 | square kilometer (km2) |
| square foot (ft2) | 929.0 | square centimeter (cm2) |
| square foot (ft2) | 0.09290 | square meter (m2) |
| square inch (in2) | 6.452 | square centimeter (cm2) |
| section (640 acres or 1 square mile) | 259.0 | square hectometer (hm2) |
| square mile (mi2) | 259.0 | hectare (ha) |
| square mile (mi2) | 2.590 | square kilometer (km2) |
| Volume |  |  |
| barrel (bbl; petroleum, 1 barrel=42 gal) | 0.1590 | cubic meter (m3) |
| ounce, fluid (fl. oz) | 0.02957 | liter (L) |

International System of Units to U.S. customary units

| Multiply | By | To obtain |
| --- | --- | --- |
| Length |  |  |
| millimeter (mm) | 0.03937 | inch (in.) |
| meter (m) | 3.281 | foot (ft) |
| kilometer (km) | 0.6214 | mile (mi) |
| Area |  |  |
| square meter (m2) | 0.0002471 | acre |
| hectare (ha) | 2.471 | acre |
| square hectometer (hm2) | 2.471 | acre |
| square kilometer (km2) | 247.1 | acre |
| square centimeter (cm2) | 0.001076 | square foot (ft2) |
| square meter (m2) | 10.76 | square foot (ft2) |
| square centimeter (cm2) | 0.1550 | square inch (in2) |
| square hectometer (hm2) | 0.003861 | section (640 acres or 1 square mile) |
| hectare (ha) | 0.003861 | square mile (mi2) |
| square kilometer (km2) | 0.3861 | square mile (mi2) |

Abbreviations

ADB	Asian Development Bank

FSM	Federated States of Micronesia

USGS	U.S. Geological Survey

ET	Evapotranspiration

P	Precipitation

ENSO	El Niño–Southern Oscillation

Introduction

Freshwater is critically important for sustaining life, supporting communities, and enabling economic activities across Papua New Guinea. In and around Port Moresby—PNG’s rapidly growing capital city—reliable hydrologic information is increasingly essential for utilities, resource managers, and the public. Such information supports planning for rising water demand, reducing the impacts of prolonged dry seasons and El Niño–driven droughts, and strengthening long-term water security. The Asian Development Bank (ADB), whose mission includes supporting sustainable development and climate resilience across Asia and the Pacific, recognizes that dependable access to safe water underpins public health, environmental stability, and economic opportunity in PNG.

Improved understanding of the timing, magnitude, and variability of freshwater availability enables water managers in the Port Moresby region to make informed decisions about resource allocation, investments in storage and conveyance infrastructure, and preparation for periods of water scarcity. Potential development actions—such as enhancing the reliability of the Rouna water supply system, expanding storage capacity, and protecting upstream catchments—depend on consistent, accessible hydrologic information. Satellite-based datasets offer an efficient and scalable way to meet these needs across PNG’s rugged terrain and often remote watersheds.

To help address these challenges, ADB and the U.S. Geological Survey (USGS) partnered to evaluate and demonstrate how Earth observation products can support waterresource management across Papua New Guinea, with a focus on the Port Moresby region. This collaboration analyzes long-term satellite datasets for precipitation, evapotranspiration (ET), seasonal rainfall patterns, drought monitoring, and indicators relevant for short and mediumrange weather forecasting. The methods used to develop these datasets are described in Volume 1 – Overview of this series, while this volume applies those methods to the hydrologic and climate conditions relevant to Port Moresby and its surrounding catchments.

Purpose and Scope

This report presents the results of a collaborative effort by the U.S. Geological Survey (USGS) and the Asian Development Bank (ADB) to assess the applicability of remotely sensed Earth-observation data for improving understanding of water-resource conditions across Papua New Guinea (PNG). The work supports regional efforts to address persistent water-availability challenges on Pacific islands, where rapidly growing urban populations, strong climatic variability, and limited surface-water storage can lead to extended periods of water scarcity.

This volume documents the analytical components of the study and provides results for the Port Moresby region, the nation’s administrative and economic center and one of its most water-stressed urban environments. The scope of the report includes satellite-based assessments of precipitation, evapotranspiration, seasonal rainfall variability, drought-monitoring indicators, and weather-forecasting resources. Methods used to generate these datasets are presented in Volume 1 of this series; the present volume focuses on applying those methods within Papua New Guinea and summarizing the resulting water-availability information.

Each chapter describes the datasets used, outlines key processing steps, and presents the products derived for the Port Moresby study area. The information provided is intended to assist water managers, utilities, and planners in evaluating hydrologic conditions, identifying periods of relative water surplus or shortage, and supporting decisions related to water conservation, infrastructure planning, and resource management.

Papua New Guinea

Beginning in 2024, the USGS entered a collaborative relationship with ADB to assess the utility of various remotely sensed datasets for improving water-resource management within Papua New Guinea (PNG), a geographically extensive nation occupying the eastern half of New Guinea and more than 600 offshore islands across the southwest Pacific (Figure 1).

Figure 1. Map of Western Pacific Ocean Island Nations. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing the Western Pacific Ocean island nations. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

The capital city of Port Moresby, located on the dry leeward coast of the island’s southern margin, represents one of PNG’s most water-stressed urban centers and serves as the primary focus of this volume (Figure 2).

Figure 2. Map of Papua New Guinea. The Geographic Coordinate System is WGS84. Sources: Esri, TomTom, FAO, NOAA, USGS.

Alt text: A map showing Papua New Guinea. The map uses the WGS84 geographic coordinate system and includes data sources from Esri, TomTom, FAO, NOAA, and USGS.

PNG encompasses a wide range of hydroclimatic and physiographic conditions, including high volcanic and fold-belt mountain ranges rising above 4,000 meters, densely forested catchments, extensive river systems, and low-lying coastal plains (Hall, 1984). While many regions receive some of the highest rainfall totals in the Pacific, Port Moresby lies within a pronounced regional rain shadow produced by the central highlands and prevailing southeast trade winds. As a result, the city experiences a semi-arid climate with strong seasonal rainfall variability, limited perennial surface-water sources, and growing pressure on groundwater and reservoir systems (McGregor, 1989). These conditions leave Port Moresby highly sensitive to changes in precipitation, evapotranspiration, El Niño–induced droughts, and the timing and intensity of wet-season recharge.

PNG also sits within one of the most active tectonic environments in the world, shaped by interactions among the Pacific, Indo-Australian, and several microplates. Subduction zones, active faults, and volcanic centers contribute to frequent earthquakes, uplift, landslides, and rapid geomorphic change. These geologic processes strongly influence watershed evolution, sediment delivery to reservoirs, and the long-term stability of water-supply infrastructure in and around Port Moresby. In consultation with PNG’s national and municipal water authorities, ADB prioritized the development of enhanced precipitation, evapotranspiration, water-balance, and drought-monitoring datasets for the Port Moresby region—reflecting the city’s acute hydrologic vulnerability, rapid population growth, and exposure to climatic and tectonic variability.

City of Port Moresby

Port Moresby, situated on the southeastern coast of the island of New Guinea, is the capital and largest urban center of Papua New Guinea, forming the National Capital District (Rooney, 2021). The city spans about 240 km² and lies on a coastal plain bounded by low hills and river valleys adjacent to Port Moresby Harbour on the Gulf of Papua. Elevation averages around 35 m above sea level. As of the 2024 census, Port Moresby’s population reached approximately 756,754, reflecting rapid urban growth over recent decades (National Statistical Office of Papua New Guinea, 2025). It serves as the administrative, political, and economic hub of PNG and experiences a tropical savanna (Aw) climate, characterized by a distinct wet season from December through May and a dry season from June to November. Average annual rainfall is around 899 mm, making it relatively dry for New Guinea, in part due to a rain shadow effect from inland highlands. Port Moresby's hydrology is shaped by its coastal setting and semi-arid climate. Surface-water supply primarily relies on the Laloki River system, supplemented by groundwater. The city is prone to localized flooding during intense wet-season storms, and water availability can be stressed during prolonged dry periods or ENSO-related droughts.

Precipitation

Precipitation is the primary source of freshwater across Pacific Island environments, because islands lack the buffering capacity of large watersheds or reservoirs (Tribble, 2008). Therefore, understanding spatial and temporal rainfall variability is essential for evaluating water availability, assessing drought risk, and guiding sustainable management of hydrologic resources. Satellite-derived precipitation datasets provide a consistent means of monitoring these patterns across remote and data-sparse island settings.

Rainfall patterns for the Port Moresby Basin were derived using the Integrated Multi-satellitE Retrievals for GPM (IMERG) precipitation dataset (Huffman et al., 2019). After the dataset was acquired, a basin-average daily precipitation time series was generated for the period 1 January 1998 through 31 September 2025 and is available through the associated USGS data release (USGS Data Release).

Port Moresby has a semi-arid to sub-humid tropical climate, strongly influenced by its location on the leeward side of the Owen Stanley Range (Figure 3).

Figure 3. Mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

Alt. text. Graphical representation of mean annual precipitation from 2015 - 2024 from IMERG Final Run dataset, displaying lower values in light blue and higher values dark blue.

This setting results in marked seasonal rainfall variability, with a pronounced dry season and limited perennial surface-water availability. Based on the IMERG analysis, the basin’s mean annual total precipitation is 2,145 mm/yr, with a mean monthly precipitation of 179 mm (Table 1). Minimum monthly precipitation averages 69 mm, reflecting the severity of dry-season conditions, while maximum monthly precipitation averages 310 mm, driven largely by monsoonal and convective rainfall events during the wet season.

Table 1. Mean IMERG Precipitation Statistics (1998–2024) for Port Moresby Basin

| Statistic | Precipitation (mm) |
| --- | --- |
| Mean annual total | 2145 |
| Mean monthly | 179 |
| Mean minimum monthly | 69 |
| Mean maximum monthly | 310 |

Port Moresby receives comparatively low to moderate precipitation, with monthly totals generally ranging from less than 70 mm during the dry season to more than 300 mm during peak wet-season months, reflecting the city’s semi-arid to sub-humid tropical climate. Rainfall is strongly seasonal, with the majority of precipitation occurring between December and April, when monsoonal flow and convective activity are most active (Figure 4). Several months exceed 300 mm, such as January 2009, March 2020, and March 2024, highlighting the potential for short but intense wet seasons driven by regional atmospheric conditions and occasional tropical disturbances. In contrast, dry-season months frequently record very low rainfall, with mean monthly minimums around 69 mm and some years experiencing extreme lows, such as August 1998, when Port Moresby received only a few millimeters of precipitation. These extended dry periods underscore the city’s persistent vulnerability to water scarcity despite intermittent heavy rainfall events.

Figure 4. Monthly precipitation for the Port Moresby for years 1998 - 2025 in mm.

Alt. text. Line graph showing the monthly precipitation for years 1998 - 2025 in mm.

Annual precipitation in the Port Moresby Basin exhibits substantial interannual variability, reflecting the city’s exposure to monsoonal shifts, localized convective activity, and strong sensitivity to ENSO-related climate oscillations. Unlike the persistently humid islands of the central Pacific, annual precipitation totals in Port Moresby vary widely around the long-term mean of approximately 2,145 mm/yr, with individual years alternating between markedly wetter and significantly drier conditions (Figure 5).

Several years exceed 2500 mm including in 2000 (2785 mm), 2011 (2570 mm) and 1998 (2535 mm) during periods influenced by strong regional climate anomalies or enhanced convective episodes. The driest year is below 1000 mm (1999 with 812 mm). The most recent years show little departures from the long-term average, no sustained multi-year trend in extreme wetness is evident, and interannual variability remains a defining characteristic of Port Moresby’s semi-arid climate.

Figure 5. Annual precipitation for years 1998 - 2024 in mm.

Alt. text. Bar chart showing annual precipitation for years 1998 to 2024 in mm.

From a water-resource perspective, this level of variability supports generally reliable surface-water availability, but management strategies should continue to account for occasional dry months, as well as increases in turbidity or short-duration runoff peaks during wetter periods.

Evapotranspiration

Because precipitation provides the dominant water input to Port Moresby’s hydrologic system, despite its relatively low and highly seasonal nature, understanding how much of this water is subsequently lost to the atmosphere is essential for evaluating freshwater availability. Evapotranspiration (ET) represents the combined atmospheric water loss from evaporation (soil, water, and surfaces) and transpiration by vegetation (DeJonge and others, 2025), and is driven by solar radiation, temperature, humidity, wind, and land-surface characteristics. Actual ET, as estimated using the SSEBop model (Senay and others, 2020), reflects the portion of potential ET realized under the prevailing surface and atmospheric conditions.

In the Port Moresby Basin, ET exhibits less variability throughout the year than precipitation, reflecting the region’s predominantly energy-driven evaporative environment (Figure 6). Rainfall is sharply seasonal, with wet-season peaks typically occurring from December through April, when monthly precipitation can exceed 300 mm (March ~ 310mm). The dry season brings a steep decline in rainfall, with several months commonly receiving less than 70 mm (July). In contrast, monthly actual ET varies within a narrower range, largely governed by changes in solar energy, humidity, and vegetation activity rather than by moisture limitation alone. ET generally reaches its lowest values during the most humid and cloudy wet-season months and increases during the drier, sunnier months when evaporative demand peaks.

Figure 6. Mean monthly precipitation and actual ET in mm.

Alt. text. Area chart showing the average monthly precipitation in mm with blue hatch lines and actual ET in mm in green (solid).

Annual ET totals also show relatively limited variability. Across the available record (2013–2025), actual ET fluctuates only modestly, with annual totals remaining within a tight range compared to the much larger interannual swings in precipitation (Figure 7). This stability reinforces the energy-driven nature of ET in the Port Moresby region, as well as the dominant role that seasonal and interannual rainfall variability plays in determining hydrologic surplus or deficit.

Figure 7. Annual actual ET for years 2013 - 2025 in mm.

Alt. text. Bar chart with annual actual ET for years 2013 - 2025 in mm.

The relative stability of ET, combined with Port Moresby’s highly variable and often limited rainfall, means that the basin frequently experiences hydrologic deficits, particularly during prolonged dry seasons or El Niño-related droughts. Water-resource planning for the region must therefore account for the strong seasonality of precipitation, sustained atmospheric evaporative demand, and the potential need for augmented storage, groundwater management, or supply-system resilience measures during extended dry periods.

Water Balance

The water balance, or net water availability, represents the surplus of water remaining after subtracting actual ET from precipitation (P) at monthly and annual scales. In the Port Moresby Basin, net water availability shows pronounced seasonality, driven by the strong contrast between the wet and dry seasons. During the wet months—typically December through April—precipitation commonly exceeds evaporative demand, producing positive net water values. These wet-season surpluses vary from modest to substantial depending on the intensity of monsoonal activity, with several months exceeding 100–250 mm of net water during particularly active rainfall periods. In contrast, negative net water values are frequent during the dry season, when ET surpasses precipitation for several consecutive months, reflecting the basin’s semi-arid climate and persistent atmospheric evaporative demand (Figure 8).

These patterns highlight Port Moresby’s strong hydrologic seasonality: wet-season months often produce meaningful surpluses that support limited recharge and temporary increases in streamflow, while dry-season months (May–November) frequently exhibit deficits, sometimes approaching or exceeding –50 to –100 mm, depending on rainfall suppression during El Niño events.

Figure 8. Monthly net water availability for years 2012 - 2025 in mm.

Alt. text. Bar chart showing monthly net water availability for years 2012 - 2025 in mm.

At the annual scale, Port Moresby’s net water availability varies widely in response to climate oscillations (Figure 9). Years with strong wet-season rainfall and active monsoonal conditions show positive annual net water, whereas El Niño-affected years often produce annual deficits or very small surpluses. Across the recent five-year period (2020–2024), annual net water shows large variability, reflecting the city’s sensitivity to interannual climate forcing rather than a consistent long-term trend.

Figure 9. Net water availability per year in mm.

Alt. text. Horizontal bar chart showing net water availability per year in mm for 2013 to 2024 starting with 2024 at the top.

Overall, the combination of highly variable rainfall and relatively stable ET creates a hydrologic system in which Port Moresby frequently experiences seasonal and sometimes annual water deficits. Wet-season surpluses are typically insufficient to offset prolonged dry-season atmospheric demand, making the basin particularly vulnerable to drought and requiring careful management of reservoirs, groundwater, and supplemental storage. Effective water-resource planning must therefore account for the likelihood of extended dry periods, reductions in wet-season rainfall during ENSO events, and the potential need for enhanced supply reliability and demand-management strategies.

Informal Settlement Mapping

Papua New Guinea (PNG) is one of the most rapidly urbanizing nations in the Pacific region. Port Moresby, the national capital, has experienced sustained and largely unplanned urban expansion driven by rural-to-urban migration, natural population increase, and inadequate formal housing supply. UN‑Habitat (2010) reported that around 45% of Port Moresby’s residents live in informal settlements. Koczberski et al. (2001) documented state-led responses to informal settlement growth, including periodic forced evictions, that have historically failed to stem the growth of squatter communities. Goddard (2005) traced the origins of Port Moresby's informal settlements to the post-World War II period, noting that village-like communities embedded within the urban fabric have persisted across successive waves of development policy. UN-Habitat (2010) reported that approximately 60% of Port Moresby land is state land and 40% customary land, with inadequate formal housing mechanisms to absorb demand from internal migrants.

Two informal settlement sites in Port Moresby were included in the Strengthening Resilient and Sustainable Urban and Water Service Delivery in the Pacific project of the Water and Urban Development Sector of the Asian Development Bank (ADB) building footprint study: 8 Mile Settlement and Taurama Valley. These sites were selected to represent contrasting landscape conditions. The 8 Mile Settlement is a dense, canopy-covered peri urban community with highly fragmented land use, while Taurama Valley is a more open peri-urban settlement in a valley corridor. The two sites are situated in distinct parts of the greater Port Moresby urban area, where 8 Mile Settlement in the northern outskirts along the Hubert Murray Highway corridor, and Taurama Valley in the southeastern outskirts. Both areas are subject to ongoing population pressure and infrastructure deficits characteristic of informal settlements in Melanesian capitals (ADB, 2012; Jones, 2016).

This section presents the datasets used, key processing steps, and derived analytical products for both PNG study sites. All building count figures are preliminary estimates based on deep learning extraction and partial manual quality assurance (QA). These results are submitted as proof-of-concept estimates to the ADB project. Further refinement was not possible within the project timeline.

Study Sites

Table 2 summarizes the two Papua New Guinea study sites, their boundary areas, survey epochs, and the imagery sources used for each epoch.

Table 2. Papua New Guinea study site summary. Settlement areas are derived from ADB-provided boundary shapefiles

| Site | Settlement Area (ha) | Epoch 1 | Epoch 2 | Epoch 3 | Imagery |
| --- | --- | --- | --- | --- | --- |
| 8 Mile Settlement | 156.15 | 2010 | 2016 | 2025 | WV-2 / WV-2 / WV-3 |
| Taurama Valley | 459.59 | 2010 | 2016 | 2025 | WV-2 / WV-2 / WV-3 |

8 Mile Settlement

8 Mile Settlement is a densely populated informal community located north of Port Moresby city center, along the Hubert Murray Highway corridor in the northern outskirts of the National Capital District. The settlement occupies a about 156 ha boundary area on relatively flat terrain that transitions into low undulating hills covered by dense tropical tree canopy. The community is characterized by a mix of corrugated iron and timber-frame structures, many of which are small (< 60 m²) and partially or wholly obscured by overhead vegetation. The settlement is served by limited formal road access and receives basic utility services on an irregular basis. It is one of the largest and most established informal settlements in Port Moresby, with a community history stretching back several decades (Goddard, 2005).

Detection of building footprints at 8 Mile Settlement was substantially impeded by dense tropical canopy cover, which prevents direct nadir viewing of rooftops across much of the site. After evaluating the Esri Africa, Australia, and USA pretrained Mask R-CNN models, the USA model at a confidence threshold of 0.20 produced the most complete detection results. Despite this, canopy occlusion means building counts at this site are systematic undercounts of the true building stock.

Taurama Valley

Taurama Valley is a peri-urban informal settlement located in a valley corridor approximately 12 kilometres south of Port Moresby. The settlement boundary encompasses approximately 460 ha, which is nearly three times the area of 8 Mile Settlement but was substantially less densely built up at the start of the study period. Vegetation cover is moderate, primarily comprising secondary scrub and riparian vegetation along the valley floor, providing better roof-to-ground contrast than 8 Mile and enabling more complete automated detection. Taurama Valley exhibits a pronounced infill and expansion pattern across the 2010–2025 study period, with building counts growing from a sparse initial base to a substantially denser settlement by 2025.

Datasets and Key Processing Steps

Satellite Imagery

Very-high-resolution (VHR) commercial satellite imagery from the Maxar archive was used for all three survey epochs at both PNG sites. WorldView-2 imagery was used for both the 2010 and 2016 epochs; WorldView-3 imagery was used for 2025. Both sensors provide panchromatic imagery at sub-meter resolution (0.46 m and 0.31 m respectively) suitable for building-scale analysis (Table 2). All imagery was delivered as orthorectified, surface-reflectance-corrected products.

| Sensor | Pan Resolution (m) | MS Resolution (m) | MS Bands | Used for |
| --- | --- | --- | --- | --- |
| WorldView-2 | 0.46 | 1.84 | 8-band | Epoch 1 (2010) and Epoch 2 (2016) |
| WorldView-3 | 0.31 | 1.24 | 8-band + SWIR | Epoch 3 (2025) |

Table 2. VHR satellite sensor specifications for Papua New Guinea survey epochs.

Image Preprocessing

Pan-sharpening was performed using the Gram-Schmidt spectral sharpening algorithm in ArcGIS Pro, producing sub-meter resolution composites at the panchromatic band spatial resolution. Following pan-sharpening, a three-band RGB composite was generated using bands 5 (Red), 3 (Green), and 2 (Blue) in the WorldView sensor band numbering scheme. All imagery was converted from native 16-bit to 8-bit unsigned integer format using a 2nd–98th percentile linear stretch prior to model inference. The model expects three‑band 8‑bit RGB input and performs best on imagery with ground sample distances in the ~10–40 cm range.

Image co-registration was applied to align the multi-temporal scene pairs using the AROSICS automated co-registration library (Scheffler et al., 2017) in a Python environment. For Taurama Valley (PNG North), global co-registration achieved sub-pixel accuracy. For 8 Mile Settlement (PNG South), AROSICS failed to produce geometrically valid results; temporal change comparisons at this site are therefore subject to residual geometric offsets between epochs.

Deep Learning Building Detection

Building footprints were extracted using the Esri USA Building Footprint Extraction pretrained model, which is a Mask R‑CNN–based instance segmentation approach consistent with Mask R‑CNN principles (He et al., 2017). The USA model was selected after comparative evaluation of the Africa, Australia, and USA pretrained models; it produced the best detection results at PNG sites, attributed to its training on diverse suburban typologies including low-vegetation landscapes sharing spectral characteristics with dry-season Port Moresby environments. All inference was performed in ArcGIS Pro using the Detect Objects Using Deep Learning tool in CPU mode.

| Parameter | Value |
| --- | --- |
| Model | USA Building Footprint Extraction (Mask R-CNN-based) |
| Confidence threshold | 0.20 (lower threshold used to maximize recall under canopy) |
| Tile size (pixels) | 256 × 256 |
| Padding (pixels) | 32 |
| Batch size | 1 |
| Test Time Augmentation | True |
| Merge policy | mean |
| NMS max overlap ratio | 0.10 |
| Processor | CPU |

Table 3. Deep learning detection parameters applied at both Papua New Guinea study sites.

Post-processing

A five-stage post-processing pipeline was applied to raw detection outputs at both PNG sites: (1) clipping to ADB-provided settlement boundary polygons; (2) area filtering to remove polygons below 12 m²; (3) Repair Geometry to resolve topological errors; (4) Regularize Building Footprints using the RIGHT_ANGLES AND DIAGONALS method with a 1.5 m tolerance; and (5) systematic manual editing to add missed buildings and remove confirmed false positives. Manual QA was carried out to the extent permitted by the project timeline; results represent the best achievable estimates within that constraint. Additional manual digitizing, if time had permitted, would have increased building counts at both sites, especially at 8 Mile Settlement where canopy occlusion produced the greatest number of missed detections.

Results

Building Count Growth

Building counts increased substantially at both Papua New Guinea sites across the three survey epochs. Taurama Valley shows the more dramatic absolute and relative growth, rising from 121 structures in 2010 to 2,634 structures in 2025. 8 Mile Settlement, despite its denser initial conditions, also grew markedly from 887 structures in 2010 to 2,004 in 2025. Table 3 provides the full building count summary for both sites.

Table 3. Building count summary for Papua New Guinea study sites across three survey epochs

| Site | Year | Building Count | Δ from prev. epoch | Boundary Area (ha) |
| --- | --- | --- | --- | --- |
| 8 Mile Settlement | 2010 | 887 | — | 156.15 |
| 8 Mile Settlement | 2016 | 1,624 | +737 | 156.15 |
| 8 Mile Settlement | 2025 | 2,004 | +380 | 156.15 |
| Taurama Valley | 2010 | 121 | — | 459.59 |
| Taurama Valley | 2016 | 732 | +611 | 459.59 |
| Taurama Valley | 2025 | 2,634 | +1,902 | 459.59 |

Figure 10 illustrates the building count trajectories for both Papua New Guinea study sites across the three survey epochs. Taurama Valley shows near-exponential growth, rising from 121 structures in 2010 to 2,634 in 2025. The 2016–2025 interval alone added 1,902 structures, which is more than double the total stock at the start of that period. 8 Mile Settlement shows a strong but more graduated increase, from 887 structures in 2010 to 2,004 in 2025.

Figure 10. Building count growth for both Papua New Guinea study sites across the three survey epochs. Values annotated at each data point.

Absolute Change in Building Count by Period

Figure 11 presents the absolute change in building count between consecutive survey epochs for both PNG sites. The most striking observation is the magnitude of the 2016–2025 interval at Taurama Valley (+1,902 structures), which is the largest single-period addition of either site across the full study period. At 8 Mile Settlement, the 2010–2016 interval contributed +737 structures, with a more modest +380 in 2016–2025. The relatively smaller increment in the later period at 8 Mile may reflect genuine consolidation of growth but is also likely affected by increasing canopy density reducing model detection capacity over the same period.

Figure 11. Absolute change in building count by survey period across all study sites. The 2016–2025 period produced the largest single-site addition in the entire study at Taurama Valley (+1,902 structures).

Compound Annual Growth Rates

Compound annual growth rates (CAGR) were computed for each consecutive survey period to express the average yearly rate of building count increase. CAGR is defined as:

CAGR = (End Count ÷ Start Count)^(1 ÷ n years) − 1

where n is the number of years between epochs. CAGR is the appropriate metric for cross-site comparisons because it accounts for compounding growth on an increasing base and normalizes for the different time spans between epochs at PNG (6-year and 9-year intervals). Results are benchmarked against the ADB Melanesian urban expansion norm of 3–4% per year.

Table 4. Compound annual growth rate (CAGR) by period for Papua New Guinea study sites. ADB Melanesian benchmark: 3–4%/yr

| Site | Period | Start Count | End Count | Years | CAGR (%/yr) | vs. ADB Benchmark |
| --- | --- | --- | --- | --- | --- | --- |
| 8 Mile Settlement | 2010–2016 | 887 | 1,624 | 6 | 10.61% | Exceeds (>4%/yr) |
| 8 Mile Settlement | 2016–2025 | 1,624 | 2,004 | 9 | 2.36% | Below (<3%/yr) |
| Taurama Valley | 2010–2016 | 121 | 732 | 6 | 34.99% | Exceeds (>4%/yr) |
| Taurama Valley | 2016–2025 | 732 | 2,634 | 9 | 15.29% | Exceeds (>4%/yr) |

The preliminary CAGR figures for Papua New Guinea sites are exceptional relative to the ADB benchmark. Taurama Valley's 2010–2016 CAGR of 34.99% per year reflects the rapid colonization of a large, previously sparse settlement area — a pattern consistent with the arrival-city dynamics described by Connell (2011) for Pacific capitals. The subsequent 2016–2025 CAGR of 15.29% per year at Taurama Valley, while lower, still represents growth more than three times the upper bound of the ADB Melanesian norm.

The 8 Mile Settlement CAGR pattern is more nuanced. The 2010–2016 rate of 10.61% per year substantially exceeds the benchmark. The 2016–2025 rate of 2.36% per year falls below the benchmark lower bound, however this may reflect detection limitations rather than a genuine slowdown. The USA model's lower recall in canopy-dense conditions disproportionately affects the 2025 count, potentially underestimating the true count relative to earlier epochs where conditions may have been marginally more favorable.

These figures should be treated as indicative. The CAGR is sensitive to starting count accuracy, which means if the 2010 counts are underestimates due to sensor resolution limitations at the boundaries of the model's optimal range, growth rates from that baseline would be overstated. All CAGR figures are submitted as proof-of-concept estimates for ADB project planning purposes.

Figure 12. Compound annual growth rates for both Papua New Guinea study sites by survey period. Blue shaded band = ADB Melanesian benchmark (3–4%/yr). Red = exceeds; amber = below; green = within benchmark.

Settlement Density

Settlement density, defined the number of structures per hectare, is the primary indicator of urban intensity and a key input to infrastructure provision planning. The Pacific informal settlement norm established by ADB is 15–25 structures per hectare. Table 5 and Figure 13 present density values for both PNG sites across the three epochs.

Table 5. Settlement density by epoch for Papua New Guinea study sites

| Site | Year | Building Count | Area (ha) | Density (str/ha) | vs. Pacific Norm (15–25/ha) |
| --- | --- | --- | --- | --- | --- |
| 8 Mile Settlement | 2010 | 887 | 156.15 | 5.68 | Below norm |
| 8 Mile Settlement | 2016 | 1,624 | 156.15 | 10.40 | Below norm |
| 8 Mile Settlement | 2025 | 2,004 | 156.15 | 12.83 | Below norm |
| Taurama Valley | 2010 | 121 | 459.59 | 0.26 | Below norm |
| Taurama Valley | 2016 | 732 | 459.59 | 1.59 | Below norm |
| Taurama Valley | 2025 | 2,634 | 459.59 | 5.73 | Below norm |

Both PNG sites remain below the Pacific norm of 15–25 structures per hectare across all survey epochs. At 8 Mile Settlement, density reached 12.83 str/ha by 2025, thus approaching the lower bound of the norm range but not yet within it. At Taurama Valley, density reached only 5.73 str/ha by 2025, reflecting the larger boundary area of the site relative to current building stock.

It is important to note that these density values are likely underestimates for both sites. Canopy occlusion at 8 Mile Settlement produces systematic undercounts of buildings. Similarly, Taurama Valley's large boundary area (459.59 ha) dilutes the density figure. Occupied areas within the boundary may already exhibit densities consistent with or exceeding the Pacific norm, while undeveloped areas pull the site-wide figure down. Visagie and Turok (2020), drawing on evidence from African informal settlements, caution that settlement-wide density metrics can obscure significant spatial heterogeneity in built-up intensity within informal settlement boundaries; a methodological concern that applies equally to the diverse Pacific Island settlement contexts examined here.

Figure 13. Settlement density (structures per hectare) for both Papua New Guinea study sites across all survey epochs. Yellow band = ADB Pacific norm (15–25 str/ha).

Built-up Coverage Ratio

Built-up coverage, defined the percentage of settlement boundary area covered by building footprints, provides a complementary measure of urban intensity that incorporates both building count and individual footprint size. Figure 14 and Table 6 present coverage trajectories for both PNG sites.

Table 6. Built-up coverage ratio by epoch for Papua New Guinea study sites

| Site | Year | Building Count | Total Bldg Area (m²) | Coverage (%) |
| --- | --- | --- | --- | --- |
| 8 Mile Settlement | 2010 | 887 | 64,489 | 4.13% |
| 8 Mile Settlement | 2016 | 1,624 | 114,037 | 7.30% |
| 8 Mile Settlement | 2025 | 2,004 | 111,562 | 7.15% |
| Taurama Valley | 2010 | 121 | 9,030 | 0.20% |
| Taurama Valley | 2016 | 732 | 83,821 | 1.82% |
| Taurama Valley | 2025 | 2,634 | 223,842 | 4.87% |

The coverage trajectory at Taurama Valley shows consistent growth across all periods, from 0.20% in 2010 to 4.87% in 2025. At 8 Mile Settlement, coverage increased from 4.13% in 2010 to 7.30% in 2016 but then slightly declined to 7.15% in 2025, despite a continued increase in building count. This apparent paradox where coverage declining while count rises is a data quality artefact rather than a real-world trend. Several factors contribute. First, canopy occlusion plays a direct role: as canopy density at 8 Mile increases over time, the model traces smaller partial footprints for structures visible only through narrowing canopy gaps, reducing the recorded footprint area per building even as new detections are added. Second, differential model parameters or QA completeness states between epochs may result in slightly larger average polygon sizes being recorded in 2016 than in 2025. Building count is therefore the more robust directional signal at this site, consistent with the guidance of Lu et al. (2004) that change detection metrics should be interpreted alongside known sensor and methodological inconsistencies between epochs.

Figure 14. Built-up coverage ratio (%) for both Papua New Guinea study sites by survey epoch. Coverage reflects the proportion of the settlement boundary area occupied by building footprints.

Median Building Size

Median building footprint area provides insight into building typology trends over time. Figure 6 presents median building area with bubble sizes scaled to building count, enabling simultaneous visualization of size and count trajectories.

Table 7. Building footprint size statistics by epoch for Papua New Guinea study sites

| Site | Year | Median Bldg Area (m²) | Mean Bldg Area (m²) | Min (m²) | Max (m²) |
| --- | --- | --- | --- | --- | --- |
| 8 Mile Settlement | 2010 | 60.75 | 72.75 | 7.30 | 534.37 |
| 8 Mile Settlement | 2016 | 60.75 | 70.22 | 2.24 | 411.75 |
| 8 Mile Settlement | 2025 | 45.00 | 55.67 | 11.81 | 490.78 |
| Taurama Valley | 2010 | 66.94 | 74.63 | 0.01 | 248.06 |
| Taurama Valley | 2016 | 101.25 | 114.51 | 8.11 | 606.38 |
| Taurama Valley | 2025 | 70.87 | 84.99 | 0.70 | 1397.03 |

At 8 Mile Settlement, the median footprint area declined steadily from 60.75 m² in 2010 to 45 m² in 2025. This trend is consistent with densification through construction of progressively smaller infill structures in available inter-building gaps. At Taurama Valley, median building size increased substantially in 2016 (to 101.25 m²) before declining to 70.87 m² in 2025. The 2016 peak likely reflects detection of larger, more visible structures in the early colonization phase; the 2025 decline reflects the addition of many smaller infill structures as the settlement densified.

Figure 15. Median building footprint area over time for both Papua New Guinea study sites. Bubble size is proportional to total building count.

The results for Papua New Guinea's two study sites, though preliminary, point consistently toward rapid and sustained informal settlement expansion that substantially exceeds the ADB Melanesian benchmark of 3–4% per year. This pattern is consistent with the broader urbanization trajectory described for Port Moresby in the literature. Jones (2012) documented that Port Moresby's urban growth rate had reached approximately 4.1% per year, placing it among the fastest-growing cities in the Pacific. Connell (2011) characterized Pacific urbanization as driven by push factors from rural areas rather than productive absorption by urban economies, which is a pattern that tends to produce rapid spatial expansion of settlement footprints without commensurate improvement in infrastructure or service provision.

The growth trajectory at Taurama Valley is particularly notable. The near-exponential increase from 121 to 2,634 structures between 2010 and 2025 indicates that Taurama Valley has transitioned from a sparsely occupied peri-urban corridor to a substantial informal community within a single decade. This rapid colonization of previously undeveloped land is consistent with Numbasa and Koczberski's (2012) observations of informal land access through non-market transactions and kin networks in PNG peri-urban areas. For infrastructure planners, this trajectory implies that network extensions planned on the basis of 2010 or even 2016 population counts are likely already undersized for current demand.

At 8 Mile Settlement, the settlement density approaching 12.83 str/ha by 2025 is still below the lower bound of the Pacific norm. This figure may be an artifact of systematic canopy-related undercounting rather than a true reflection of built-up intensity. Field observations and visual inspection of VHR imagery suggest that large portions of 8 Mile Settlement are already saturated with structures, with canopy cover preventing their automated detection. Infrastructure planning for this site should assume higher true densities than the satellite-derived estimates indicate.

Taken together, the PNG results underscore the need for robust, regularly updated geospatial datasets to support urban infrastructure investment planning in Port Moresby. The deep learning workflow documented in this report provides a scalable mechanism for such updates, though improvements to canopy occlusion handling, through model fine-tuning with local training labels or alternative detection approaches such as SAM-based methods, would improve data quality at 8 Mile Settlement.

Seismicity

Papua New Guinea occupies one of the most tectonically complex and earthquake-prone regions on Earth, situated along the active boundary between the Australian and Pacific Plates. Much of the country experiences frequent moderate to large earthquakes associated with oblique convergence, subduction, arc–arc collision, and complex strike-slip faulting. However, Port Moresby, located on the south coast of Papua New Guinea, lies in a comparatively low-seismicity zone relative to the highly active regions along the northern and northeastern parts of the country. The capital is positioned on the more stable portion of the Australian Plate, south of the primary plate-boundary deformation front that runs through the Highlands and along the Bismarck and Solomon arcs. As a result, Port Moresby experiences far fewer felt earthquakes than other major population centers such as Rabaul, Lae, or Madang, where seismicity and volcanic activity are common. Regional catalogs do record occasional moderate-magnitude earthquakes that originate well offshore or within the broader Papua New Guinea region, and these events may be weakly felt in the city; however, damaging shaking in Port Moresby is uncommon, and no locally generated, large historical earthquakes are documented in its immediate vicinity. Nonetheless, the country’s overall tectonic setting places it firmly within the Pacific “Ring of Fire,” and national emergency-management guidance recommends maintaining routine preparedness due to the potential for shaking from more distant but powerful regional events.

Discussion

The hydrologic and settlement‑mapping results presented in this report provide complementary information about environmental conditions and patterns of urban development in the Port Moresby region. Considered together, these datasets illustrate how seasonal and interannual variability in water availability coincides with rapid expansion of informal settlements, creating conditions in which water‑resource demands may change quickly relative to available supply.

Satellite‑based precipitation and evapotranspiration (ET) datasets show that Port Moresby is characterized by strong hydrologic seasonality. Rainfall is concentrated in the wet season, and several months each year receive relatively low precipitation. Actual ET remains comparatively stable throughout the year and exhibits less interannual variability than precipitation. As a result, many dry‑season months show negative net water, indicating that evaporative demand exceeds precipitation for extended periods. These findings are consistent with the region’s semiarid to subhumid climate and its sensitivity to El Niño–Southern Oscillation conditions.

The informal‑settlement mapping analysis indicates substantial increases in built‑up area and building counts at both 8 Mile Settlement and Taurama Valley over the 2010–2025 period. Growth is most pronounced at Taurama Valley, where building counts increased from 121 to 2,634 structures. At 8 Mile Settlement, the number of identified structures also increased, although persistent canopy cover limits automated detection and likely results in underestimation of building stock. Settlement densities and built‑up coverage ratios remain below typical Pacific informal‑settlement norms, but these values likely reflect boundary effects and detection limitations rather than actual settlement intensity.

The combined hydrologic and urban‑growth results highlight several considerations relevant to understanding water‑resource conditions in the Port Moresby Basin. First, the strong seasonality of precipitation suggests that areas experiencing rapid population growth may be vulnerable to dry‑season deficits, especially where access to managed storage or reliable distribution infrastructure is limited. Second, the rapid pace of settlement expansion indicates that water demand in peri‑urban areas may be increasing in ways not captured by historical planning baselines. Third, the coincidence of highly variable rainfall and expanding built‑up areas underscores the value of routinely updated geospatial information to support assessments of water availability and infrastructure needs.

These datasets do not provide direct measurements of water use or distribution capacity, and canopy‑related limitations introduce uncertainty into building‑count and coverage estimates. Nevertheless, the results demonstrate that remotely sensed precipitation, ET, and settlement‑mapping products can provide consistent, repeatable indicators of environmental and urban‑development patterns. When combined with local hydrologic records and infrastructure assessments, these datasets can help resource managers better understand spatial and temporal variability in factors that influence water supply and demand in the Port Moresby region.

Conclusion

The analysis presented in this report provide a consistent set of satellite-derived hydrologic and settlement-mapping datasets for the Port Moresby region. Precipitation, evapotranspiration, and net-water results indicate a strongly seasonal hydrologic regime characterized by substantial variability in rainfall and comparatively stable evaporative demand. The resulting wet-season surpluses and dry-season deficits reflect the semiarid to subhumid climate of the region and its sensitivity to interannual climate oscillations.

The building-footprint mapping results for 8 Mile Settlement and Taurama Valley show substantial increases in building counts and built-up area from 2010 to 2025, with the most rapid expansion occurring at Taurama Valley. Although canopy cover and other factors introduce uncertainty into building-count estimates, the overall trajectories indicate continued informal-settlement growth in peri-urban areas of Port Moresby.

Together, the hydrologic and urban-growth datasets provide a complementary perspective on environmental conditions and land-use change in the study area. These results do not independently quantify water use or infrastructure capacity; however, they offer consistent, repeatable indicators that can support ongoing assessments of water availability, settlement expansion, and potential pressures on water-resource systems. Continued monitoring using satellite-based products, combined with local observations and administrative data, can assist resource managers and planning agencies in evaluating changing conditions and supporting long-term water-resource planning in the Port Moresby region.

References Cited

Asian Development Bank (ADB), 1965, Agreement establishing the Asian Development Bank signed at Manila, December 4, 1965; entered into force August 22, 1966: United Nations Treaty Series, v. 608, p. 380. Available at: https://www.adb.org/sites/default/files/institutional-document/32120/charter.pdf. Accessed January 29, 2026.

Asian Development Bank (ADB), 2012, The state of Pacific towns and cities: Urbanization in ADB’s Pacific developing member countries: Pacific Studies Series, Manila, Asian Development Bank.

Connell, J., 2011, Elephants in the Pacific? Pacific urbanisation and its discontents: Asia Pacific Viewpoint, v. 52, no. 2, p. 121–135. https://doi.org/10.1111/j.1467-8373.2011.01445.x

DeJonge, K.C., Allen, R.G., Kilic, A., Thorp, K.R., Kukal, M., Marek, G., Altenhofen, J., Amatya, D.M., Blankenau, P., Datta, S., Grabow, G., Hashem, A., Kisekka, I., Kjaersgaard, J., Marek, T., Peters, T., Porter, D., Reba, M., Rudnick, D., Senay, G., Sharma, V., Sridhar, V., Sun, G., Taghvaeian, S., Trezza, R., and Trout, T., 2025, Evapotranspiration terminology and definitions: U.S. Geological Survey Publication, accessed April 11, 2026, at https://www.usgs.gov/publications/evapotranspiration-terminology-and-definitions.

Goddard, M., 2005, The unseen city: Anthropological perspectives on Port Moresby, Papua New Guinea: Canberra, Pandanus Books.

He, K., Gkioxari, G., Dollár, P., and Girshick, R., 2017, Mask R-CNN: Proceedings of the IEEE International Conference on Computer Vision (ICCV), p. 2961–2969. https://doi.org/10.1109/ICCV.2017.322

Hall, A.J., 1984, Hydrology in tropical Australia and Papua New Guinea: Journal of Hydrological Sciences, v. 29, no. 4, p. 399–400.

McGregor, G.R., 1989, An assessment of the annual variability of rainfall: Port Moresby, Papua New Guinea: Singapore Journal of Tropical Geography, v. 10, no. 1, p. 1–12. Documents Port Moresby’s climate as “lowland dry subhumid” with a distinct dry season (May–October), and strong seasonal rainfall variability tied to southeastern trade winds and monsoon shifts.

National Statistical Office of Papua New Guinea, 2025, 2024 National Population Census—Final figures: National Statistical Office report, https://www.nso.gov.pg/wpfd_file/2024-national-population-census-final-figures/, accessed April 14, 2026.

Rooney, M.N., 2021, We want development: Land and water (dis)connections in Port Moresby, urban Papua New Guinea: The Contemporary Pacific, v. 33, no. 1, p. 1–30.

Scheffler, D., Hollstein, A., Diedrich, H., Segl, K., and Hostert, P., 2017, AROSICS: An automated and robust open-source image co-registration software for multi-sensor satellite data: Remote Sensing, v. 9, no. 7, p. 676. https://doi.org/10.3390/rs9070676

Senay, G.B., Kagone, S., and Velpuri, N.M., 2020, Operational global actual evapotranspiration using the SSEBop model—USGS data release: U.S. Geological Survey Data Release, DOI:10.5066/P9OUVUUI.

USGS Data Release

UN-Habitat, 2010, Papua New Guinea: Port Moresby urban profile: Nairobi, United Nations Human Settlements Programme.

Visagie, J., and Turok, I., 2020, Getting urban density to work in informal settlements in Africa: Environment and Urbanization, v. 32, no. 2, p. 351–370. https://doi.org/10.1177/0956247820907808